| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4084 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Proposed call signalling procedures for multi-screen telepresence terminals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

It’s proposed to give the description of call signalling procedures for multi-screen telepresence terminals based on single call setup for H.323 telepresence system.

# Introduction

There was a consensus that single call-setup session will be the baseline approach for telepresence system recommendations developed by Q5/16 at the March 2011 Q.5/16 meeting. Experts give some suggestions on the high level structure, this contribution try to refine the structure and give the description of call signalling procedures.

# Discussion

For the telepresence system structure, the method of creating call-setup session and media stream channels is important. Hence the high-level structure will be discussed from these aspects.

In the following scenarios 3 types of meeting rooms are considered to be participating in the same telepresence meeting controlled by one MCU. The meeting rooms are respectively one screen, 2-screens and 3-screens telepresence terminal subsystem.

Case 1 shows that each telepresence terminal subsystem creates only one call-setup session, but multi-video and a single audio stream channels with MCU.

Case 2 shows that each telepresence terminal subsystem creates only one call-setup session, moreover, one bidirectional video and one audio stream channel with MCU.

Case 3 shows that a telepresence terminal subsystem has only one telepresence terminal which connects with multiple video/audio peripherals. It creates one call-setup session, multiple or single video channel, multiple or single audio channel with MCU.

This contribution tries to give the call signalling procedures for above scenarios.

# Conclusion

It is proposed to use the text below for H.323 architecture and single call setup procedures.

# 9	H.323 Architecture

## 9.1 Mapping Telepresence on a H.323 Architecture

……

## 9.2 Single call setup procedures

For multi-screens terminal subsystem, there are one primary telepresence terminal and at least one secondary telepresence terminal. The primary telepresence terminal creates connection with remote endpoint, such as an MCU, handles call-setup, terminal capabilities exchange, open logic media channel. By default, the numbers of input and output video streams are same, also for audio stream.

In case 1, T1, T2, T5 are primary telepresence terminals which handle call setup procedures, T3, T4, T6 are secondary terminals.  For each telepresence terminal subsystem, the primary terminal handles call-setup session, capabilities negotiation and exchange, creation of media logic channels.

Figure 1: Each telepresence terminal subsystem creates 
one audio channel, but multiple video channels

For audio stream, the primary telepresence terminal handles media stream encoding, decoding, sending and receiving. There are one audio sending channel and one receiving channel between the terminal subsystem and remote endpoint, such as an MCU.

For video stream, each telepresence terminal handles one media stream encoding, decoding, sending and receiving. There are multiple sending channels and receiving channels between the terminal subsystem and remote endpoint. Each video sending channel or receiving channel is connected with each terminal in the subsystem and remote endpoint directly and handles one media stream separately. Each channel corresponds to the specific media stream identified by the media input and output position information.

The following figure shows the procedure for multiple media channel between the terminal subsystem and remote endpoint:

Figure 2: Procedure for multiple media channels

The procedure includes steps below:

- H.225 RAS registration process

- H.225 Call setup process

- Exchange capability includes streams position information, such as the number of input/output media stream and spatial relationships.

- Open bidirectional multiple logic media channels between each terminal in the subsystem and remote endpoint. Each channel corresponds to the specific media stream identified by the media steam type, input and output position information.

- Media stream transport process. Media stream is transported between each telepresence terminal in the subsystem and remote endpoint directly through corresponding channel.

- Close media channels and Call disconnecting process.

In case 2, T1, T2, T5 are primary terminals which handle call setup procedures, T3, T4, T6 are secondary terminals.  For each terminal subsystem, the primary terminal handles call-setup session, capabilities negotiation and exchange, creation of media logic channels.

Figure 3: Each telepresence terminal subsystem creates 
one audio channel and one video channel

For media stream, the primary telepresence terminal handles creation of media channels and transport of media stream between the subsystem and remote endpoint, as well as forwarding media stream from/to corresponding secondary terminals. There are one audio/video sending channel and one receiving channel between the terminal subsystem and remote endpoint, such as an MCU. The primary telepresence terminal or remote endpoint distinguishes the media stream by the media type and from/to position information in the head of media data packet.

The following figure shows the procedure which uses bidirectional multiplex media channel between the terminal subsystem and remote endpoint to transport multiple media stream:

Figure 4: Procedure for multiplex media channels

The procedure includes steps below:

- H.225 RAS registration process

- H.225 Call setup process

- Exchange capability includes streams position information, such as the number of input/output media stream and spatial relationships.

- Open bidirectional multiplex media channel between the primary terminal in the subsystem and remote endpoint. Multiple video streams are transported though one sending channel and one receiving channel as well as audio.

- Media stream transport process. The primary telepresence terminal transport s the media stream with remote endpoint, as well as forwarding media stream from/to corresponding secondary terminals. The media streams are distinguished by the media type and from/to position information in the head of media data packet.

- Close media channels and Call disconnecting process.

In case 3, each telepresence terminal subsystem has only one telepresence terminal which has multiple video/audio interfaces and connects with multiple video/audio peripherals. The telepresence terminal creates one call-setup session with remote endpoint, exchanges media capabilities and multiple audio and video input/output position information and creates media logic channels.

Figure 8: Each telepresence terminal subsystem has one telepresence terminal

For multiple media stream, the telepresence terminal uses one audio send and one audio receive channel, or multiple audio send and multiple audio receive channels; one video send and one video receive channel, or multiple video send and multiple video receive channels.

The media streams are distinguished by the media type and from/to position information in the head of media data packet when transporting multiple media stream by one channel, and by channel information when transporting each media stream by different media channel separately, then each channel corresponds to the specific media stream identified by the media type and input/output position information.

The following figure shows the procedure for telepresence terminal with multiple media interfaces:

Figure 4: Procedure for telepresence terminal with multiple media interfaces

The procedure includes steps below:

- H.225 RAS registration process

- H.225 Call setup process

- Exchange capability includes streams position information, such as the number of input/output media stream and spatial relationships.

- Open bidirectional logic media channel between the telepresence terminal and remote endpoint.

- Media stream transport process.

- Close media channels and Call disconnecting process.

__________________