| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4123 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 4 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI, POSTECH |  |  |  |
| Title: | Performance analysis of a proposed fountain encoded (AL-FEC) symbol transmission scheme |  |  |  |
| Purpose: | Proposal / Discussion / Analysis |  |  |  |

Summary

This contribution proposes a robust And-Or tree-based Luby Transform encoded symbol packetization algorithm over error-prone network. The proposed packetization algorithm is designed to minimize packet loss effects by reducing the dependency among packets conveying Luby Transform encoded symbols. A mathematical analysis is conducted to prove performance of proposed packetization algorithm of Luby Transform codes compared with conventional packetization algorithm.

# Introduction

Fountain codes such as Luby Transform (LT), Raptor, and Online codes are emerging erasure codes and block-based FEC schemes. The characteristics such as high coding efficiency, low encoding/decoding processing time, and flexibility are very useful for transmitting delay-sensitive data over error-prone network. LT codes are the first realization of a class of erasure codes that we call universal erasure codes, which are adopted in this contribution due to their simple structure. The goal of the proposed algorithm is to effectively transmit LT encoded symbols over error-prone network. First of all, the relationship among LT encoded symbols is analyzed by using And-Or tree. Based on the analysis, LT encoded symbols are packetized to reduce the correlation among packets so that the effect of a lost packet is locally limited.

# Discussion

Figure 1 shows the overall system block diagram under our consideration over error-prone network. The application data are LT encoded for robust transmission through network. During the LT encoding process, the application data should be divided into source blocks. A source block is partitioned into source symbols of a predefined size. At this moment, an LT encoded symbol is mapped to a portion of the source symbols, i.e. it is generated by performing bitwise XOR operations on the selected source symbols. The process is repeated until the last LT encoded symbol is generated. The mapping process is controlled by the generator polynomial, which is one of the most important design factors. Now, the LT encoded symbols should be packetized before the transmission because most of the state-of-the-art networks support packet switching. Sometimes, packet losses during transmission are inevitable over error-prone network. Thus, the number of the received LT encoded symbols may not always be enough for the successful decoding in time. To efficiently handle this problem, it is important to estimate how many packets (or additional encoded symbols) should be transmitted to deliver a sufficient number of encoded symbols to the receiver through the error-prone network (it is called overhead in the following). Another essential problem is how to reduce the LT decoding failure rate with the same number of packets. In fact, LT decoding performance is related to how to packetize the encoded symbols because the dependency among them exists due to the iterative decoding process. Hence, the decoding performance may be seriously deteriorated when the dependency is not considered. For example, when the LT encoded symbols are randomly grouped into packets, a lost packet may hinder many source symbols from being successfully decoded. Based on the analysis by using And-Or tree, the proposed packetization algorithm is designed to reduce the correlations among packets while increasing those among encoded symbols in each packet. As a result, the LT decoding success rate can be improved at the receiver by minimizing the effect of a lost packet. In other words, we can achieve the same LT decoding success rate with a smaller amount of overhead.

Figure 1. Overall system block diagram under our consideration.

## Symbols

This contribution uses the following terms:

Target source symbol

The ith target encoded symbol of the

Degree of

Set of source symbols associated with  ( ‘s children)

Set of encoded symbols associated with

The number of selected encoded symbols in

The number of encoded symbols per packet

## AND-OR Tree-based Relationship Analysis of LT Encoded Symbols

And-Or tree is a useful tool to investigate these relationships. In general, And-Or tree  is a tree with a depth of 2l, in which the depth of the root is zero and the depth of children directly connected to the root is 1. The depth of each node increases further down the tree. The nodes at even depths such as 0, 2, 4, …, 2l-2 are called Or-nodes and those at odd depth such as 1, 3, 5, …, 2l-1 are called And-nodes. Every node has the value of 0 or 1. The Or-nodes have Or-operation value of their children (0 in the case of no children). The And-nodes have And-operation value of their children (1 in the case of no children). The Or-nodes are source symbols while the And-nodes are encoded symbols. They are mapped to each other as parents and children. A simple example is given in Figure 2. Since the root is an Or-node and  is an And-node, all source symbols in the  should be effectively decoded to obtain the  by using  regardless of the other target encoded symbols. Thus the successful decoding of the root by using  is affected by encoded symbols at depth 3. That is, the elements of  should be decoded in advance to make  contribute to the successful decoding of the root. As shown in Figure 2, the receiver can successfully decode more source symbols in  when  and more elements of  are available simultaneously. Accordingly, the decoding success probability of the root is improved. This means that  and the elements of  have a strong dependency on each other. Therefore, when  is lost during transmission, its children encoded symbols in  are disconnected from the  over the tree. Thus they cannot contribute the decoding of the . To enhance the LT decoding success rate, this relationship must be considered during the packetization process. Based on the above observations, we can obtain the following useful properties.

Property 1: The decoding success rate of the root can be improved by assigning all  (’s children) to different packets.

Property 2: The decoding success rate of the source symbols in  can be increased if more symbols of  are available.

Property 3: The decoding success rate of the root can be enhanced by packetizing  and the elements of its  into the same packet.

Figure 2. A example of relationship analysis among LT encoded symbols over the And-Or tree.

## Proposed Packetization Algorithm of LT Encoded Symbols

is a selected element of all  to make a contribution towards decoding the . Based on the previous properties, the proposed packetization algorithm is designed to insert more elements of  and  into the same packet. The packet is called the target packet in the following. The number of elements of  that are packetized with  is determined by a control variable  of the proposed packetization algorithm. When  is 0, the proposed packetization algorithm is equivalent to the conventional packetization algorithm. The Figure 3 is a flow chart of the proposed packetization algorithm.

Figure 3. Flow chart of proposed packetization algorithm.

## Performance Analysis of the Proposed Packetization Algorithm

We can claim that the proposed packetization algorithm works better than the conventional algorithm if , where  and   are defined as subsets of   whose elements arrive at the receiver without errors in the cases of the proposed packetization algorithm and the conventional algorithm, respectively. To analyze the LT decoding performance of the proposed packetization, we assume that  successfully arrives at the receiver. Actually, we do not need to consider the case that  is lost because  does not affect the decoding of the root as mentioned previously. Under the assumption that packet losses occur randomly, we can calculate and  as follows.

,                                          (1)

,                                       (2)

,                                                         (3)

where  is the number of transmitted packets (i.e. it is the smallest integer greater than , where  is the number of encoded symbols),  denotes the average number of  symbols in the target packet when the remaining symbols are randomly allocated to packets, and  is the packet loss rate.

To guarantee, our control variable  should be set to satisfy the following equation.

.                                                  (4)

In general, the probability distribution of the degree of an LT encoded symbol is represented by, where  is the number of source symbols and  means the probability that an encoded symbol has the degree . Then the distribution is represented by the generator polynomial , and the average degree of an encoded symbol is given by . Thus we can derive the sufficient condition (5) for  from (4) and so that the proposed packetization algorithm works better than the conventional packetization algorithm.

,                                          (5)

where  is the LT encoding overhead ratio (i.e. ).

## Experimental Environments

The proposed packetization algorithm is implemented using Java and C/C++. Foreman of CIF size and 15 fps (frames per second) is used as the test video sequence. The GOP structure is set to include one I-frame and 29 P-frames. H.264/AVC JM software is used as a video codec. The copy frame technique is adopted for the error concealment to recover the damaged regions in a frame. The experiment consists of two parts; in the first part, the experiment is executed to verify that the proposed packetization algorithm works reasonably, and in the second part, it is applied to deliver video streaming data over an error-prone wireless network. Every simulation is repeated 1,000 times and the is determined by the robust soliton distribution.

## LT Decoding Failure Rate Comparison

We compare the proposed packetization algorithm and the conventional packetization algorithm in terms of the LT decoding failure rate. Figure 4 shows the average LT decoding failure rate when  is 10 and packet losses happen randomly. The proposed packetization algorithm always provides a lower average LT decoding failure rate than the conventional packetization algorithm regardless of . It is also observed that the average LT decoding failure rate of the proposed packetization algorithm decreases as  increases because more elements in  are available with  at the receiver. However, the LT decoding failure rate is almost saturated when  is larger than about 1.25. Even in these ranges, the proposed packetization algorithm still provides slightly better performance than the conventional packetization algorithm.

(a)                                                                      (b)

Figure 4. Average LT decoding failure rate comparison between the proposed and conventional packetization algorithms ( is set to 10), (a) when  and , (b) when  and .

## Video Quality Comparison

The proposed packetization algorithm is applied to video streaming service over wireless network. The peak signal-to noise ratio (PSNR) is used as a performance measure. Figure 5 shows the average LT decoding failure rate and PSNR of test video sequence Foreman when packet loss patterns are random and burst. The symbol size of LT codes, packet size and  are set to 50, 512 bytes and 5, respectively.  for random loss is set to 0.1. To examine the performance of the proposed packetization algorithm in burst packet loss pattern, a two-state Markov channel model is employed and the state transition probability  (transition probability from good to bad) and  (transition probability from bad to good) are set to 0.0418 and 0.5306, respectively. The proposed packetization algorithm provides lower average LT decoding failure rate and higher average PSNR value than the conventional packetization algorithm for all packet loss patterns. It is observed that the maximum PSNR improvement is about 4 dB and 8 dB when  is around 1.18 and 1.28 when packet losses are random and burst, respectively. When  is less than 1.18 and 1.24 according to packet loss patterns, the decoding failure rate is so high that the receiver cannot display the video. Some captured frames are provided for the subjective video quality comparison in Figure 6. Although video decoder at the receiver uses the Copy frame technique for error concealment, it cannot avoid video quality degradation completely. As shown in the captured frames, the proposed packetization algorithm can provide much better human visual perceptual quality.

(a)

(b)

Figure 5. Comparison of average LT decoding failure rate and PSNR of the test video sequence Foreman between the proposed and the conventional packetization algorithms when , , (a) when packet loss pattern is random , (b) when packet loss pattern is burst (good : no packet loss, bad : packet loss).

(a)

(b)

Figure 6. Comparison of subjective video quality of test video sequence Foreman between the proposed (right figures) and the conventional (left figure) packetization algorithms when , , (a) 10th frame when packet loss pattern is random and  is set to 1.24, (b) 111th frame when packet loss pattern is burst and  is set to 1.34.

# Conclusion

This contribution proposes an effective packetization algorithm of LT codes that considers the relationship between LT encoded symbols over error-prone network with a smaller overhead. To analyze the relationship between LT encoded symbols, we used the And-Or tree. Based on the relationship between LT encoded symbols, the packetization algorithm was designed to minimize the effect of lost packets and improve the LT decoding success rate. LT decoding performance of the proposed packetization algorithm was verified mathematically and empirically. Finally, the experimental results demonstrated that the proposed algorithm provides much better video streaming service over error-prone wireless network.

__________________