| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4126 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.82: Statistics |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This document contains some proposals from Ericsson to clarify the use of the statistics defined in the H.248.82 Recommendation.

# Discussion

The ecnrous package defined in this Recommendation defines statistics for the transfer of data to the MGC regarding number of packets received with the different ECN marks as well as loss packet counters. Within an RTP session, information about received ECN marks is transferred by the MG to the sender with RTCP ECN Feedback packets and RTCP XR ECN Summary reports. The sender can then compare this information with its own accounts of data sent in order to detect errors.  Thus the information conveyed by the H.248 statistics is not needed in the MGC for the correct functioning of the RTP session nor for the correct use of ECN, although the MGC can use it for its own purposes.

Some clarifications are added in this respect.

The draft also suggests the possibility of conditional statistics reporting with the use of H.248.47. The conditional reporting is based on comparing the value of the statistics with a numeric –type double- value and thus it is suitable for statistics also of numeric type. The statistics defined in the ecnrous package, however, are of type list of integers, or list of doubles. The comparison of a list of integers with a single integer value is not a defined operation. Thus the use of conditional reporting for the type of statistics defined in this package is questionable.  Ericsson proposes to remove the text containing the suggestion.

Finally, a reference to the statistics in the RTP package is proposed.

[Begin Correction]

### 7.6.2	Negotiation of the capability to use ECN with RTP/UDP/IP

As per [IETF ECN] a MG supporting ECN with RTP/UDP/IP requires different RTCP extensions:

- RTP/AVPF [RFC4585] transport layer feedback format for urgent ECN information,

- RTCP XR [RFC3611] ECN summary report block type for regular reporting of the ECN marking information.

Thus in addition to the methods for indicating/negotiating ECN described below the MGC shall send other information (i.e. SDP m and a lines) indicating the support of RTP/AVPF [RFC4585] and RTCP XR [RFC3611] as well as the appropriate address information for the Stream.

Note: In some cases (i.e. where timely feedback is not required and usage of the leap of faith initialization method)  transport layer feedback for urgent ECN information may not be required. See clause 3.3 / [IETF ECN] for further information.

In order to be notified of failures the MGC shall also set the "ECN failure" (ecnrous/fail) event.

If the MGC wishes to collect and audit information regarding the use of ECN the MGC should set the "Source" (ecnrous/ssrc), "CE Counter" (ecnrous/cecount), "ECT 0 Counter" (ecnrous/zero), "ECT 1 Counter" (ecnrous/ectone), "Not-ECT Counter" (ecnrous/notetc), "Lost Packets Counter" (ecnrous/lost) and the "Extended Highest Sequence Number" (ecnrous/ehsn) Statistics. Alternatively, for packet loss counters, the statistics defined in the RTP package [ITU-T H.248.1] can be used.

[End Correction]

[Begin Correction]

### 7.6.4	Handling of dynamic groups through failure detection, verification and fallback

When detecting failures the MG should operate according to clause 7.4 / [IETF ECN]. This allows the MG to act autonomously to congestion, high packet loss rate and other failures without the need for interaction with a MGC. However if the MG determines that the use of ECN should be disabled it should indicate this to the MGC via the ECN failure event with type set to "USE".

Associated with ECN are summary reports that may be used to detect various types of ECN path issues. These reports are per source SSRC and contain the following information: CE counter, ECT (0/1) counters, not-ECT Counter, Lost Packets counter, Extended Highest Sequence number. For more information see clause 7.4.2 / [IETF ECN]. The MG sends these reports to its peer using RTCP. The MGC may also request this information for its own purposes, which are outside this Recommendation. In order to make this information available to a MGC this information is recorded in H.248 statistics. Thus if the MGC wishes to receive this information,  it should set the statistics   during the capability negotiation phase (see clause 6.1). If the MGC may then perform an AuditValue.req on the statistics. In order to support multiple SSRCs per Stream the values of the Statistics are in a sub-list form. The values in a certain sub-list position are relative to each other across all the statistics in the ECN package.

[End Correction]

# Conclusion

It is proposed to accept above changes in the H.248.82 draft.

__________________