| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4135 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 12 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur |  |  |  |
| Title: | AMS Issues under consideration |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

As the H.325/AMS experts continue to make progress on defining the new multimedia communication system, the Rapporteur has maintained a list of issues that are currently under discussion and/or still need closure.  This list is reproduced from the last Q12/16 meeting report and provided for discussion.

# Issues under Discussion

- Control and Indication (C&I) messages, including message types and transmission methods, such as:

  - Transmission methods may include transmission directly between applications over a media path, transmission directly between applications over a signalling channel established between applications (an “Interface K”), or transmission through the Container along with other H.325 signalling

  - The types of messages vary significantly depending on the application, including messages like User Input, Far-End Camera Control (FECC), Video Fast Update, whiteboard control messages, file transfer pause/resume, etc.

  - Transmission modes must consider effects on the network and network elements, as well as application requirements (e.g., latency requirements)

- Review of currently identified interfaces

  - Interface A – Container to Container signalling

  - Interface B – Application to container signalling

  - Interface C – Media interface

  - Interface D – Container / proxy interface (likely co-located with Container)

  - Interface E – Container to Service Node signalling

  - Interface K – Application to Application signalling

- Application handover (movement of  an application instance from one device to another) and application replication (bringing a new application entity into an already existing conference)

- “Personal Network” concept wherein the user is able to invoke two or more applications locally through the H.325 Container, such as presenting slides or other content between an application that can source the slide material and an application that can display images

- The Next Generation Network and considerations that must be given for H.325, including those related to QoS, NAT/FW, and SIP interworking

- Capability exchange, including capabilities of the AMS container and various applications

- Signalling / message flows between the applications, Container, network, and remote container, including registration, address resolution, application invocation, and session establishment

- Signalling syntax, with a specific focus on XML and XML Schema

- Media flow establishment and related signalling

- Terminal (“Assemblage”) and network architecture, including functionality provided by the “service node” and various network services (e.g., transcoding)

# Conclusion

The Rapporteur seeks input from experts on revising this list.  The objective is to help guide experts in submitting contributions that help to bring closure to open issues.  Consideration should be given to transforming this into a “living list” type of document.

__________________