| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | TD-15 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur |  |  |  |
| Title: | Report of Q5/16 Rapp. Group meeting (Andover, 18 July - 22 July 2011) |  |  |  |
| Purpose: | Report |  |  |  |

Summary

Question 5/16 held a successful meeting and considered 15 contributions related to F.TPS-Reqs, F/H.TPS-Arch, and H.TPS-AV.  Baseline text was produced for all three items in the work program and the living list of topics was updated.  Q5/16 prepared two outgoing Liaison Statements and its experts will meet next during the SG 16 meeting planned in Geneva, 21 November – 02 December 2011.

# Question 5/16 – Telepresence Systems

The Question 5/16 (Telepresence Systems) experts meeting was held in Andover, MA, United States during the week of 18 July - 22 July 2011. The meeting was chaired by Mr. Stephen Botzko.

## Objectives

- Coordinate with other Questions

- Progress Topics related to

  - F.TPS-Reqs

  - F/H.TPS-Arch

  - H.TPS-AV

- Review output of AHGs

  - Telepresence Living List AHG

  - Q5/16 Definitions AHG

- Consider new material.

# Documentation

Documents considered by Q5/16 at this meeting can be found in the AVC documents area at http://ftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/.

The list of meeting contributions was reviewed (TD-02).  The following contributions were relevant to Q5/16:

- Contributions: AVD-4075, AVD-4082, AVD-4083, AVD-4084, AVD-4086, AVD-4087, AVD-4088*,AVD-4091, AVD-4092, AVD-4094, AVD-4095, AVD-4106, AVD-4107, AVD-4108, AVD-4109 (15 contributions).

- Contribution examined in joint session with Questions 1,5 and 12/16: AVD-4132

- Temporary Documents: TD-06, TD-30, TD-33, TD-45, TD-46, TD-xx

Note that AVD-4088 was received late; however it was agreed to accept and discuss this contribution.

## Review of the meeting agenda

The draft agenda TD-06 was accepted.

### Intellectual property statements

Members were reminded to make disclosures of known patents to the ITU Secretariat as defined in the IPR policy in a timely manner.

Call for IPR: Does anyone have knowledge of patents or pending patent applications, the use of which may be required to implement Recommendations being considered?

There were no answers recorded to this call for IPR.

## Incoming liaison statements

AVD-4155 was presented and discussed in joint session with Questions 1 and 2/16, and is included in the Q1/16 report (xxx).

AVD-4137, AVD-4141, AVD-4142, AVD-4143, AVD-4146, AVD-4147, AVD-4148, AVD-4149, AVD-4150, AVD-4151, AVD-4152, AVD-4154 were presented and discussed during the general WP2 session,  and are included in that report (TD-20).

# Discussions

## Contributions to F.TPS-Reqs

| AVD-4083 | Proposed requirements for Telepresence system | ZTE Corporation |
| --- | --- | --- |

| AVD-4087 | Proposed texts for requirements of Telepresence services | ETRI |
| --- | --- | --- |

These documents were presented and discussed.  With some revisions they were accepted for the output draft for F.TPS-Reqs (TD-33).

In addition, the F.TPS-Reqs editor was requested to provide draft for the scope of F.TPS-Reqs

| TD-33 | Draft Scope of ITU-T F.TPS-Reqs - “Definitions, requirements, and use cases for Telepresence Systems | Editor |
| --- | --- | --- |

This draft scope was discussed, and with some modifications was accepted as baseline text.

| TD-45a | F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence Systems” (New): Output draft | Editor |
| --- | --- | --- |

This contribution from the editor was presented and reviewed.  The document was accepted as the output draft for F.TPS-Reqs for this meeting.

| AVD-4088 | Telepresence Use Cases Involving Everyday Terminals | Research in Motion |
| --- | --- | --- |

This was presented and discussed.  An Editor’s note was added to F.TPS-Reqs, suggesting the need for use cases focused on functionality rather than terminal types, and soliciting contributions of such use cases.  There is agreement that broad interoperability is needed.

The use cases themselves were not accepted as baseline text.  However, they were added to the living list.

## Contributions to F.TPS-Arch

| AVD-4094 | F/H.TPS-Arch “Telepresence System Architecture” (New): Input draft | Editor |
| --- | --- | --- |

| AVD-4095 | F/H.TPS.Arch: Iconography | Huawei |
| --- | --- | --- |

| AVD-4091 | F/H-TPS-Arch: High Level Architecture | Huawei |
| --- | --- | --- |

| AVD-4107 | F/H-TPS-Arch: Sub-system Architectures | Huawei |
| --- | --- | --- |

| AVD-4108 | F/H-TPS-Arch: H.323 Architecture | Huawei |
| --- | --- | --- |

| AVD-4109 | F/H-TPS-Arch: Telepresence room design | Huawei |
| --- | --- | --- |

The above contributions were presented and discussed.  They were accepted as baseline text with some modifications.

| TD-33 | F/H.TPS-Arch “Telepresence System Architecture” (New): Output draft | Editor |
| --- | --- | --- |

This contribution from the editor was presented and reviewed.  The document was accepted as the output draft for F/H.TPS-Arch for this meeting.

| AVD-4084 | Proposed call signalling procedures for multi-screen telepresence terminals | ZTE Corporation |
| --- | --- | --- |

ZTE clarified that there was the use of one audio stream in the examples did not mean that multiple audio streams were not supported.

Experts observed that the physical architecture presented in this contribution could be mapped onto the functional architecture in F/H.TPS-Arch.  It was felt that it might be confusing to present multiple physical architectures as well as the functional architecture, so we did not agree to include these figures in F/H.TPS-Arch.

There was no agreement as to whether spatial information is expressed in capabilities, or signaled in some other way.

## Contributions to H.TPS-AV

| AVD-4092 | Telepresence System Audio/Video Parameters | Huawei |
| --- | --- | --- |

| AVD-4075 | Video Resolution in Telepresence Systems | Polycom |
| --- | --- | --- |

| AVD-4106 | Standard Colour Gamut for Telepresence Systems | Polycom |
| --- | --- | --- |

These recommendations were presented and reviewed.  They were accepted.

| TD-46 | H.TPS-AV “Audio/Video Parameters for Telepresence systems” (New:) Output draft | Editor |
| --- | --- | --- |

This contribution from the editor was presented and reviewed.  The document was accepted as the output draft for H.TPS-AV for this meeting.

## Contributions on Definitions

| AVD-4082 | Discussion of some definitions related to Telepresence | ZTE Corporation |
| --- | --- | --- |

Some additional work is needed to align these definitions with the architecture document usage. Further contributions on this are solicited for future meetings.

As a way of going forward, we will place these on the living list for further discussions.  In future meetings, definitions like this should be proposed as new baseline text for the requirements document. (Note there was no baseline text for requirements prior to this meeting).

| AVD-4086 | Proposed texts for differentiating Telepresence services from videoconference services | ETRI |
| --- | --- | --- |

The proposed text on differentiation was added to the living list, so that this text could be used in future output drafts.

## New Material

| AVD-4132 | Capability Exchange within AMS | Cisco Systems |
| --- | --- | --- |

AVC-4132 was presented and discussed in joint session with Questions 2 and 12/16, and is included in the Q12/16 report (TD-16)

# Presentation of Reports

The draft report for Q5/16 was reviewed in the general WP2 session and the experts accepted its content. The report can be found in TD-15.

# Outgoing liaison statements

- Reply LS to the IMTC was prepared in joint session with Question 1 and 2/16 (q1k10).

- LS to ITU-R and VQEG on Colour Gamuts for HD telepresence systems was prepared (TD-52)

# Work programme

E-mail correspondences pertaining to the activities of this group are routinely conducted using the e-mail reflector currently hosted by the ITU. Those wishing to subscribe or unsubscribe to this email reflector should go the ITU web page at http://itu.int/ITU-T/services/, click on “Login” and, after providing the TIES/Guest user credentials, click on “Subscription”.

E-mails to all subscribed Q5/16 experts should be sent to t09sg16q5@itu.int.

Those wishing to subscribe or unsubscribe to e-mail reflectors hosted by ITU should follow the instructions at http://www.itu.int/ITU-T/edh/faqs-email.html.

Rapporteur group documentation can be found in the external FTP server for Q5/16 located at http://ftp3.itu.int/av-arch/avc-site.

The currently open work items for Q5/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| F.TPS-Req | Definitions, requirements, and use cases for Telepresence Systems | X. Ye (ye.xiaoyang@zte.com.cn) | 2013 | TD-45a |
| F/H.TPS-Arch | Telepresence System Architecture | C. Grove (Christian.Groves@nteczone.com) | 2013 | TD-30 |
| H.TPS-AV | Audio/Video Parameters for Telepresence systems | S. Botzko (Stephen.Botzko@polycom.com) | 2013 | TD-46 |

## Editor Changes

Mark Gorzynski and Ping Fang have resigned their editor roles for F.TPS-Reqs and F/H.TPS-Arch respectively.  Xiaoyang Ye will continue as editor of F.TPS-Reqs, and Christian Grove has been appointed as the new editor of F/H.TPS-Arch.

## Telepresence Living List and Q5/16 Definitions Ad Hoc Groups

The Q5/16 experts agreed that these AHGs were no longer needed now that baseline text is available.  Therefore these two groups were terminated.

## Future meetings

Question 5/16 is planning to meet at the next Study Group 16 meeting 21 November - 2 December 2011 in Geneva, Switzerland.

Annex Q5.A:
Living List for Q5/16

## A.1. Definitions

Develop a definition of telepresence which distinguishes it from videoconferencing. This can include definitions of specific categories of telepresence. The working definition is:

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

Also develop a common vocabulary to describe telepresence equipment, the telepresence environment, and various attributes of the telepresence experience.

AVD-4013, and AVD-4024 propose some definitions and describe some attributes. Progressing this work through the Definitions AHG (TD 547/WP2) and contributions would be useful to our work.

## A.2. Reference Model (System Components, Connections)

Develop a reference model that includes both infrastructure components, terminals, and other room equipment. AVD-3971a , AVD-3972a, COM16-573, COM16-574, COM16-578, and COM16-581 include block diagrams that are useful as a starting point. ITU-T Rec. F.700 may also be a useful starting point.

## A.3. Use cases and Interoperability Scenarios

Develop use cases and describe the interoperability problems that need to be solved. AVD-3971a provides some input on interoperability issues and AVD-4024 captures several desirable characteristics and implies several use cases. See also COM16-580, COM16-576, and COM16-581.

## A.4. Requirements for Recommendations

List agreed specific requirements which need to be addressed through new Recommendations and extensions to existing Recommendations. Include priorities, which will be used to develop the timetable of the work program.

## A.5. Quality of Experience

List QoE aspects necessary to achieve high end-user satisfaction in Telepresence Systems for study. AVD-4013 provides a starting point. SG 12 input has been provided in TD 363/Gen, COM12–C178, COM12–C179, COM12–C190, COM12–C191.

## A.6. Differences between videoconferencing and telepresence

The text below is taken from AVD-4086.  Experts believe that this material could be useful as introductory or explanatory material.

Telepresence looks like high-quality videoconferencing, but it is composed of more functional and advanced emotional technologies such as gaze control, eye-line detection, and so on.

Videoconference, which is defined in F.703, provides real-time bidirectional transfer of voice and moving colour picture communications. The main point of difference between videoconference and telepresence is feeling of immersive and realism. Videoconference only satisfies that user can see remote user’s movement or face whereas telepresence aims at strong sense of realism and presence. Telepresence provides not only high-density video and CD quality spatial audio but also emotional realism functions such as eye contact, eye line, camera control, gaze control and life-size video as if remote participants seem to be located in front of user. It also supports multiple displays, microphones and speakers whereas videoconference is usually composed of one audio-visual device. In addition, in order to achieve realistic experiences on remote user’s presence, telepresence provides cooperation functions such as remote presentation and file transfer as well as high-quality audio-visual multimedia stream.

## A.7. Telepresence Use Cases Involving Everyday Terminals

The text below is taken from AVD-4088.  The experts believe that these use cases could be of value in assessing interoperability.

## A.7.1 Unified communications terminals for business

Figure 1 shows a unified communications use case. In this use case a tablet is connected to a conference speakerphone. The benefits of Telepresence in this use case include better talker identification and less fatigue.

Figure 1. Unified communications use case.

## A.7.2 Home terminals for connecting family

Figure 2 shows a home terminal use case. In this use case a student using a laptop is calling home to family using a telepresence-enabled TV. The greater “sense of presence” is important in this situation. Note that some playback systems (e.g., home theatre, vehicle, binaural headset) will result in sounds being perceived from behind the listener.

Figure 2. Home terminal use case.

## A.7.3 Automotive terminals for reduced driver distraction

Figure 3 shows an automotive terminal use case. In this use case employees in a vehicle are calling into a conference room. Although the exact benefits still need to be quantified, Telepresence systems have the potential to reduce driver distraction. This use case also illustrates how spatial capture and playback is decoupled from signal transport. Peripherals provide spatial capture and playback. Mobile phones provide signal transport.

Figure 3. Automotive terminal use case.

## A.7.4 Medical terminals for better situation awareness

Figure 4 shows a medical terminal use case. A Telepresence terminal built-in to the hospital bed is communicating with a tablet at the nursing station. The nurse has better situation awareness and the patient can hear nurse better. Note that TeleMedicine applications may have more stringent requirements than the use case described here.

Figure 4. Medical terminals use case.

## A.7.5 Personal computers for improved collaboration

Figure 5 shows the personal computer use case. Connected laptops are everywhere. Headphones shown at one of the locations enable localizing sound sources behind the listener.

Figure 5. Personal computer use case.

## A.7.6 Multi-party conferencing

Figure 6 shows the multi-party conferencing use case. Multi-party conferencing can be enhanced by localizing the image of each party from a different location.

Figure 6. Multi-party conferencing use case.

A.7. Some Definitions related to Telepresence Systems Components

The text below is taken from AVD-4082.  Some additional work is needed to align these definitions with the architecture document usage. Further contributions on this are solicited on this topic.

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size. [NOTE – From Living List for Q5/16 ]

Telepresence system: A complete telepresence system consists of telepresence network subsystem and telepresence terminal subsystem, which function together to provide the user experience of Telepresence.

Telepresence network subsystem include Telepresence MCU, Gatekeeper, Gateway, etc, providing registration and management of Telepresence terminal and user, and control of Telepresence call setup, media negotiation and meeting control, appropriate translation between transmission formats, etc.

Telepresence terminal subsystem include telepresence terminal, microphones, loudspeakers, cameras, displays, centre control devices, meeting room environment (such as furniture, lighting, independent walls, conference tables, chairs), all chosen to optimize the user experience of Telepresence.

Telepresence terminal: A telepresence terminal is a component on the network which provides for real-time, two-way communications with another telepresence terminal, Gateway, or telepresence MCU. This communication consists of control, indications, audio, moving color video pictures, and/or data between the two terminals. It's used to provide the user experience of Telepresence.  [NOTE –Reference to ITU-T Rec. H.323 “terminal”]

Telepresence MCU: The telepresence MCU is a component on the network which provides the capability for three or more telepresence terminals and Gateways to participate in a multipoint conference. It may also connect two telepresence terminals in a point-to-point conference which may later develop into a multipoint conference. It's used to provide the user experience of Telepresence. [NOTE –Reference to ITU-T Rec. H.323 “MCU”]

## A.9.	Bibliography

S. Zhao (2003), Toward a taxonomy of copresence, “Presence: Teleoperators & Virtual Environments”, 2003 - MIT Press.

W. A. Ijsselsteijn et al (2001), Toward a Core Bibliography of Presence, CyberPsychology & Behavior. April 2001, Vol. 4, No. 2: 317-321

M. Minsky (1980), Telepresence, Omni, June 1980, pp. 45-51.

K. M. Lee, (2004), Presence, Explicated. Communication Theory, 2004, 14: 27–50.

Ontario Telepresence Project, Final Report, 1995, http://www.dgp.toronto.edu/tp/techdocs/Final_Report.pdf.

Bill Buxton, Ontario Telepresence Project Videos, http://www.billbuxton.com/buxtonOTPVideos.html

Steve Harrison (editor), (2009), Media Space 20+ Years of Mediated Life (Computer Supported Cooperative Work), Springer, NY, 2009.

S. W. Littlejohn and K. A. Foss (2005). Theories of Human Communication, Thomson Wadsworth Communication, pp. 107–108.

Adam Kendon (2004) , Gesture: Visible Action as Utterance, Cambridge University Press, 2004.

Fernando Poyatos (2002), Nonverbal Communication Across Disciplines: Paralanguage, Kinesics, Silence, Personal and Environmental Interaction, John Benjamins Publishing, 2002

R. A. Bolt (1980). Put that there: Voice and gesture at the graphics interface. Computer Graphics, 14(3):262–270, 1980.

S. Qu and J. Chai (2008). Incorporating Temporal and Semantic Information with Eye Gaze for Automatic Word Acquisition in Multimodal Conversational Systems. In Proceedings of the Conference on Empirical Methods in Natural Language Processing (EMNLP), pages 244-253, 2008.

S. Qu and J. Chai (2008). Beyond attention: The Role of Deictic Gesture in Intention Recognition in Multimodal Conversational Interfaces. In Proceedings of the International Conference on Intelligent User Interfaces (IUI), pages 237-246, 2008.

IESNA (2005) – Fundamentals of Lighting for Videoconferencing

_______________________