| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 671R1 (WP 2/16) |
|  |  | English only Original: English |
| Question(s): | 12/16 | Geneva, 21 November - 2 December 2011 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Editor H.AMS |  |
| Title: | H.AMS "Advanced multimedia systems for next generation and other packet-switched networks" (New): Updated draft |  |

This document provides an updated draft H.AMS relative to AMS-004. The following changes have been included in this draft:

1/	Modifications agreed at the Andover meeting in July 2011 as recorded in §6.4, TD 606/WP2.

2/	The discussion on Interface K at the current meeting as recorded in TD 656/WP2 (Q12/16 report) as Editor’s note 2 in clause 8.7.

3/	Obvious editorial improvements such as addition of abbreviations, clause numbering, alignment with the designated template, use of Italic for Editor’s notes. These are not all indicated with the revision mark.

CONTENTS

1	Scope	3

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	4

4	Abbreviations and acronyms	5

5	Conventions	6

6	System description	6

6.1	System architecture	6

6.2	Information flow	7

6.3	System profiles	7

7	System components	7

7.1	Container	8

7.2	Applications	8

7.3	Service Node	8

7.4	Application Servers	8

8	Signalling procedures	8

8.1	Container registration	8

8.2	Application registration	8

8.3	Session establishment	8

8.4	Application invocation	8

8.5	Application handover	8

8.6	Session termination	8

8.7	Application control and indication	8

9	Media procedures	8

10	Interoperation with non-AMS devices	8

11	Optional enhancements	8

12	Maintenance	8

ITU-T Recommendation H.AMS

AMS: Advanced multimedia systems for next generation and other packet-switched networks

AAP Summary

[TO BE PROVIDED BEFORE CONSENT]

Summary

This Recommendation describes …

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

This Recommendation covers the technical requirements for an Advanced Multimedia System for NGN and other packet-switched networks (AMS).

AMS is designed to be an open application platform, specifying the underlying communications methods and protocols, but leaving the exact implementation of the services to the end-equipment manufacturer.

AMS is characterized by the following features among others:

- Flexible combination of various media for a multimedia communication session

- Decomposed architecture, namely separation of control and media processing

- Flexible use of distributed input/output devices

In this way, AMS intends to enable rich-media communication between a vast array of devices, allowing users to take advantage of enabled end-points on an ad-hoc basis.  Additionally, through interworking functions, it is intended that access to certain functions of the AMS infrastructure can be enabled from non-AMS devices and end-points.

AMS represents the user to the network as a logical entity where several facets of the communication could be split over a number of physical devices; for example, in a full audio and video conference, the video portion could be delivered on a conventional television set, whilst the audio is delivered through a mobile cellular telephone.

This Recommendation describes the components of AMS. This includes the AMS Assemblage, Devices, Application Servers, Service Nodes and the Container comprising three elements; the Application Registry, the Orchestration Manager and Transport Agent. Control messages and procedures within this Recommendation define how these components communicate. Detailed descriptions of these components are contained in clause 6.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

# 3	Definitions

{Editor’s Note: The relationship of the service node, network services and facilities nodes and assemblages were agreed to be updated in H.AMS and ideally the relationship be captured in a figure, in that document. Further contributions are solicited in this area.}

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Application Servers: Application servers are elements on the network that provide common services, for example IPTV, interactive gaming, multipoint conferencing.

3.2.2	Application: The Application is the element in the AMS environment that represents one aspect of communication, e.g. Voice, Video.

3.2.3	Application Set: An Application Set is a group of Applications required to create a complete communication experience.  For example, in a user interaction requiring voice, video and text, the three discrete applications would comprise a single Application Set.

3.2.4	Assemblage: The AMS Assemblage is the set of AMS elements that represent the logical association between the elements required for the user interaction.  For example, an AMS, a video conferencing terminal consisting of distributed voice and video elements would be referred to as an AMS Assemblage.

3.2.5	Container: The container is the entity that represents the user to the network.  It comprises three elements:

- Application Registry

The Application Registry is the entity where individual Applications register to declare their availability to a particular container.

- Orchestration Manager

The Orchestration Manager coordinates events between the Applications in order to create an AMS session.

- Transport Agent

The Transport Agent manages the signaling between AMS Assemblages

3.2.6	Interface “A”: This interface refers to the communication control between two AMS Assemblages.

3.2.7	Interface “B”: This interface refers to the communication control between the Applications within the AMS Assemblage and the Container function.

3.2.8	Interface “C”: This interface refers to the media stream between two AMS Assemblages.

3.2.9	Interface “E”: This interface refers to the network access control between the Assemblage and Service Node for use of the network resources.

3.2.10	Interface “K”: This interface refers to the Application control between two Applications.

3.2.11	Session: A session refers to communication between two AMS Assemblages, which may involve one or more communicating applications.

3.2.12	Service Node: The Service Node is a network entity that enables the container to establish communication with a remote entity, facilitate NAT/FW traversal, and provide other network-based services.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AMS | Advanced Multimedia System |
| --- | --- |
| FW | Firewall |
| NAT | Network Address Translation |
| NGN | Next Generation Network |
| SN | Service Node |

<Include all abbreviations and acronyms used in this Recommendation>

# 5	Conventions

In this Recommendation, the following conventions are used:

- "Shall" indicates a mandatory requirement.

- "Should" indicates a suggested but optional course of action.

- "May" indicates an optional course of action rather than a recommendation that something take place.

References to clauses, sub-clauses, annexes and appendices refer to those items within this Recommendation unless another text is explicitly listed.

# 6	System description

{Editor’s note: Contributions are solicited in this area.}

## 6.1	System architecture

AMS system entities and interfaces are shown in Figure 1.

Figure 1/H.AMS: AMS Entities and Interfaces

{Editor’s note: Home network and NAT/FW traversal aspects are to be studied. Even if Applications are within the same Assemblage, data transfer between Applications may traverse one or more service provider networks. As an example, between an Application on a mobile phone and an Application in a home, the Application data may transfer over a 4G wireless connection to the mobile operator and then over a broadband connection back to the Application within the home[§3, TD 593/WP2].}

## 6.2	Information flow

{Editor’s note: The figures and descriptions in [C.676] for a regular communication case and an Application handover case can be here if so agreed.}

## 6.3	System profiles

{Editor’s note: A set of  system profiles may be defined according to the system capabilities, namely Assemblage functionalities and/or available network resources.}

# 7	System components

{Editor’s note: Contributions are solicited in this area.}

## 7.1	Container

## 7.2	Applications

## 7.3	Service Node

## 7.4	Application Servers

# 8	Signalling procedures

{Editor’s note: Contributions are solicited in this area.}

## 8.1	Container registration

## 8.2	Application registration

## 8.3	Session establishment

## 8.4	Application invocation

## 8.5	Application handover

## 8.6	Session termination

## 8.7	Application control and indication

{Editor's note 1: Descriptions of various type of C&I signals and their transport methods are here. These signals may be carried over a media path, through Interface K, or through the Container with other AMS signalling. We need carefully consider the appropriate transmission method on a signal by signal basis[§3 in TD 593/WP2].}

{Editor’s note 2: As to Interface K, our direction is to represent the message in XML and to use TCP plus application-layer retransmission of messages for transport [TD656/WP2, Q12 report].}

# 9	Media procedures

{Editor's note: Method(s) of media transfer through Interface C and other media handling are to be here.}

# 10	Interoperation with non-AMS devices

{Editor’s note: Contributions are solicited in this area.}

# 11	Optional enhancements

{Editor’s note: Contributions are solicited in this area.}

# 12	Maintenance

{Editor’s note: Contributions are solicited in this area.}

Annex A

<Placeholder: Annex Title>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Appendix I

<Placeholder: Appendix Title>

(This appendix does not form an integral part of this Recommendation)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

_____________________