| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4197 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposal for editorial and technical improvements of F.USN-SM |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes some editorial and technical comments to improve F.USN-SM. They are given in revision marks in the up-to-date F.USN-SM, TD 677 (WP 2/16), released at the November 2011 meeting of ITU-T SG 16.

__________________

Justification of this Recommendation:

(Editor’s note)  This is not a part of the main body of F.USN-SM and was initially prepared for information. It is because it could keep used for information until its final state.

Gap analysis on smart metering of ITU and other SDOs

There are two related activities for smart metering issues in the ITU and other SDOs.

- ITU-T

In ITU-T, there is an activity to deal with a topic about smart metering. The title is “Smart metering using connected objects in ubiquitous home networking environments,” and deals with the concept, characteristics, scenario and functional architecture for smart metering in telecommunication perspectives.

- ETSI

ETSI TC M2M (Machine-to-Machine) published Smart Metering Use Cases. This introduces M2M application for smart metering and examples, and it describes the use cases for the smart metering M2M applications.

However, these two activities do not deal with USN capabilities for supporting smart metering services.

Contents

Pages

1	Scope	5

2	References	5

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	6

4	Abbreviations and acronyms	6

5	Conventions	6

6	Overview of smart metering	6

6.1	Smart grids and smart metering	6

6.2	Technical overview of smart metering	8

7	Smart metering service scenarios	9

7.1	Scenario I: regularly scheduled remote meter reading	9

7.2	Scenario II: on-demand remote meter reading	9

7.3	Scenario III: demand response	10

7.4	Scenario IV: tariff configuration	11

7.5	Scenario V: meter reading data aggregation	12

8	Requirements of smart metering services	12

8.1	Reliable information delivery	12

8.2	Real-time delivery of meter reading data	13

8.3	Bi-directional communication between meters and operators	13

8.4	Security support	13

8.5	Meter reading data processing	13

9	USN-based smart metering services	13

10	USN capabilities for smart metering services	14

10.1	Reliable transmission	14

10.2	Scalability	15

10.3	Mobility support	15

10.4	Delivery latency	15

10.5	Fault detection and recovery	15

10.6	Security	15

10.7	Connectivity	15

10.8	Unicasting & Multicasting	15

10.9	Data aggregation	15

Bibliography	16

Draft new Recommendation ITU-T F.USN-SM

Capabilities of Ubiquitous Sensor Network (USN) for supporting requirements of smart metering services

AAP Summary

To be provided before Consent.

Summary

This Recommendation identifies capabilities of Ubiquitous Sensor Network (USN) for supporting requirements of smart metering services. To this end, an overview of smart metering is described with clarification between smart grids and smart metering. This Recommendation takes into account a few typical use case scenarios of smart metering and identifies technical requirements and USN-based smart metering services to support the use cases. Finally this Recommendation defines USN capabilities based on the identified requirements to provide smart metering services.

Keywords

Smart grid, smart metering, USN

Scope

The main purpose of this Recommendation is to identify capabilities of Ubiquitous Sensor Network (USN) which supports requirements of smart metering services. The scope of this Recommendation covers the following:

- Overview of smart metering;

- Smart metering use case scenarios;

- Requirements of  smart metering services;

- USN capabilities for supporting requirements of  smart metering services;

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]	ITU-T Recommendation Y.2221 (2010), Requirements for support of ubiquitous sensor network (USN) applications and services in the NGN environment

Definitions

Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.2	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.3	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.4	smart grid [ITU-T FG-Smart]: An electricity network that can intelligently integrate the actions of all users connected to it - generators, consumers and those that do both - in order to efficiently deliver sustainable, economic and secure electricity supplies.

3.1.5	ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at any time, and where the information is generated by using context awareness.

Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	smart meter: A premise device to monitor and control electrical power, gas and water usage of home appliances based on demand response information from home appliances

3.2.3	smart metering: An operation to provide information to consumers and smart metering operator about energy consumption. The information includes how much energy the consumers are consuming or generating and how much it costs

3.2.4	demand response: A smart metering feature that allows consumers to reduce or change their use patterns of electricity, gas and water during peak demand usually in exchange for a financial incentive

Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ETSI | European Telecommunications Standards Institute |
| --- | --- |
| M2M | Machine-to-Machine |

(Editor’ Note) The Abbreviations and acronyms will be added.

Conventions

None.

Overview of smart metering

Smart metering is an operation to provide consumers with useful information about consumption of energy, such as electricity, gas, and water. It has more advanced features than simple automated meter reading. Also, smart metering can be part of a smart grid, but only smart meters do not constitute a smart grid. Therefore, this section introduces smart grids, relationships between smart grids and smart meters, and technical issues of smart metering.

Smart grids and smart metering

Smart grid is a type of electrical grids, which attempt to efficiently deliver reliable, economic, and sustainable electricity services through prediction and response to the behavior and actions of all electric power consumers (see Figure 1). It is because existing systems of power grids cannot keep pace with modern challenges, such as:

- Security threats from cyber attack

- National goals to employ alternative power generation sources

- Conservation goals that seek to lesson peak demand surges so that less energy is wasted

- High demand for an electricity supply

- Digital controlled devices that can alter the nature of electrical load and result in electricity demand

Therefore, the smart grids which can overcome the challenges provide a lot of benefits as follows:

- Enhancement of reliability

- Reducing peak demand

- Shifting usage to off-peak hours

- Lower total energy consumption

- Actively managing electricity charging

- Actively managing other usage to respond to other renewable resources

Smart grids technologies have already used in other applications, such as manufacturing and telecommunications. In general, smart grid technology may be divided into seven areas, such as integrated communications, sensing and measurement, smart metering, advanced components and control, improved interfaces and decision support, and smart power generation. In other words, smart meters are part of the smart grids as key components, and also smart metering is one of crucial features for the smart grids.

Figure 1: Smart Grid Architecture [b-NIST]

Technical overview of smart metering

Not only governments and utility companies, such as electricity, gas, and water suppliers, but also researchers have been interested in automatic meter reading based on communication system, known as smart metering because smart metering can bring following benefits to customers and governments as well as utility companies, for examples:

- Lower metering cost

- Energy savings for residential consumers

- Reliability of supply

- Various pricing schemes to make new consumers attracted

- Easier detection of fraud and outage

- Automated billing

Smart metering performs metering and exchanging meter information between smart meters and utility companies. Various technologies for metering and exchanging meter information have been implemented. Power line communication has been used as means for delivering electricity power to consumers and transmitting gas and water measurements to the utility providers. In addition, messaging over GSM network, for example, has been utilized for an automatic meter reading system.

Sensor network technologies may be used for metering and collecting information of utility usages, and communication networks can be used for exchanging the information. Figure 2 depicts an overall diagram of smart metering systems. The meter information, obtained from home appliances by sensor nodes, is collected and delivered to operators of utility companies.

The operators manage the collected information and inform consumers of variable pricing information or enforced load control messages. Such operators’ action leads to consumers’ responses to reduce energy consumption.

Figure 2: Overview of smart metering

Smart metering service scenarios

Scenario I: regularly scheduled remote meter reading

Scenario I describes procedures that meter reading data are delivered to smart metering operators every scheduled time.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- According to the schedule, smart meters measure the amount of energy consumption from home appliances and obtain meter reading data.

- The meter reading data are delivered to a smart metering operator.

Figure 3: Regularly scheduled remote metering

Scenario II: on-demand remote meter reading

Scenario II describes procedures that meter reading data delivered to smart metering operator on demand.

- When a smart metering operator  requires collecting meter reading data from the smart meters, the operator send the message to smart meters for collecting meter reading data.

- The smart meters verify the operator’s message and then obtain meter reading data according to requirements of the message if the message is valid.

- The meter reading data are delivered to the smart metering operator.

Figure 4:  On-demand remote meter reading

Scenario III: demand response

Scenario III describes how customers respond metering operator’s demands.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- When the number of consumers using home appliances simultaneously is on the increase, the total amount of energy consumption also steeply increases.

- Information about the increasing amount of energy consumption is being reported to the smart metering operator every scheduled time, and the cost of the energy may also be increased by tariff policies depending on supply and demand.

- (a) The smart metering operator sends a message which may inform customers of the increased cost.

(b) The smart metering operator sends load control messages to enforce to reduce energy consumption or shut off by force due to a reason of operational policy.

- (a) The smart meters display the message to the consumers, and the consumers may decide to reduce energy consumption, or the home appliances may be switched off by a consumer.

(b) When receiving the load control messages, the smart meter display the message and shut off. The connected home appliance is automatically switched off.

Figure 5: Demand response

Scenario IV: tariff configuration

Scenario IV describes how cost of energy consumption is decided and configured into the meters.

- Smart meters may be scheduled to obtain meter reading data and transfer them to a smart metering operator.

- The smart meters regularly deliver meter reading data to the metering operator according to the schedule.

- The cost of the electricity may be dynamically changed by tariff policies depending on supply and demand.

- When the cost is changed, a message including changed cost information is delivered to all of the smart meters.

- The changed cost information may be shown on their displays, and the changed cost is applied.

Figure 6: Tariff configuration

Scenario V: meter reading data aggregation

This scenario describes procedures that meter reading data, obtained every scheduled time by smart meter, are collected at aggregating smart meter (or at aggregating smart metering gateway) and then delivered to metering operators.

- Smart meters may be scheduled to obtain meter reading data. According to the schedule, smart meters measure the amount of energy consumption and obtain meter reading data from home appliances.

- The obtained meter reading data are collected in an aggregating smart meter (or aggregating smart metering gateway).

- Finally, meter reading data are transferred to a smart metering operator.

(Editor’s Note) The data aggregation for batch process can give negative effects to smart metering real-time services. This fact need to be took into account.

(Editor’s Note) The scenario V is not enough to explain metering data aggregation so required to be updated with a figure in the further contributions.

Requirements of smart metering services

Reliable information delivery

In the smart metering services, meter reading data can be frequently transferred from distributed meters, and also operators can send many control messages to smart meters. When delivery is failed, the operators cannot collect all of meter reading data from smart meters. As a result, missing meter reading data would lead to unwanted results. Therefore, the smart metering services are required to provide reliable information delivery, such as meter reading data and control messages described in the scenario.

Real-time delivery of meter reading data

Demands of smart metering operators require real-time responses of consumers. Therefore, meter reading data are required to arrive at the operators within a given amount of time. In the scenario I-3 and II-3, if the meter reading data are not delivered within the given time, the operators cannot obtain real-time results of smart metering. Therefore, smart metering services are required to guarantee real-time delivery.

Bi-directional communication between meters and operators

Meter reading data are delivered from smart meters to smart metering operators as shown in the scenario I-3, II-3, and III-2, and the control messages are also sent from the operators to the smart meters vice versa (in scenario III-3 and IV-3). For doing this, the smart metering services are required to support bi-directional communication between meters and operators.

Security support

Operators need to control meters in order that the meters take actions according to the indication of operators. In this case, the operators should be authorized by smart meters for taking secure actions (scenario II-2).

Because metering information may include privacy information, it should not be revealed to unauthenticated third party. Therefore, the metering information is required to be secured to unauthenticated third party.

[Contributor’s Comment] there are two similar keywords: meter reading data and metering information. Clarify them and resolve this for making consistency.

Meter reading data processing

Either operators or smart meters should be able to perform data processing on meter reading such as filtering, validation and aggregation. In some case, data mining processing is necessary, for example, analyzing patterns and predicting some events.

USN-based smart metering services

[Contributor’s Comment] It is proposed to relocate this clause 9 into clause 6 as clause 6.3. The following clauses are matched well:

- Clause 7 – Smart metering service scenarios

- Clause 8 – Requirements of smart metering services

- Clause 10 - USN capabilities for smart metering services

But clause 9 describes functional components and a typical configuration of USN to support smart metering services. It looks like an overview description in terms of USN. This clause had better go into clause 6, “Overview of smart metering” as clause 6.3.

The smart metering services shown in Figure 7 require technological facilities to support metering and exchanging metering information between smart meters and metering operators in utility control center. Basic characteristic of USN applications and services is gathering data by sensor networks and transmitting data to a remote server through communication networks. Also, control information is transmitted to sensor networks from the remote server. Furthermore, if USN middleware is deployed, the middleware can provide USN applications or sensor networks with various functions such as data query, data mining, event processing, sensor network metadata directory service, data filtering, context-aware rule processing and sensor network management. This feature of USN can be applied to the smart metering services. Sensor nodes in sensor networks have capabilities of metering and delivering metering information to a server of utility control center. USN middleware can provide functions which are required by on-demand remote meter reading scenario and demand response meter reading scenario.

A smart metering system can be implemented by sensor nodes and USN middleware as follows.

- Smart metering sensor nodes: each sensor node has capabilities of smart metering, so sensor nodes process smart metering information for the smart metering procedures, such as meter reading collection and tariff setting.

- Smart metering gateway: A gateway just delivers information between the sensor nodes and metering operators. In case a gateway has capabilities of smart metering, it collects meter reading data and then delivers to the metering operator(s).

- USN middleware: USN middleware provides data filtering, data query, data mining and context-aware rule processing, etc. These features of USN middleware satisfy meter reading data processing requirement and can be applied to on-demand remote meter reading scenario and demand response meter reading scenario.

Figure 7: Smart metering service based on USN

USN capabilities for smart metering services

Reliable transmission

Smart metering requires reliability on metering information delivery to get correct results. Therefore, USN is recommended to guarantee reliable transmission for data and message delivery.

Scalability

New smart meters (sensor nodes) can be added to, or one of the existing meters (sensor nodes) can be removed from an existing smart metering group (sensor networks). Such a change should not degrade the performance of the smart metering.  Therefore, sensor networks in USN are required to support scalability.

Mobility support

In some cases, a consumer can change the positions of home appliances. This change may involve the changes meters’ position in intra-sensor network or inter-sensor networks. Therefore, sensor networks in USN are recommended to support mobility of smart meters (sensor nodes).

Delivery latency

Meter reading data should be delivered within a given time. For this requirement, USN is recommended to guarantee data delivery within a given time.

Fault detection and recovery

There is a possibility of link failure between nodes due to the characteristics of wireless transmission when sensor nodes use wireless communications,. Such a failure can give negative effects on reliability and delivery latency of the smart metering services. USN is recommended to detect faults and recover from the faults.

Security

Meter reading data and messages may be transferred to/from operators and meters by hop-by-hop transmission among sensor nodes. Therefore, hop-by-hop security among sensor nodes is recommended to be implemented.

Connectivity

Meter reading data, obtained from each sensor node, are collected by a gateway, and then the collected meter reading data are delivered to smart metering operators through transport networks.  In addition, control messages of the operators are also delivered to each sensor node. For doing this, connectivity between sensor nodes, connectivity between sensor nodes and the gateway, and connectivity between the gateway and outer networks are recommended to be guaranteed.

Unicasting and Multicasting

A unicast networking service is required to be supported among sensor nodes in order that meter reading data are delivered to smart metering operators. Furthermore, control messages of the metering operators should be delivered to all of sensor nodes. Therefore, multicast networking support is also required.

Data aggregation

Meter reading data from smart meters is collected and aggregated at smart meter or smart metering gateway. And then they are transferred to the smart metering operator(s).

Bibliography

[b-ESTI TR 102.691]	ESTI TR 102.691 v1.1.1, “Machine-to-Machine communications (M2M); Smart Metering Use Cases,” May 2010.

[b-ITU-T Smart-I-6]	ITU-T FG-Smart Grid, Smart-I-6, “Smart metering using connected objects in ubiquitous home networking environments,” June 2010.

[b-Khalifa] 	T. Khalifa et. al., “A Survey of Communication Protocols for Automatic Meter Reading Applications,” IEEE Communications surveys and tutorials, Vol. PP, Issue 99, January 2010.

[b-NIST] 	NIST Special Publication 1108, NIST Framework and Roadmap for Smart Grid Interoperability Standards, Release 1.0, January, 2010

[TBD]

__________________