| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4201 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |  |
| Title: | F.TPS-reqs: Proposed texts for scope and additional requirements for capability probing |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to clarify the scope of Recommendation in the point of application as agreed in the previous meeting. In addition, we also propose several requirements for probing the capabilities of remote telepresence system.

Introduction/Discussion

According to the meeting report TD655 of previous November Geneva meeting, it was agreed that the F.TPS-Reqs requirements on telepresence systems generally apply to the telepresence system architecture described in F/H.TPS-Arch, and do not necessarily apply to all conformant implementations. If so, we think it is appropriate to put the agreed texts to the scope section of F.TPS-Reqs Recommendation for further contributions and preventing misunderstandings.

In addition, telepresence system vendors may compose capability of their systems for their own purpose. This may burden on interoperating with each other. Hence, it is essential to exchange capability information for establishing telepresence session. Of course, mandatory information for session setup will be exchanged and negotiated during session establishment, but it may just include the capability information for making a session and does not include additional features that are not involved in that session. Furthermore, it is useful to figure out what kinds of features are supported by the remote system. If each system shares its full capability, a user can decide which other services are available. This capability information can be acquired within a session or outside of session.

In some cases, a specific telepresence session may require specific additional features such as instant messaging, document sharing, TTS (type to speech), and so on. It is also possible to pre-specify minimum audio/video requirements for better immersive experience. In these cases, it is useful if telepresence system provide the capability requirements prior to join the session. A joining user can check whether his system can join the session prior to actual joining procedure.

In case of SIP, it provides capability probing mechanism that using OPTIONS message. By use of this message, a user can check the capabilities of end point outside of session.

The capability information may include spatial information, supporting codecs and its attributes, additional extended functions. By sharing this capability information among participating systems, each user can check whether it is possible to communicate with specific service or not prior to service initiation.

Proposals

Proposal 1

We propose to add following sentences at the end of Scope section of F.TPS-Reqs.

[Begin Proposed Texts]

This Recommendation provides requirements for constructing telepresence system architecture, and it does not necessarily apply to all conformant implementations.

[End Proposed Texts]

Proposal 2

The Cal02 may include the proposed new requirements implicitly, but these requirements differ from existing Cal02 in the point of the behaviour and its purpose. The Cal02 aims at spatial coordinated rendering, but this requirements focus on sharing and gathering capability information of each end point for interoperation on additional features as well as basic telepresence functionalities. This probing procedure may be preceded prior to establishment of session and may be performed through session establishment.

On using gathered information, a telepresence system can present what features are available with each remote system.

[Begin Proposed Texts]

CalXX: The telepresence system shall support of probing mechanism for acquiring capability information of end point. The capability information may include spatial information, supporting codecs and its attributes, additional optional functions such as instant messaging, whiteboard, TTS, and so on.

CalXX: The telepresence system should support of displaying remote systems capabilities on user’s request.

[End Proposed Texts]