| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4213 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | F/H.TPS-Req: Addressing open editor’s notes |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution discusses a number of editor’s notes and proposes updates to the draft Recommendation text based on the notes.

# Discussion

This contribution discusses a number of editor’s notes and proposes changes to the draft Recommendation based on them. It also proposes a number of editorial enhancements to the text. The proposed changes are described by {Contributor’s notes: …} below.

# Proposal

It is proposed to incorporate the marked up changes below in draft F.TPS-Req. Changes are marked up against TD 649R1/WP2 from the November/December ITU-T Q.5/16 meeting.

Note to editor: The contributor’s notes should be deleted when adding the changes to the draft.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

…

# 1	Scope

# {Contributor’s note: It is proposed to delete the editor’s note below as the text is no longer an initial draft. Proposals to update the section can be made without the need for an editor’s note.}

This Recommendation provides definitions, use cases and functional requirements for telepresence systems. Specific requirements include those for:

- Call control functions

- Media control functions

- Conference control functions

- Collaboration

- Inter-operability

- Network

- Security

{Contributor’s note: At the November/December 2012 meeting there was some discussion regarding the scope of the requirements and how they related to deployment (see editor’s notes in clauses 10 and 11. The Contributors believe that the requirements in this document are for the purposes of protocols (and possible profiles) related to Telepresence systems. The requirements do not indicate mandatory features that must be in a deployed Telepresence system. The following wording is proposed to capture this point.}

The requirements in this Recommendation relate to functions that a Telepresence system shall be able to support. That is, a Telepresence system should be able to provide the functions in its design or by utilising existing mechanisms. It does not mean that the requirement must be implemented. The requirements do not mandate the deployment of a particular function. For example: Whilst a telepresence system should support mechanisms for assured QoS, it may be deployed to a network where these are not needed.

{Contributor’s note: It is proposed to add to text below to address the editor’s note in section 8.1 and to address the discussion from the previous Q.5/16 meeting regarding the scope of the requirements.}

Unless the requirement mentions a specific function or device the requirements listed in this Recommendation apply to a Telepresence system as a whole. The inclusion of a requirement does not necessarily mean that something new is needed for the support of Telepresence. For example whilst there may be a requirement to support QoS for Telepresence, ultimately existing QoS mechanisms may be utilised for this.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

{Contributor’s note: It is proposed to add the following missing references.}

[ITU-T H.239]	ITU-T Recommendation H.239 (09/05), Role management and additional media channels for H.300-series terminals.

[ITU-T H.241]	ITU-T Recommendation H.241 (05/06), Extended video procedures and control signals for H.300-series terminals.

[ITU-T H.243]	ITU-T Recommendation H.243 (10/05), Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 1920 kbit/s.

[H/F.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[H.TPS-AV]	ITU-T Recommendation X.yyy (date), Audio/Video Parameters for Telepresence systems.

[IETF CLUE REQ]	IETF draft-ietf-clue-telepresence-requirements-01, Requirements for Telepresence Multi-Streams.

[IETF CLUE UCS] IETF draft-ietf-clue-telepresence-use-cases-02, Use Cases for Telepresence Multi-streams.

# {Contributor’s note: The CLUE use cases draft is now version 2. The Contributors have reviewed both version 1 and version 2 and do not believe that changes are needed to F.TPS-Req as a result. Therefore a simple update of the version is proposed.}

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

{Contributor’s: It is proposed to move the definitions to the appropriate section and to move Natural Response to keep the section in alphabetic order. It is also proposed to remove the NOTE regarding the living list from the Telepresence definition and the editor’s note regarding other definitions.}

Natural response:  Interaction between local and remote participants that is not impeded by the delay introduced by the telepresence system.

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Actual sized: A rendered figure in a display is the same size as if the person is in the room.

3.2.2	Natural response:  Interaction between local and remote participants that is not impeded by the delay introduced by the telepresence system.

3.2.3	Telepresence: An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

# 4	Abbreviations and acronyms

{Contributor’s note: It is proposed to add the following missing acronyms.}

This Recommendation uses the following abbreviations and acronyms:

| 3D | Three Dimensional |
| --- | --- |
| IPv4, IPV6 | Internet Protocol Version 4, Version 6 |
| MCU | Multipoint Control Unit |
| NAT | Network Address Traversal |
| PC | Personal Computer |
| QoS | Quality of Service |

# 5	Conventions

{Contributor’s note: it is proposed to remove the template note below and replace it with “none” as there is currently no particular style used.}

None.

# 6	Scenarios and Use cases

The use cases in this clause illustrate various telepresence configurations for the purposes of identification of requirements. These configurations apply to scenarios where telepresence is in use and do not include legacy systems (i.e. non-telepresence systems) unless specifically indicated in the use case. They do not imply the need for mandatory support of any particular configurations.

## 6.1	Use Case 1: Symmetric point-to-point meeting

{Contributor’s note: It is proposed to add generic text above addressing the editor’s note regarding legacy systems. With respect to the number of screen and cameras the Contributors believe that the fact there is more than one is not relevant. This use is about “symmetry”, systems that utilise one screen and camera are relevant to this case. It is proposed to add some text regarding this. With regards to the comment regarding sensor, the term “sensor” is not used in the text. Therefore we believe this may have been added erroneously. As the editor’s note is addressed we further propose the deletion of the editor’s note.}

In this case each of the two sites has an identical number of screens, with cameras having fixed fields of view, and one camera for each screen.  The number of screens or cameras is not relevant so long as both sites have the same number. The sound type is the same at each end. Figure 1 illustrates the use case.

Two-screen Telepresence Endpoint

Two–screen Telepresence Endpoint

IP Network

Figure 1 – Use Case 1: Symmetric point to point meeting

For further information see clause 3.1 / [IETF CLUE UCS].

## 6.2	Use Case 2: Asymmetric point-to-point meeting

In this case, each site has a different number of screens and cameras than the other site. The number of screens or cameras is not relevant so long as both sites have a different number. Figure 2 illustrates the use case.

Two-screen

Telepresence Endpoint

IP Network

Three-screen-- ScreenRec mean that ITU-T Q.nerein.

Telepresence Endpoint

Figure 2 – Use Case 2: Asymmetric point to point meeting

For further information see clause 3.2 / [IETF CLUE UCS].

…

## 6.4	Use Case 4: Presentation

{Contributor’s note: To address the editor’s note in this section it is proposed to update the diagram to use the term “Presentation capture” and to include text in the paragraph below to illustrate example sources. As such it is further proposed to delete the editor’s note.}

This use case builds on the previous use cases that in addition to the video and audio streams showing the participants, additional streams are used for presentations. For example the additional stream may be for content sources such as: document cameras, electronic whiteboards, PC or instant messaging. Figure 4 illustrates the use case.

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Figure 4 – Use Case 4: Presentation

For further information see clause 3.4 / [IETF CLUE UCS].

## 6.5	Use Case 5: Heterogeneous systems

{Contributor’s note: It is proposed to provide additional text regarding this use case and legacy systems to address the editor’s note in this section. This additional text is in keeping with section 3.5/[IETF CLUE UCS] which states “(We are not here talking about legacy systems, but rather systems built to participate in such a conference, although they are single stream only.)”. It does not matter what the underlying technology is to determine to whether a device is “legacy” or not, the deciding factor is whether it supports telepresence related protocols (or protocol extensions). }

This use case illustrates the common meeting scenario for people to join the conference from a variety of environments, using different types of endpoint devices.  In a multi-screen immersive telepresence conference may include someone on a PC-based video conferencing system, a participant calling in by phone, and someone on a handheld device. This use case assumes that the different endpoint devices are able to participate is signalling related to Telepresence systems. Legacy systems that do not understand telepresence related protocols (or protocol extensions) are not in scope. Figure 5 illustrates the use case.

-

Two-screen

Telepresence Endpoint

IP Network

Three-screen

Telepresence Endpoint

Document CameraDocument Camera

Presentation

DisplayPresentation

Display

Document CameraDocument Camera

Presentation

DisplayPresentation

Display

Three-screen

Telepresence EndpointThree-screen

Telepresence Endpoint

Personal Computer

Mobile Phone

Figure 5 – Use Case 5: Heterogeneous systems

For further information see clause 3.5 / [IETF CLUE UCS].

…

## 6.8	Use case 8: Legacy Interworking

{Contributor’s note: As use case 5 has been updated to clarify its relationship to legacy devices it is proposed to clarify that this use case not only applies to videoconferencing terminals but also applies to heterogeneous devices, e.g. voice only devices.}

This use case describes a case where a legacy videoconferencing endpoint (or other heterogeneous device such as voice only endpoint) is involved in the telepresence session. The legacy endpoint does not understand additions related to telepresence functionality and interworking is required to communicate between the telepresence and legacy endpoints. Figure 8 illustrates the use case.

Figure 8 – Use Case 8: Legacy Interworking

{Contributor’s note: This is proposed to be added due to require Int01 from clause 9.1}

This use case may also utilise other legacy equipment such as a legacy H.323 MCU in order to participate in a conference.

# 7	User experience requirements

{Editor’s Note: We need to consider if a system perspective is required, in other words use “system requirements” to capture “user experience requirements”. We also need to consider the difference between performance requirements and functional requirements, and if user experience in terms of performance metrics is out of scope.  Further contributions are needed to describe these requirements more precisely, and to limit them to not require detailed performance metrics which may not be available in the near-term. Treat the following as examples for now.

User01: The telepresence system should support actual size high-definition (includes 1080p, 1080i, etc.) imaging of video conference participants at the remote end. The support of actual size is considered an advanced Telepresence feature.

{Contributor’s note: It is proposed to add a definition of “Actual Size” to the definitions section, incorporate the advanced support in the text and to modify the editor’s note.}

{Editor’s Note: The supported HD modes need further consideration.}

User02: The telepresence system should provide a natural eye contact experience.

{Contributor’s note: It is proposed to remove the last sentence and the editor’s note regarding synchronisation. User03 is about rendering a speaker correctly rather than synchronisation. User04 already covers synchronisation.}

User03: The telepresence system should support a means of locating the speaker in a remote meeting room so that local participants can readily identify the position of different remote speakers.

User04: The telepresence system shall enable synchronization of audio and video from the remote location. Synchronization includes “lip-sync” and the sequencing of multiple media streams.

{Contributor’s note: With respect to the editor’s note below the Contributors believe that the key requirement is that a telepresence system should provide a control function to allow a user to perform an operation with a minimum of input. Whether this is provided via a touch-screen, voice command etc is up to a vendors own user interface. Therefore it is proposed to re-word “one-touch with the description below.

User05: The telepresence system should provide a control function to allow a user to perform an operation with a minimum of input thus enabling convenient centralized management and one-touch smart control capabilities.

{Contributor’s note: It is proposed to capture the editor’s note as part of the requirement.}

User06: The telepresence system should provide harmonized environment for assuring immersive presences of remote users. Typically in order to provide immersive and realistic experiences to users, some environmental factors are aligned. These environmental factors for example include: interior design such as shape of desk, wall colour, illumination, position of light, etc.

{Contributor’s note: There appears to be superfluous text in User07.}

User07: The telepresence system should support multiple video sources and multiple displays in each meeting room. The overlap of the scene from neighbouring cameras should be minimized. The output from the neighbouring cameras should provide a seamless view of the conference participants.

User-08: The telepresence system shall provide natural response on facial expression and conversational voice.

User09: The telepresence system should support the receiver end-user switching of display position.

# 8	Functional requirements

## 8.1	Call control functions

Cal01: The telepresence system shall support interoperability with other devices that send or receive a different number of media streams.

Cal02: The telepresence system shall support the description, identification and negotiation of appropriate information to enable spatially coordinated rendering of audio and video streams.

{Contributor’s note: It is proposed to remove the editor’s note below as it is captured in section 1.}

## 8.2	Media control functions

Med001: The telepresence system shall support communication regardless of different aspect ratio.

Med002: The telepresence system shall support a method for describing the spatial arrangement of audio and video stream in a manner which enables a satisfactory reproduction at the receiver in a spatially correct manner. Furthermore it shall support a method to enable a satisfactory spatial matching between audio and video streams coming from the same endpoints.

This requirement encompasses the requirements REQMT-1, REQMT-2 and REQMT-3 as defined in [IETF CLUE REQ].

Med003: Where endpoints send and receive different numbers of media streams the telepresence system shall provide interoperability between them. This requirement encompasses requirement REQMT-9 as defined in [IETF CLUE REQ].

Med004: The telepresence system optionally support 3D multimedia stream.

Med005: The telepresence system optionally supports a means for describing 3-Dimensional multimedia stream.

{Editor’s Note: Inclusion of 3D requirements depends on appropriate contributions to support them.}

{Contributor’s note: The Contributors believe that the requirement should apply to multiple streams.}

Med006: The telepresence system should be able to add additional media streams, and remove existing media streams during the telepresence session.

{Contributor’s note: It proposed to formalise the reference to H.239 and H.241.}

Med007: The telepresence system should provide a suitable range of video frame rates, resolutions, and bit rates for presenting the shared data. For example those provided by ITU-T H.239, ITU-T H.241.

## 8.3	Conference control functions

{Editor’s Note:  These are examples for now, that will need further discussion.}

Con01: The telepresence system shall enable switching the video from the whole of the remote meeting rooms to another meeting room, or from partial images to another image.

Con02: The telepresence system shall enable the display of all remote participants at once, including remote participants (if any) from non-telepresence systems.

Con03: The telepresence system shall enable the conference chair person to select and view video from a given meeting room or from partial seats of a meeting room.

{Contributor’s note: It proposed to formalise the reference to H.243.}

Con04: The telepresence system shall support adding or removing an endpoint to an ongoing telepresence session. For example such as the procedures described by [ITU-T H.243].

## 8.4	Collaboration functions

Col001: It is optional for a telepresence system to provide a means of collaboration such as remote presentation, instant messaging, online documentation, white board and file transferring based on existing protocols.

Col002: The telepresence system shall support a captioning function.

## 8.5	Other functions

{Contributor’s note: It is proposed to add normative text to address the editor’s note.}

This sub-clause contains requirements that are not related to the other sub-clauses in clause 8.

Other01: The telepresence system shall be able to provide various statistical information which includes but is not limited to: media type, maximum number of endpoints, duration, and data volume of each media type.

{Editor’s note: we need more contributions on the precise list for reporting.}

# 9	Interoperability requirements

# {Contributor’s note: It is proposed to update the section below to make it more generic. According to the use cases it should be possible to for legacy devices (e.g. mobile phones as well as video terminals to participate in a conference. A new use case is added to clarify this. It is also proposed to clarify that these “legacy” devices may not perceive “telepresence”.}

## 9.1	Interoperation with traditional systems

This clause presents requirements for interoperation with legacy devices/systems. When used in a conference with Telepresence system these legacy devices may not perceive telepresence.

Int01: The telepresence system shall support that both traditional videoconference terminals and telepresence terminals can participate in a same conference.

Int02: The telepresence system should support that a traditional videoconference system can connect to the telepresence system in multiple cascaded conference mode.

Int0x: The telepresence system should allow audio only legacy devices to participate in a conference.

{Contributor’s note: It is proposed to remove the editor’s note below and incorporate text regarding the cascaded MCUs in use case 8 on legacy interworking.}

## 9.2	Interoperation between telepresence systems

{Contributor’s note: Clause 9.1 discusses interwork with existing video conference terminals, whereas the title of clause  9.2 relates to Telepresence systems. It is unclear from the requirement Int03 and the editor’s note that requirement Int03 actually belongs in this section. It is proposed to introduce a new requirement and modify the text of Int03.}

This clause presents requirements for interoperation with devices/systems that have been enhanced for telepresence.

Int03: The telepresence system shall enable audio and video terminals based on different open standards to participate in the same conference.

Int04: It shall be possible for telepresence systems based on different open standards to participate in the same conference and experience telepresence. For example: A telepresence enhanced SIP endpoint and a telepresence enhanced H.323 endpoint via control signalling gateway.

# 10	Network requirements

{Contributor’s note: It is proposed to delete the editor’s note below due to the text added to clause 1 regarding deployment.}

{Contributor’s note: It is proposed to modify the text of the requirements below to remove the term “is suggested” as this is typically not a phrase used for requirements. It is also proposed to incorporate the Editor’s note regarding best effort as an example.}

Net001: The telepresence system should enable the reservation network resource for assuring QoS for the telepresence session.

Net002: The telepresence system should enable adaptation of  the quality of media based on the network status. For example: Under best-effort network such as Internet which cannot provide strictly guaranteed network QoS, the telepresence system may need to manipulate its video quality based on network status.

Net003: The telepresence system should provide a means to traverse NATs or firewalls automatically to assure connectivity of telepresence services with minimal user operation.

{Editor’s Note:  This requirement needs more discussion.}

Net004: The telepresence system shall support both IPv4 and IPv6.

{Editor’s Note: This requirement needs more discussion. Use cases would be helpful.}

# 11	Security requirements

{Contributor’s note: It is proposed to delete the editor’s note due to the text introduced in clause 1 regarding optional deployment.}

Sec001: The telepresence system is suggested to support of security mechanism for securing sessions.

Sec002: The telepresence system is suggested to support of negotiation mechanism for making a secured session.

Sec003: The telepresence system is suggested to support both signal security and media security.

Sec004: The telepresence system shall be able to authenticate each end point.

__________________