| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4215 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Alignment with CLUE |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution proposes a number of updates to H.TPS-AV as a result of a new version of the IETF draft <draft–ietf-clue-framework>.

# Discussion

At the last ITU-T Q.5/16 meeting a number of parameters were introduced in to the H.TPS-AV draft based on the work in the IETF CLUE framework draft. This draft has now been updated to version 2 (draft-ietf-clue-framework-03).

As discussed at the previous meeting, on-going alignment of the parameters between the IETF CLUE work and the Q.5/16 work would be required. The Contributors have analysed the changes in version 2 and have noted the following changes that have an impact on H.TPS-AV:

- The directions related to the coordinate system have been clarified (See clause 5.3 / [IETF CLUE FW].

- The “Area of capture” attribute now consists of a set of four points rather than a set of ranges (See clause 5.4.4 / [IETF CLUE FW].

- The “Area of scene” attribute now consists of a set of four points rather than a set of ranges (See clause 5.6.1 / [IETF CLUE FW].

- The “Area Scale” attribute now has several values rather than a Boolean setting (see clause 5.6.2 / [IETF CLUE FW].

There was a large amount of text changes from version 2 to version 3 however these changes were largely updates to procedural descriptions rather than changes to the parameters themselves. However the encoding group attributes: maxBandWidth and maxVideoMbps were renamed to maxGroupBandwidth and maxGroupH264Mbps respectively. The individual encoding attribute maxVideoMbps was renamed to maxH264Mbps.

There was also an editor’s note regarding Figure 1 and 2 which related an example for Area of Capture. As this example and figures relate to the CLUE framework changes it is also proposed to update these.

These changes should be reflected in H.TPS-AV.

# Proposal

It is proposed to add the following changes indicated by the change marks to H.TPS-AV. Change marks are against TD 645R1/WP2 from the Geneva, Switzerland meeting 21 November – 02 December 2011 meeting.

…

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-03, Framework for Telepresence Multi-Streams.

[ISO 11664-2]		ISO 11664-2:2007 (CIE S 014-2/E:2006) (2007), Colorimetry -- Part 2: CIE standard illuminants

…

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	 Camera-Left and Right [IETF CLUE FW]: For media captures, camera-left and camera- right are from the point of view of a person observing the rendered media.  They are the opposite of stage-left and stage-right.

…

#### 7.1.1.3 General Encoding Information

##### 7.1.1.3.1 Overall maximum bandwidth

| Identity: | maxGroupBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to all encoded media of a particular type (audio or video) sent from the endpoint. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See “maxGroupBandwidth” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.1.3.2 Overall maximum macroblocks per second

| Identity: | maxGroupMpbs |
| --- | --- |
| Description: | This parameter indicates the maximum number of H264 macroblocks per second for all encoded video sent from the endpoint. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See “maxGroupH264Mbps” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

This clause describes parameters related to encoding information of a single video capture.

##### 7.1.2.5.1 Maximum Video Bandwidth

| Identity: | maxVideoBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to a single video encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See “maxBandwidth” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

##### 7.1.2.5.2 Maximum H264 macroblocks per second

| Identity: | maxH264Mpbs |
| --- | --- |
| Description: | This parameter indicates the maximum number of H264 macroblocks per second relating to a single video encoding. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See “maxH264Mbps” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

…

##### 7.1.1.2.1 Scene Area

| Identity: | SceneArea |
| --- | --- |
| Description: | This parameter indicates the area of the overall scene related to the endpoint. It may be larger than what is described by the individual media “Capture Area” parameters. |
| Format: | List of String |
| Possible Values: | 4 list elements each with (X, Y, Z) Where X, Y and Z are numbers. |
| Default: | None. |
| Reference: | See “Area of Scene” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | This parameter allows a renderer to determine from what area in an overall scene a capture comes from in order to render a media stream correctly. |

##### 7.1.1.2.2 Area Scale

| Identity: | AreaScale |
| --- | --- |
| Description: | This parameter indicates the type of scale that is being applied to the spatial information parameters |
| Format: | Enumeration |
| Possible Values: | “millimeters” The metric system with the millimetre unit is being applied. “unknown” An unknown scale is being used that is the same for all captures in the set. This may be used to indicate relative distances. “No scale” This indicates that the scale may be different for each capture in a set. <Other?> |
| Default: | <TBD> |
| Reference: | See “Area Scale” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

…

##### 7.1.2.4.1 Capture area

| Identity: | CapArea |
| --- | --- |
| Description: | This parameter indicates which area the video capture represents from the overall capture scene in terms of a set of four X,Y and Z Cartesian coordinates. |
| Format: | List of String |
| Possible Values: | 4 list elements each with (X, Y, Z) Where X, Y and Z are numbers. |
| Default: | None. |
| Reference: | See “Area of Capture” in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | This parameter is used to indicate the area of the video capture, and is also used to determine the spatial relationship between video captures. It is also possible to determine from the capture area information where gaps are present due to display borders etc. which should be taken into consideration by a renderer when rendering video streams. |

###### 7.1.2.4.1.1 Example usage of the capture area parameter

Capturers send the Capture Area parameters renderers, in terms of a set of four X, Y and Z coordinates, respectively. For example, the coplanar capture area is represented by for point bottom left (X1, Y1, Z1), bottom right (X2. Y2, Z2), top left (X3, Y3, Z3) and top right (X4 ,Y4, Z4). Following the conventions of “Area of Capture” [IETF CLUE FW], as numbers move from lower to higher, the location goes from camera left to right (in the case of the ’X’ dimension), audience front to back (in the case of the ’y’ dimension ) or low to high (in the case of the ’z’ dimension ), which is shown in figure 1(top view) and figure 2 (3D view).

Figure 1: Capture area axis (top view)

Region A

Region B

Region C

MIC 1

MIC 2

MIC 3

Camera Set

Display 1

Display 2

Display 3

Speaker

x

y

Figure 2: Capture area axis (3D view)

NOTE - The “Capture point”, “Scene Area” or origin parameters are not considered in the above diagrams.

On reception of the Capture Area information, renderers may use this information in several ways to render the received video streams illustrated below. The examples below only discusses the x-axis for simplicity, the calculations may equally be applied to the Y and Z axis. A scale of millimetres is assumed.

First, the capture area can be used to indicate the spatial information of the video capture. For example, a capture of Region A with an area of capture:

bottom left    	bottom right  		top left         		top right

Region A 	(0,2000,0) 		(980,2200,0) 		(0,2000,600) 	(980,2200,600)

. The capture area above indicates that in terms of the X-axis the width of 980mm. Considering the Y coordinates this leads to a planer width of 1000mm. Thus the renderer can use this information to render the pictures in correct spatial place

Second, the capture area can be used to describe the spatial relationship between video captures.For example if we consider the above capture of Region A and add a capture for Region B below:

bottom left    	bottom right  	top left         	top right

Region B 	(990,2200,0) 	(1990,2200,0) 	(990,2200,600) 	(1990,2200,600)

By analysing the capture area of Region A and  Region B, respectively, the spatial relationship of the two video captures, that Region A is to the left of Region B and Region B is to the right of Region A can be determined. Therefore the renderer can use this information to render the pictures in correct spatial arrangement.

Third, the capture area can be used to determine the gap between video captures. For example, utilising the above Area of Capture coordinates for Region A and B. From analysing the respective X coordinates (i.e. 980 and 990) is can be seen that there is a gap of 10mm between Regions A and B. So the renderer can use this information to render the video streams in a continuous manner. That is, if the width of the gap is larger than the width of the borders between the displays on the receiver side, a virtual display border (For example, a black zone) can be added to the captured images to achieve visual continuity. If the width of the gap is smaller than the width of the borders between the displays on the receiver side, the captured images can be cropped and scaled to achieve a continuous vision.

_____________