| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4216 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Update of colour parameters |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution proposes a number of updates to lighting / colour parameters in H.TPS-AV.

# Discussion

At the Geneva, November-December 2011 meeting three parameters were added to H/TPS-AV regarding lighting.  These parameters were: Type of Lighting, Colour Rendering Index and  Colour temperature. The information contained in these parameters is rather generic. The Contributors propose to add more detailed information referencing the relevant International Commission on Illumination (CIE)  documents.

# Proposal

It is proposed to add the following changes indicated by the change marks to H.TPS-AV. Change marks are against TD 645R1/WP2 from the Geneva, Switzerland meeting 21 November – 02 December 2011 meeting.

Note to Editor: The contributor’s notes should not be added to draft H.TPS-AV.

Draft new ITU-T H.TPS-AV

Audio/Video Parameters for Telepresence systems

…

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| CCT | Correlated colour temperature |
| --- | --- |
| CRI | Colour Rendering Index |
| RAS | Registration, Admission and Status |
| RTP | Real Time Protocol |
| RTCP | RTP Control Protocol |

…

## 7.3	Telepresence system environment parameters:

### 7.3.1	Illuminant

| Identity: | Type. |
| --- | --- |
| Description: | This parameter describes the profile of the visible light at a telepresence endpoint. It is characterised by a CIE standard illuminant value. |
| Format: | Enumeration. |
| Possible Values: | “A” Illuminant representing a gas-filled tungsten filament lamp. “B” Illuminant representing Direct Sunlight (Deprecated). “C” Illuminant representing Average Daylight, correlated colour temperature of approximately 6800K. “Dx” Illuminant representing Daylight (x is in integer, i.e. D65 Default, D50, 55, 75). “E” Illuminant representing an equal energy radiator. “FLx” Illuminant representing fluorescent lamp (x is an integer, i.e. FL1, FL2, FL12 etc). “FL3.x” Illuminant representing a three band fluorescent lamps (x is an integer, i.e.FL3.1, FL3.14). “HPx” Illuminant representing a high-pressure lamp spectra (x is an integer, i.e. HP1 Standard high pressure lamp HP2 Colour enhanced high pressure sodium lamp HP3to HP5 High Pressure metal halide lamps) |
| Default: | None. |
| Reference: | [b-CIE 15.3:2004] and [ISO 11664-2] |
| Interface: | - |
| Supplementary Information: | If the CRI value and colour temperature are be omitted they are implied by the type. For example, type “A” indicates that the light source is tungsten-filament lighting with CRI of more than 95 and colour temperature of 2856K. |

### 7.3.2	Colour Rendering Index

| Identity: | CRI. |
| --- | --- |
| Description: | This parameter describes the colour rendering index (CRI) of the visible (ambient) light at the telepresence endpoint. The method of calculation of the CRI is according to [b-CIE 13.3-1995]. |
| Format: | Numeric. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | [b-CIE 13.3-1995] |
| Interface: | - |
| Supplementary Information: | If the light source is a standard CIE illuminant, the CRI value can be omitted. |

### 7.3.3	Colour temperature

| Identity: | ColTemp. |
| --- | --- |
| Description: | This parameter describes the correlated colour temperature (CCT) [b-CIE 17.4-1987] of the visible (ambient) light at the telepresence endpoint. |
| Format: | Numeric, degrees Kelvin. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | If the light source is a standard CIE illuminant, the CCT value can be omitted. |

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Bibliography

[b-CIE 15.3:2004]	CIE 15.3:2004  Colorimetry, Third Edition, 2004.

[b-CIE 13.3-1995] 	CIE 13.3-1995, Method of Measuring and Specifying Colour Rendering Properties of Light Sources, 1995.

[b-CIE 17.4-1987] 	CIE 17.4-1987 International Lighting Vocabulary, 1987.

_______________