| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4218 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Discussion of colour and lighting related parameters |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution discusses the use of colour and lighting related parameters inH.TPS-AV.

# Discussion

At the last ITU-T Q.5/16 meeting a number of lighting and colour parameters were introduced in to the H.TPS-AV draft. The video colour gamut, type of light, CRI, and colour temperature parameters were accepted in the baseline text. These parameters were discussed in the meeting and it was felt that further contributions were required. I.e. the editor’s note in clause 7.1.2.1 / [H.TPS-AV]:

{Editor’s note: Further contributions are solicited on whether out-of-band signalling is needed.}

One of the goals of telepresence is to provide a “being there” experience. One of the aspects of the human vision system is to perform chromatic adaptation based on the ambient lighting conditions so that objects appear to have the same colour under various lighting conditions. Therefore people at the same location would generally perceive scenes in the same way. However in telepresence systems people are at disparate locations with different lighting conditions. Therefore a local scene would be perceived to be a different colour by people at the remote end to the local people.

In a telepresence system the scenes are captured and displayed by devices that do not have the same adaptation capability as people, i.e.

Several aspects of the acquisition and display process make such color correction essential – including the fact that the acquisition sensors do not match the sensors in the human eye, that the properties of the display medium must be accounted for, and that the ambient viewing conditions of the acquisition differ from the display viewing conditions. (en.wikipedia.org/wiki/White_balance)

Capture devices such as cameras can perform “white balancing” on the captured images so that the colour more closely correlates to what a person sees. In transforming the image colour the device must estimate the ambient light (illuminant) and its white point in order to scale the image colours correctly. This estimation of the ambient light can be provided by several means:

- A manual selection – a user provides an indication of the illuminant

- A custom white balance – by capturing a image a “grey card”, this image may be analysed to determine the illuminant.

- Automatic white balance – an algorithmic methods may be applied to determine the white balance based on the captured images.

Note: It is also possible to adjust the colour by altering the light sources themselves or by applying physical filters to the capture and display equipment however it is considered impractical for common usage. Therefore it is not considered further.

When an image is transmitted to the remote end and displayed it will in many cases be under a different ambient light. As discussed above if the colours of the image and luminance of the image are perceived by the remote participants as being different to the local conditions then the “being there” experience is diminished. In order to maximize the “being there” experience the display equipment may transform the colours of the image to take into account the local ambient light (illuminant). This local illuminant may be provided as per the above bullet list.

However in order to apply an accurate scaling it simplifies the process if the illuminant (white point) of the captured image is known.

There are several ways for this to be determined:

- Profile – A default “virtual illuminant” and its associated white point for telepresence systems could be established. This virtual illuminant would be separate from the actual ambient light at the telepresence endpoint.  Therefore a capture and display devices could assume a standard virtual illuminant which they could transform from/to. Therefore the remote device would know how to transform the image.

- Signalled – The illuminant can be signalled (either in-band or out-of-band) to the remote endpoint so that the display devices can determine the correct scaling for the transform.

Note: This may have the benefit that white balancing and colour enhancement at the capture point could be skipped if it could be assumed that the remote end would apply the correct transformation.

- Automatic – An algorithmic method may be applied to determine the illuminant. No standardisation would be needed as this would be a local function.

Note: It is likely that the illuminant used could be determined at the start of a telepresence session and used for the entirety of the session. Particularly in engineered telepresence rooms subject to artificial lighting. This assumption may not hold where daylight is used and the weather is changeable or where the session spans multiple hours over different times of the day: i.e. morning light versus midday.

The white point should be calibrated using a white point conversion matrix that is applicable for the different illuminants. The matrices are defined based on the lighting type and colour temperature of both the receiver and the provider. For example: a list of such conversion matrices is provided by the International society for optics and photonics (SPIE) in its book “Computational Color Technology”.

So clearly for telepresence a white point balancing and colour enhancement function is desirable. In order to perform this function the telepresence endpoint must know at least its own local illuminant (and its white point). Therefore the type of lighting (illuminant), colour rendering index and colour temperature are appropriate environment parameters.

The question remains whether the above parameters need to be signalled? The addition of the video “colour gamut parameter” to H.TPS-AV and its reference to and default of [ITU-R BT.709] means that a “profile” approach as outlined above is possible. This is due to ITU-R BT.709 referencing default chromaticity coordinates and a reference white for opto-electronic conversion.

The addition of the “video colour gamut” parameter also allows an in-band signalling approach as it mentions video usability information (VUI) signalling from Annex E.2 / [ITU-T H.264] element “colour primaries” which allows the signalling of the chromaticity and white point.

The use of an out of band signalling approach i.e. during session establishment to send illuminant (white point) is currently not supported by a standard means such as H.245 or SDP. An out of band approach would be beneficial if an in-band or profile cannot be supported.

Again it is clear that chromaticity and white point being communicated between the local and remote endpoint (either implicitly through provisioning or signalled). Therefore the video colour gamut parameter is appropriate. As such it is proposed to add some text regarding the relationship between the colour and lighting parameters and their use locally by a telepresence endpoint and use by a remote endpoint.

# Proposal

It is proposed to add the following changes to H.TPS-AV. Change marks are against TD 645R1/WP2 from the Geneva, Switzerland 21 November – 02 December 2011 meeting.

Note: Editorial updates to the parameters have been proposed in an accompanying contribution.

Draft new ITU-T H.TPS-AV

Audio/Video Parameters for Telepresence systems

…

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.264]	ITU-T Recommendation H.264 (01/2012), Advanced video coding for generic audiovisual services.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-01, Framework for Telepresence Multi-Streams.

[ISO 11664-2]		ISO 11664-2:2007 (CIE S 014-2/E:2006) (2007), Colorimetry -- Part 2: CIE standard illuminants.

…

### 7.1.2	Visual parameters

This clause describes the telepresence parameters related to the visual capabilities.

#### 7.1.2.1	Video Colour Gamut

| Identity: | ColourGamut |
| --- | --- |
| Description: | This parameter indicates the Colour Gamut used in a Telepresence Video Stream. |
| Format: | TBD |
| Possible Values: | BT.709, BT.601, BT.1361 |
| Default: | BT.709. |
| Reference: | None |
| Interface: | None |
| Supplementary Information: | In the absence of visual usability information (VUI) signalling (or if the VUI signaling indicates “unspecified”), telepresence systems shall process all received video signals per [ITU-R BT.709]. For example: Annex E.2 / [ITU-T H.264] regarding VUI has the “colour primaries” element that allows the signalling of the chromaticity and white point. If the transmitted video is not conformant with [ITU-R BT.709], senders should always signal the black level and range of the luma and chroma signals, transfer characteristics, the colour primaries, and the matrix coefficients in their video bitstreams. Receivers may use this information to optimally render the received pictures. See clause 7.3.1.4 for more information on the use of colour and lighting parameters. |

## 7.3	Telepresence system environment parameters:

### 7.3.1Colour and lighting parameters

#### 7.3.1.1	Type of Lighting

| Identity: | Type. |
| --- | --- |
| Description: | This parameter describes the type of the ambient light. |
| Format: | Enumeration. |
| Possible Values: | “I” The type of the ambient light is incandescent. “F” The type of the ambient light is fluorescent. “H” The type of the ambient light is halogen. “IG” The type of the ambient light is inert gas. Etc. |
| Default: | None. |
| Reference: | [ISO 11664-2] |
| Interface: | - |
| Supplementary Information: | It is an element of the parameter “Lighting”. If the CRI value and colour temperature are be omitted they are implied by the type. For example, type “A” indicates that the light source is tungsten-filament lighting with CRI of more than 95 and colour temperature of 2856K. |

#### 7.3.1.2	Colour Rendering Index

| Identity: | CRI. |
| --- | --- |
| Description: | This parameter describes the CRI of the ambient light. |
| Format: | Numeric. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | It is an element of the parameter “Lighting”. If the light source is a standard CIE illuminant, the CRI value, whose unit is 10, can be omitted. |

#### 7.3.1. 3	Colour temperature

| Identity: | ColTemp. |
| --- | --- |
| Description: | This parameter describes the colour temperature of the ambient light. |
| Format: | Numeric, degrees Kelvin. |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | It is an element of the parameter “Lighting”. If the light source is a standard CIE illuminant, the colour temperature value, whose unit is 100K, can be omitted. |

#### 7.3.1.4 Colour and lighting parameter usage

One of the goals of telepresence is to provide a “being there” experience. One of the aspects of the human vision system is to perform chromatic adaptation based on the ambient lighting conditions so that objects appear to have the same colour under various lighting conditions. Therefore people at the same location would generally perceive scenes in the same way. However in telepresence systems people are at disparate locations with different lighting conditions. Therefore a local scene would be perceived to be a different colour by people at the remote end to the local people.

Capture devices such as cameras can perform “white balancing” on the captured images so that the colour more closely correlates to what a person sees. In transforming the image colour the device must estimate the ambient light (illuminant) and its white point in order to scale the image colours correctly. The “type of lighting”, “colour rendering index” and “colour temperature” are local environmental parameters that may be utilised for white balancing colour enhancement.

When an image is transmitted to the remote end and displayed it will in many cases be under a different ambient light. As discussed above, if the colours of the image and luminance of the image are perceived by the remote participants as being different to the local conditions then the “being there” experience is diminished. In order to maximize the “being there” experience the display equipment may transform the colours of the image to take into account the local ambient light (illuminant) and white point and chromaticity of the received image. The white point and chromaticity of the received image may be based on pre-agreed profile (i.e. the support of [ITU-R BT.709) or through in-band signalling such as the VUI parameters in [ITU-T H.264]. The “Video colour gamut” parameter (clause 7.1.2.1) provides a means for the communication of this information.

___________________