| HRapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4219 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Ltd. |  |  |  |
| Title: | H.TPS-AV: Audio level considerations for Telepresence Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This contribution provides background and a proposal for further parameters necessary to define the appropriate audio levels for successful interoperability of Telepresence systems, currently specified in section 7.1.3.4 of draft H.TPS-AV.

# Discussion

A critical factor in achieving high QoE in Telepresence systems is to ensure that the end to end audio levels are satisfactory for the users. End to end audio level in any communications system is a combination of the acoustic-electric transfer function at the sending end and the electric-acoustic transfer function at the receiving end. In a proprietary system, the designer is free to arbitrarily set the values of these transfer functions to achieve the desired overall audio levels. However, to achieve interoperability between different systems it is necessary to specify the sending acoustic to electric and receiving electric to acoustic transfer functions. For conventional telephones (handset and handsfree) these are normally specified in terms of send and receive loudness ratings (SLR. RLR) [e.g. 1, 2], but there is no reason why this concept cannot be extended to Telepresence systems, and in fact Rec. P.300 [3] should apply. It should be noted that the often-quoted “audio level” is usually the electrical level used for testing purposes, especially for subjective testing of audio codecs. By itself, this is not sufficient to specify the audio levels perceived by the end user.

A further confounding factor is that it has recently been brought to the attention of SG12 that there may be anomalies in the codec overload point of audio codecs in the G.720-series. As noted in [4], codec overload point is an important issue, for several reasons:

- codec overload point impacts send and receive loudness ratings in terminals, and hence impacts end to end audio levels

- codec overload point impacts the usable dynamic range of a codec. If the overload point is too low, there may be overload on speech peaks. It may be noted that overload on speech peaks is likely to be more noticeable subjectively in WB, SWB and FB, compared to NB. If the overload point is too high, more distortion will be introduced on low-level speech.

Specifically, the codec overload point for G.722 is defined as +9 dBm0, but is not defined at all for other WB, SWB and FB codecs in the G.720-series. In fact, it has been confirmed [5] that the nominal level for subjective testing of all codecs since G.722 has been -26dBoV, which implies a codec overload point of +3 dBm0 (same as for G.711), but this may not be adequate to handle speech peaks, and this is the rationale for the value of -31 dBov given in section 7.1.3.4 of draft H.TPS-AV. Since many of these codecs are likely to be used in Telepresence systems it is important to understand the assumptions for the codec overload point.

# Proposal

Taking into account the above considerations it is proposed to include the following new parameters in H.TPS-AV to specify send and receive loudness ratings of Telepresence systems.

#### 7.1.3.X  Send Loudness Rating

| Identity: | SendLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the acoustic to electric transfer function of the Telepresence audio stream. |
| Format: | dB |
| Possible Values: | TBD |
| Default: | TBD |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall send audio to other telepresence systems with a SendLoudnessRating of X dB, measured in accordance with [ITU-T P.300].

#### 7.1.3.X  Receive Loudness Rating

| Identity: | ReceiveLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the electric to acoustic transfer function of the Telepresence audio stream. |
| Format: | dB |
| Possible Values: | TBD |
| Default: | TBD |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall receive audio from other telepresence systems with a nominal ReceiveLoudnessRating of Y dB, measured in accordance with [ITU-T P.300].

It remains to be discussed whether audio level per se (as currently given in section 7.1.3.4) is still needed and what codec overload point should be assumed. Given that SG12 has expertise in this area, it is proposed to send an LS to them soliciting their advice, as well as guidance on numerical values for X and Y.

# References

[1] ITU-T Rec. P.311 (2011-03) “Transmission characteristics for wideband digital handset and headset telephones”

[2] ITU-T Rec. P.341 (2011-03) “Transmission characteristics for wideband digital loudspeaking and hands-free telephony terminals”

[3] ITU-T Rec. P.300 (2001-11) “Transmission performance of group audio terminals (GATs)”

[4] TD 474 (GEN/16) November 2011 “LS from ITU-T SG 12 on codec overload point”

[5] COM 16 – LS 290 December 2011 “LS to ITU-T SG12 on audio coding matters”

___________________________

Assuming that there is no additional loss in the transmission path (true in modern digital/IP networks).