| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-09 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur Q.3/16 (C.Groves) |  |  |  |
| Title: | Report of Question 3/16 “Multimedia Gateway Control Architectures and Protocols” |  |  |  |
| Purpose: | Proposal |  |  |  |

# Question 3/16 – Multimedia Gateway Control Architectures and Protocols

Q3/16 met for 4 quarters to review some 36 documents. Q3/16 continued work on our existing work items as well as adding two new work items for H.248.TLSPROF and H.248.RTCPPROF.

The Rapporteur would like to thank the contributors to the Q.3 work and especially the editors for their work in preparing the work items.

## Objectives

Question 3/16 was addressed in 4 sessions under the chairmanship of Christian Groves (NTEC Australia, Australia). The group adopted the agenda in TD-03.

The objectives for this meeting were to:

- Coordinate with other SDOs, Questions, or Study Groups

- Progress work on:

  - H.248.66 (ex H.248.RTSP)

  - H.248.74 (ex H.248.MRCP)

  - H.248.80 (ex H.248.SDPMAPPER)

  - H.248.82 (ex H.248.ECN)

  - H.248.DPI

  - H.248.NATTP2P

  - H.248.RTPTOPO

  - H.248.LOOPB

  - H.248.TLS

  - H.248 Sub Series IG

  - H Series Supp. 2

- Consider new material.

### Documentation

The following documents were examined:

- AVD: 4160, 4161, 4162, 4163, 4164, 4165, 4166, 4167, 4168, 4169, 4170, 4171, 4172, 4173, 4174, 4175, 4176, 4177, 4178, 4179, 4180, 4181, 4182, 4183, 4184, 4185, 4186, 4187, 4188 (29 contributions);

- TDs: 3, 20, 22, 23, 24, 25a, 26. (7)

### Report of interim activities

None.

### Discussions

#### Incoming liaison statements

None.

#### Co-ordination

None.

#### H.248.1v4 Living List

| AVD-4181 | H.248.1v4: Input living list | Rapporteur Q3/16 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes, there is no output draft from the meeting.

#### H.248 Sub-Series Implementors Guide

| AVD-4184 | H.248 Sub-series Implementors' Guide (revised): Input draft | Editor H.248.x IG |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4169 | H.248.81: Priority traffic – Semantical issue of MG actions | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and was noted. The difference between generic and practical profiles was questioned. It was noted that generic profiles were subject to SDO specification rather than practical profiles.  No action was taken on the document.

| AVD-4173 | H.248.11: Optimization concept by piggy-backing reply commands | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposals were not accepted.

There were concerns that using an error code results in an additional action that the MGC will not know about. It was suggested that using an a= line in a response message could be a possible solution. Another method could utilise putting a Notify.req in the same message as an ADD.resp. Alcatel-Lucent preferred not to have the extra message. The delegates also noted that it would be preferable if the indication from the MG was still associated with the NULL context.

It was agreed to add an Implementor’s guide item indicating that including the Notify.req in the same message as an ADD.resp was a potential optimization.

The output draft H.248 Sub-series IG can be found in TD-20.

#### H.248.66 (ex H.248.RTSP) Packages for RTSP and H.248 interworking

| AVD-4185 | H.248.66 (ex H.248.RTSP) "Packages for RTSP and H.248 interworking" (New): input draft | Editor H.248.66 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting. The work is complete pending progress in the IETF.

As there were no changes, there is no output draft from the meeting.

#### H.248.74 (ex H.248.MRCP) Media Resource Control Enhancement Packages

| AVD-4186 | H.248.74 (ex H.248.MRCP) "Media Resource Control enhancement Packages" (New): input draft | Editor H.248.74 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting. The work is complete pending progress in the IETF.

As there were no changes, there is no output draft from the meeting.

#### H.248.80 (ex H.248.SDPMAPPER) Usage of the revised SDP offer / answer model with H.248

| AVD-4182 | H.248.80 (ex H.248.SDPMapper) "Usage of the revised SDP offer / answer model with H.248" (New): input draft | Editor H.248.80 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting. The work is complete pending progress in the IETF.

As there were no changes, there is no output draft from the meeting.

#### H.248.82 (ex H.248.ECN) Explicit Congestion Notification Support

| AVD-4187 | H.248.82 (ex H.248.ECN) "Gateway control protocol: Explicit Congestion Notification Support" (New): input draft | Editor H.248.82 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4170 | H.248.82: Discussion of interaction between ECN and ETS | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed. The delegates agreed with the analysis, examples and conclusions. It was noted that the conclusions did not cover example 1. The delegates agreed that ECN enabled ETS would be beneficial where there was a significant amount of traffic. However the delegates could not quantify “significant” as this may be different for different media gateway types.

| AVD-4171 | H.248.82: Relevance of Congestion Exposure (ConEx) protocol? | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and noted.

It was noted that the IETF ECN draft is now version 6 and that an analysis of the difference document between versions indicated that no changes to H.248.82 are necessary. It was agreed to update the reference in the draft to version 6 however given that it was a minor change it was agreed to incorporate it into an input document to the next meeting rather than to generate an output document from this meeting.

As there were no changes, there is no output draft from the meeting.

#### H.248.DPI Deep Packet Inspection

| AVD-4160 | H.248.DPI "Gateway control protocol: H.248 Support for deep packet inspection" (New): Ed. 0.10, input draft | Editor H.248.DPI |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4167 | H.248.DPI: Examples for pointer- and container-based rule signalling | ALCATEL-LUCENT |
| --- | --- | --- |

This document was presented. In general it was thought that it was a good analysis of the options available although it was hard to specify a way forward without analysis of concrete PSLs. There was a question on section 1.2 opt.2 whether we would we define a syntax for DPI rules? It was agreed that option 2 would mean that the container method would not be transparent. There were no concrete proposals however the document should be considered when developing contributions for the next meeting.

| AVD-4168 | H.248.DPI: Alignment with other policy control protocols | ALCATEL-LUCENT |
| --- | --- | --- |

This document was presented. In general the delegates felt it was a good analysis despite there being concrete proposals. However the document should be considered when developing contributions for the next meeting.

As there were no changes, there is no output draft from the meeting.

#### H.248. NATTP2P NAT-Traversal for Peer-to-Peer Services

| AVD-4161 | H.248.NATTP2P "NAT-traversal for peer-to-peer services" (New): Ed. 0.8, input draft | Editor H.248.NATTP2P |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

| AVD-4163 | H.248.NATTP2P: TCP protocol timers | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposal was accepted with the modification that rather than specifying specific behaviour (i.e. release) a sentence would be added to indicate that connection establishment may take up to 75 seconds and that the MGC should be aware of this.

tion that rather hen proposal was accepted wposal  a good analysis of the options available although it was hard

| AVD-4164 | H.248.NATTP2P: H.248 error codes | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposal was accepted, except that it was agreed that error code 477 should instead be 473.

| AVD-4165 | H.248.NATTP2P: Discussion of living list items | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposal was accepted.

| AVD-4166 | H.248.NATTP2P: cl. 13.3 Supported connection models | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposal was accepted.

There was a general discussion on whether the work item would be ready for Consent at the May ITU-T SG16 meeting. Even though there was further work needed on advanced NAT traversal it was thought that there may be a possibility for Consent. Delegates were encouraged to perform a thorough review before the next meeting.

PackageIDs will be requested from IANA in the interim.

The output draft H.248.NATTP2P can be found in TD-22.

#### H.248. RTPTOPO H.248 IP-to-IP Gateways – RTCP handling vs RTP topologies

| AVD-4162 | H.248.RTPTOPO "RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations" (New): Ed. 0.8, input draft | Editor H.248.RTPTOPO |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

As there were no changes, there is no output draft from the meeting.

#### H.248. LOOPB Usage of Loopback in H.248

| AVD-4188 | H.248.LOOPB "Gateway control protocol: Usage of Loopback in H.248" (New): input draft | Editor H.248.LOOPB |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting. The work is complete pending progress in the IETF.

| AVD-4172 | H.248.LOOPB: Use cases in H.248.48 environments | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposal was accepted. It was further agreed to reword the changes in section 6.3 to indicate that there may be exceptions to the requirement that RTCP and RTCP-XR should be supported.

The output draft H.248.LOOPB can be found in TD-25a.

#### H.248. TLS Transport Layer Security

| AVD-4174 | H.248.TLS: Use case descriptions | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposals were accepted.

| AVD-4175 | H.248.TLS: Basic H.248 requirements | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and in general the proposals were accepted. It was noted that the section 8.2 requirements shouldn’t specify specific protocol behaviour, i.e. referencing a package property. The requirements should be made generic. Rather than incorporate the proposal into H.248.TLS it was decided to capture this in a separate Recommendation that discusses TLS profiles. This new work item would be called H.248.TLSPROF.

Mr. J.Guballa from Alcatel-Lucent was volunteered to be the editor.

| AVD-4176 | H.248.TLS: Initial package proposal | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and was noted as there were no concrete proposals.

| AVD-4180 | H.248.TLS: TLS-based H.248 transport mode | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed and the proposals were accepted.

The output draft H.248.TLS can be found in TD-23.

The output draft H.248.TLSPROF can be found in TD-26.

#### H Series Supplement 2 Release 16

| AVD-4183 | H-Series Supplement 2: "H.248.x sub-series packages guide - Release 16" (revised): input draft | Editor H.248 Sup.2 |
| --- | --- | --- |

This document was discussed and it was accepted as the baseline for the meeting.

#### New Material

| AVD-4177 | Proposed new H.248.RTCPPROF work item: Profile specification guidelines for H.248-based Performance Monitoring in RTP networks | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed. The delegates thought that the principle of having a document that SDOs could refer to to determine the correct H.248 elements in order to support different RTCP scenarios was beneficial. However there wasn’t agreement on the correct way to capture the information. It was thought that a strict following of the H.248 Profile template may not be the best way. It was also noted that H.248.RTCP-xnq no longer exists and that we shouldn’t refer to it normatively.

| AVD-4178 | H.248.RTCPPROF: Network monitoring architectural requirements vs H.248 gateway types | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed. There was some discussion about how the requirements are used. It was clarified that based on the use cases an SDO/implementor would select which role the MG is undertaking. Based on this role the requirements for a MG could be identified. It was agreed that the scope needed to indicate how the document would be used. There were also some specific questions:

- R-7.2 does transport level mean below RTP? Isn’t that out of scope? I was indicated that yes, it was below RTP. The assumption was that every kind of H.248 MG with RTP supports an IP interface, thus measurements could be provided. It was agreed to add an editor’s to clarify the scope.

- R-7.8/2: Why is MG support of application level metrics prohibited? There was some discussion that indicated that when the MG was acting as a measurement point it did not make sense to calculate certain metrics. It was agreed to add an editor’s note to clarify this requirement.

- I.8 clause 4. Bullet 1 – This bullet wasn’t clear to the delegates, as such it was agreed to add an editor’s note.

| AVD-4179 | H.248.RTCPPROF: Skeleton for Draft Recommendation | ALCATEL-LUCENT |
| --- | --- | --- |

This document was discussed. It was agreed to incorporate the proposal in the baseline text. However rather than adding the package detail tables it was agreed to only add the headings listing the packages at this time. Further work is needed to understand the relationship between the package elements and various requirements/use cases.

It was agreed to start a new work item on RTCP profiles, H.248.RTCPPROF. Mr. A.Schwarz kindly volunteered to be the editor.

The output draft H.248.RTCPPROF can be found in TD-24.

### Intellectual property statements

None.

### Outgoing liaison statements

None.

### Work programme

#### Future work

E-mail correspondences pertaining to the activities of this group are routinely conducted using the e-mail reflector currently hosted by Packetizer. Those wishing to subscribe or unsubscribe to this email reflector should visit the Packetizer mailing list web page at:

http://lists.packetizer.com/mailman/listinfo/itu-sg16

E-mail correspondences pertaining to H.248 are routinely conducted using the IETF MEGACO e-mail reflector. Those wishing to subscribe or unsubscribe to this email reflector should visit the IETF Megaco Mailing List web page at:

https://www.ietf.org/mailman/listinfo/megaco

E-mails regarding H.248 should be sent to megaco@ietf.org.

Rapporteur group documentation can be found in the external FTP server for Q3/16 located at http://ftp3.itu.int/av-arch/avc-site.

The up-to-date work programme for Q3/16 can be checked online at: http://itu.int/ITU-T/workprog/‌wp_search.aspx?isn_sp=545&isn_sg=554&isn_qu=692.

The currently open work items for Q3/16 are as follows:

| Acronym | Title | Editor | Consent / Approval | Reference |
| --- | --- | --- | --- | --- |
| H.248.66 (ex H.248.RTSP) | “Packages for RTSP and H.248 interworking” (New) | Y.Lin | 05/2012 | AVD-4185 |
| H.248.74 (ex H.248.MRCP) | “Media Resource Control Enhancement Packages” (New) | T.Young | 05/2012 | AVD-4186 |
| H.248.80 (ex H.SDPMAPPER) | “Usage of the revised SDP offer / answer model with H.248” (New) | C.Groves | 2012 | AVD-4182 |
| H.248.82 (ex H.248.ECN) | "Explicit Congestion Notification Support" | T.Young | 05/2012 | AVD-4187 |
| H.248.DPI | “Deep Packet Inspection” (New) | A.Schwarz | 2012/2013 | AVD-4160 |
| H.248.NATTP2P | “NAT-Traversal for Peer-to-Peer Services” | A.Schwarz | 2012 | TD-22 |
| H.248.RTPTOPO | “H.248 IP-to-IP Gateways – RTCP handling vs RTP topologies” | A.Schwarz | 2012 | AVD-4162 |
| H.248.LOOPB | "Usage of Loopback in H.248" | T.Young | 2012 | TD-25a |
| H.248.TLS | “Transport Layer Security” | J.Guballa | 2013 | TD-23 |
| H.248.TLSPROF | “Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles” | J.Guballa | 2013 | TD-26 |
| H.248.RTCPPROF | “Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles” | A.Schwarz | 2013 | TD-24 |
| H.248 Sub Series IG | “H.248.x sub-series Implementors' Guide” (Revised) | C.Groves | 12/2011 | TD-20 |
| H Series Supp. 2 | “H.248.x sub-series packages guide – Release 16” (Revised) | C.Groves | 12/2011 | AVD-4183 |

#### Future meetings

Question 3/16 is planning to meet at the next Study Group 16 meeting 30 April-11 May 2012 in Geneva, Switzerland.

Annex Q3.A
Proposed new ITU-T Recommendation H.248.RTCPPROF

| Question: | 3/16 | Proposed new ITU-T Recommendation | Geneva (CH), 20 - 24 February 2012 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.248.RTCPPROF "Profile specification guidelines for H.248-based Performance Monitoring in RTP networks" |  |  |  |  |
| Base text: | Contribution AVD-4177 |  | Timing: | 2013 |  |
| Editor(s): | A.Schwarz |  |  |  |  |
| Scope: |  |  |  |  |  |
| Major items for a new draft H.248.RTCPPROF - there should be at least following aspects considered: There should be at least following aspects considered: overview of “entire H.248 toolkit” for RTCP extension report (XR )support, i.e., the H.248 packages from H.248.30, H.248.xnq and H.248.48; overview of H.248 packages for RTCP basic report (SR, RR) capabilities, non-RTCP XR defined metrics for RTP traffic; example network use cases with different H.248 MG involvement (primarily physical-to-RTP gateways, media-agnostic IP-to-IP gateways, RTP-topology-aware RTP-to-RTP gateways); collection of functional requirements (“similar like H.248.81”), in order to justify dedicated profile guidelines (“stage 3 versus a particular set of stage 2 …”); application-level versus transport-level performance metrics (as defined as RTCP XR) principal H.248 MG tasks of - measurements of RTP traffic, - reporting of measurement data via RTCP XR reports, - collection of RTCP XR carried measurement data, - loopback of RTP (in order to relocate a measurement point), - filtering of RTCP XR reports, - modification of RTCP XR reports and/or - reporting of measurement data via H.248 statistics to the MGC control of such tasks via H.248, i.e. - the identification of appropriate H.248 packages - the package usage detailed specification - the example call-dependent procedures for a H.248 profile the association to other related H.248.x Recommendations such as H.248.71, H.248.79, H.248.LOOPB and H.248.RTPTOPO control of measurement methods (such as interval, cumulative, alert) usage of H.248.47-based conditional reporting (“any applicability limitations”) others |  |  |  |  |  |
| Summary |  |  |  |  |  |
| Development of guideline document for specifying network use case specific H.248 profiles in the area of H.248-based performance monitoring in RTP networks. |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| H.248.1, H.248.30, H.248.48, H.248.58, H.248.71, H.248.79, H.248.RTPTOPO, H.248.LOOPB |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Possibly with MSF, i3 Forum, ETSI TISPAN, 3GPP. |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Alcatel-Lucent |  |  |  |  |  |

_______________________

_______________________