| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-29 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 22/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor F.LIMSreqs |  |  |  |
| Title: | F.LIMSreqs "Low-latency interactive multimedia stream requirements" (New):: Output draft |  |  |  |

This document contains the editor’s output draft for F.LIMSreqs "Low-latency interactive multimedia stream requirements" based on contributions and discussion in this Rapporteur meeting.

CONTENTS

1	Scope	3

2	References	3

3	Definitions	3

3.1	Terms defined elsewhere	3

3.2	Terms Defined within this document	3

4	Abbreviations	3

5	Convention	4

6	Introduction	4

7	Scenarios for LIMS	5

7.1	LIMS for Enterprise Application Delivery	5

7.1.1	One Platform to Delivery All Applications	5

7.1.2	Multiple Terminal Supports for an Application	5

7.1.3	Customizable Unified Client	6

7.1.4	Secure Application	6

7.1.5	Centralized Management	6

7.2	LIMS for Application Delivery Service	6

7.2.1	Service Tryout	7

8	Reference Architecture	7

9	Requirements for LIMS	7

Appendix I  Typical LIMS Architecture	8

Bibliography	8

Draft new Recommendation F.LIMSReqs

Low-latency Interactive Multimedia Stream Requirements

AAP Summary

[To be provided before Consent]

Summary

[To be provided before Consent]

Keywords

Interactive Multimedia services, service provider

# Scope

This draft Recommendation identifies the scenarios and requirements for Low-latency Interactive Multimedia Service (LIMS).

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[TBD]

# Definitions

## Terms defined elsewhere

This document uses the following terms defined elsewhere:

3.1.1	[TBD]: [TBD]

## Terms Defined within this document

This document defines the following terms

3.2.1	[TBD]: [TBD]

# Abbreviations

# Convention

No specific conventions have been used in this document.

# Introduction

In a Low-Latency Interactive Multimedia Service (LIMS) system, a user’s experience of the service is the display of an interactive video stream. The user generates an input message based on his/her current view of the video stream. The LIMS client collects the input message and transfers it to the LIMS platform. The LIMS platform processes the input message to update its virtual visual output maintained for the service that the user is using. The updated virtual visual output, which is generally a sequence of images, is then been processed by LIMS platform to generate a real-time video stream. This real-time video stream is the interactive video stream displayed to the user. To support an impeccable interactive service, all the above processes must be guaranteed to be finished in a relatively short time so as to make the user senses almost no latency. For better illustrating the working theory of a LIMS service, Appendix I shows a typical LIMS system architecture.

In a LIMS system, the LIMS platform works similar to a service proxy. From the service provider view point, the using of LIMS platform introduces little or no change. From the end user’s view point, the difference is that his terminal receives a video stream but not certain service specific response messages.

By moving the complex computational work load from the user terminals to the LIMS platform, the using of LIMS service simplifies the architecture of the terminal devices. Such change also enables more convenient service adjustment and easier system upgrading. LIMS platform serves users by interactive video streams. This provides better protection to the intelligent property of those applications on the LIMS platform.

# Scenarios for LIMS

## LIMS for Enterprise Application Delivery

Figure 7-1 – Scenario of LIMS for Enterprise Application Delivery

An enterprise can provide all its business applications to its employee or customers via LIMS. No matter what kind of application it is.

By doing so, the following LIMS characterized use cases can be provided.

### One Platform to Delivery All Applications

The LIMS platform can supports the delivery of every kinds of application.

Once the enterprise to deploy a new application, it just need to deploy the servers for the new application, connect the new servers to the LIMS platform, and make some relate configurations and tests on the LIMS platform.

### Multiple Terminal Supports for an Application

When developing an application, the developer does not need to consider the adaptation of different terminals. The terminal adaptation problem can be solved by LIMS platform. No matter on what kinds of terminal the application is designed to run. Once the application is released, it can be used smoothly on all kinds of terminals, such as PC, Phone, and TV. The usage of an application is not limited by the computing or storage resource on the terminal, although all LIMS enabled terminals must support stream video decoding.

For applications allow multiple users to access simultaneously, each user accessing the service may use a different kind of terminals.

If a user stops an application on a terminal, i.e. his notebook, he can resume the application from any terminal, starting right from where the application is last stopped.

### Customizable Unified Client

A unified client is provided to the users, either an employee or a customer. With this client, a personal portal that can be customized by the user is provided. This portal provides a user consistent experience for the services. In another words, no matter what kinds of terminal it is, a users’ view of the services is similar.

From this portal, the user can access all applications and resources the enterprise provides. The adding of new applications, the removing of old applications or the upgrading to applications requires no modification to the unified client.

### Secure Application

LIMS platform provides enhanced security protection for the applications.

When a hacker tries to hack an application delivered by LIMS, he plays with a video stream. Most information about the application is unrecoverable based on the LIMS video stream. Even though the interactive information may be recoverable, such as username, password, etc., it has been perturbed [pɚ’təb] by the video codec and become harder to recover by a short program. The hacking of the application becomes much more difficult.

When an attack occurs, such as DDOS, the LIMS platform can filter the attack request.

As the user receives only a video stream, it is relatively easier to have sensitive information to be readable but not allow to be copied.

### Centralized Management

LIMS intuitively provides centralized management capability for the enterprise. Such capability includes:

- Single Sign-on: A user is allowed to log in the LIMS platform and be authenticated to all the applications and resources. The using of applications and resources do not require a second authentication process.

- Centralized Data backup: All the data and transaction logs are commonly saved in the LIMS platform. This builds a centralized data backup. Such data can be share among the enterprise under particular security policy.

- Centralized application maintenance: The adding of new applications, the removing of old application or the upgrading of applications requires no modification to the unified client. All the maintenance is performed within the LIMS platform or the Enterprise Application Servers.

## LIMS for Application Delivery Service

Figure 7-2 – Scenario of LIMS for Application Delivery Service

By deploy LIMS platform in Internet, its owner can provide commercial LIMS application delivery service. Besides the use cases listed in the Section 7.1, in this scenario, the following special use cases can be supported.

### Service Tryout

As the application delivered by LIMS platform can be used without download and installation process. It is convenient for an application to release for tryout. At the beginning, this application allows the user to use it for free, such as in the first 3 days, then it requires the user to pay for his further usage. As there is no downloading and installing overhead, such service tryout is more acceptable for a user.

# Reference Architecture

[TBD]

# Requirements for LIMS

[TBD]

Appendix I

Typical LIMS Architecture

LIMS system requires coordination between the Functional Entities (FE) within the LIMS clients and the LIMS platform. Generally, a LIMS system may also need support from the original service provider (SP). Thus, the LIMS FE can be separated into three areas, i.e. client side, platform side, and SP side. A typical reference architecture of LIMS is showed in Fig. I-1.

At the client side, there are two FEs, the LIMS Client Middleware (LCM) and Input and Output device (I/O). I/O device collects the input message and provides output video show for a user. I/O device may be a mouse, keyboard, touchpad, monitor, etc. The LCM functions in two folds. On the one hand, it informs the LIMS Platform about the user activity. On the other hand, it receives the real-time multimedia stream from the LIMS Platform and displays it to the user.

At the platform side, there are four important FEs, the LIMS Platform Communication Module (LPC), the LIMS Proxy Service Module (LPS), the LIMS Visualization and Compression Module (LVC), and the LIMS Stream Service Module (LSS). LPC is responsible for the communication between the LIMS Platform and LIMS clients. The client side activity received by LPC is sent to LPS for further processing. From the SP side, LPS functions as a normal terminal accessing the service, while the input to the terminal is not from local I/O devices but from the LPC module. LPS generates the updated image of the service based on the response received from the SP, such image is send to the LVC module. LVC is responsible for the compression and processing for the continuous image sequence received from LPS, so as to generate a multimedia stream for the service. LSS module is configured to provide stream service based on the multimedia stream generated by LVC. Note that, in a LIMS system, the clients’ view of a service is presented by LCM accessing the multimedia stream serviced by the LSS module at the LIMS platform.

At the SP side, there is only a LIMS related FE, that is Application Server (AS). Generally, LIMS requires no change to the original AS of a SP.

Platform Side

AS

LPS

LPC

LCM

LSS

LVC

I/O

SP Side

Client Side

Figure I-1 - Architecture of a LIMS system

Bibliography

[TBD]	TBD

_________________________