| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | TD-31 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | F.USN-CC “Deployment guidelines for ubiquitous sensor network (USN) applications and services for mitigating the climate change”: Output text |  |  |  |
| Purpose: | Information |  |  |  |

This document contains revised text of draft new Recommendation ITU-T F.USN-CC based on the discussion during Rapporteur Meeting of WP2/16 which was held from 20 – 24 February 2012. Moreover, ITU-T F.USN-CC has a plan for Consent at the next SG16 meeting.

The following table shows discussion results for contribution.

| Contribution | Contribution title and proposals | Agreements |
| --- | --- | --- |
| AVD-4198 | Proposal for editorial and technical improvements of F.USN-CC |  |
|  | - Delete introduction clause | - Agreed as proposed |
|  | - Add new text in scope clause | - Agreed as proposed |
|  | - Add new abbreviations | - Agreed as proposed |
|  | - Add new text and figure to explain climate change monitoring in new clause 6 | - Agreed as proposed |
|  | - Add a new consideration point in clause 8.1.1 | - Agreed as proposed |
|  | - Add new text about operation conditions of GHG sensors in new clause 8.3 | - Agreed as proposed |
|  | - Modify the bibliography | - Agreed as proposed |
|  | - Minor editorial changes in the whole text | - Agreed as proposed excluding changes to use capital letters on Ubiquitous Sensor Network |
| AVD-4202 | ITU-T F.USN-CC: Proposed modification of draft New Recommendation |  |
|  | - Add USN middleware definition in clause 3.1.7 | - Agreed as proposed |
|  | - Add a new abbreviation | - Agreed as proposed |
|  | - Replace 4 example figures in clause 7.2 | - Agreed as proposed |
|  | - Change the title of clause 8 and delete editor’s note under the title | - Agreed as proposed |
|  | - Add a new consideration point in clause 8.2.3 | - Agreed as proposed |
|  | - Minor editorial changes in the whole text | - Agreed as proposed |

The following table shows discussion results for the comments from the floor.

| Comments | Agreements |
| --- | --- |
| - To consider more information for overview of climate change monitoring | - Put the editor’s note under the title of clause 6. “Editor’s note: Relevant contributions are invited for overview of climate change monitoring. If no more relevant contributions are submitted before Consent, this editor’s note should be removed.” |

.

CONTENTS

1	Scope	4

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	5

5	Conventions	5

6	Overview of climate change monitoring	5

6.1	Global GHG monitoring network	6

6.2	Local GHG monitoring network	6

7	Analysis of environmental impact by USN applications and services	7

7.1	Deployment elements of USN	7

7.2	Positive environmental impacts	7

7.2.1	Direct climate change monitoring	7

7.2.2	Monitoring and control of the GHG emission	8

7.2.3	Indirect monitoring to learn climate features	10

7.3	Negative environmental impacts	10

7.3.1	Use of hazardous materials	10

7.3.2	Indirect emission of GHG	10

8	Requirements for deployment of USN applications and services for mitigating the climate change	10

8.1	Environmental friendly resources	11

8.1.1	Materials for elements	11

8.1.2	Batteries	11

8.2	Energy efficiency	11

8.2.1	Energy efficient hardware setting	12

8.2.2	Energy efficient protocol	12

8.2.3	Energy efficient applications and services	12

8.3	Operation conditions of GHG sensors	13

Bibliography	13

Draft new Recommendation ITU-T F.USN-CC

Deployment guidelines for ubiquitous sensor network (USN) applications and services for mitigating the climate change

AAP Summary

To be provided before Consent.

Summary

This Recommendation provides deployment guidelines for Ubiquitous Sensor Network (USN) applications and services for mitigating the climate change.

Keywords

Ubiquitous Sensor Network (USN), Climate Change (CC), Greenhouse Gas (GHG)

Scope

This Recommendation provides deployment guidelines for Ubiquitous Sensor Network (USN) applications and services for mitigating the climate change. The scope of this Recommendation includes:

- Overview of climate change monitoring;

- Analysis of environmental impact by USN applications and services; and

- Requirements for deployment of USN applications and services for mitigating the climate change.

Monitoring the climate change covers monitoring status of GHG emissions as well as monitoring the climate change by tracing temporal changes of GHG emissions.

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]	ITU-T Recommendation Y.2221 (2009), Requirements for support of Ubiquitous Sensor Network (USN) applications and services in the NGN environment.

Definitions

Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	climate change [b-IPCC]: Climate change refers to a statistically significant variation in either the mean state of the climate or in its variability, persisting for an extended period (typically decades or longer). Climate change may be due to natural internal processes or external forces, or to persistent anthropogenic changes in the composition of the atmosphere or in land use. Note that the United Nations Framework Convention on Climate Change (UNFCCC), in its Article 1, defines “climate change” as: “a change of climate which is attributed directly or indirectly to human activity that alters the composition of the global atmosphere and which is in addition to natural climate variability observed over comparable time periods.” The UNFCCC thus makes a distinction between “climate change” attributable to human activities altering the atmospheric composition, and “climate variability” attributable to natural causes.

3.1.2	greenhouse gas [ISO 14064-1]: Gaseous constituent of the atmosphere, both natural and anthropogenic, that absorbs and emits radiation at specific wavelengths within the spectrum of infrared radiation emitted by the Earth's surface, the atmosphere and clouds.

3.1.3	sensor [ITU-T Y.2221]: Electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.4	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.5	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.6	ubiquitous sensor network [ITU-T Y.2221]: A conceptual network built over existing physical networks which make use of sensed data and provide knowledge services to anyone, anywhere and at anytime, and where the information is generated by using context awareness.

3.1.7	USN middleware [ITU-T Y.2221]: A set of logical functions to support USN applications and services.

Terms defined in this Recommendation

None.

Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| API CC GHG UNFCCC | Application Program Interface Climate Change Greenhouse gas United Nations Framework Convention on Climate Change |
| --- | --- |
| USN | Ubiquitous Sensor Network |

Conventions

None.

Overview of climate change monitoring

Editor’s note: Relevant contributions are invited for overview of climate change monitoring. If no more relevant contributions are submitted before Consent, this editor’s note should be removed.

Global GHG monitoring network

Monitoring Greenhouse gas (GHG) emissions as well as the climate change requires that GHG sensors, sensor nodes and sensor networks be installed nationally and/or globally. A nation-wide GHG monitoring network may interwork with a global GHG monitoring network whose unique example is shown in Figure 1and maintained by the Global Atmosphere Watch (GAW) programme of the World Meteorological Organization (WMO).

Figure 1 – The WMO-GAW global network for GHG

Local GHG monitoring network

[b-IPCC Guidelines] define three Tiers for estimating GHG emissions from fossil fuel combustion as follows:

- The Tier 1 method is fuel-based, since emissions from all sources of combustion can be estimated on the basis of the quantities of fuel combusted (usually from national energy statistics) and average emission factors. Tier 1 emission factors are available for all relevant direct greenhouse gases.

- The Tier 2 method is estimated from similar fuel statistics as used in the Tier 1 method, but country-specific emission factors are used in place of the Tier 1 defaults because different specific fuels, combustion technologies or even individual plants may produce different country-specific emission factors.

- The Tier 3 method uses either detailed emission models or measurements and data at individual plant level where appropriate. Properly applied, these models and measurements should provide better estimates primarily for non-CO2 greenhouse gases, though at the cost of more detailed information and effort.

The Tier 3 method allows an enterprise to measure real GHG emissions to avoid overestimations which may happen due to the conservativeness principle by which conservative assumptions, values and procedures are used when data and assumptions are uncertain and the cost of measures to reduce uncertainty is not worth the increase in accuracy. Conservative accounting results for GHG emissions may be more likely to be overestimated than underestimated.

Enterprises may install a local GHG monitoring network at their plant level.

Analysis of environmental impact by USN applications and services

Deployment elements of USN

[ITU-T Y.2221] define the USN as a conceptual network and an informational infrastructure which delivers sensed information and knowledge services to anyone at anywhere and anytime where the information and knowledge is developed by using context awareness.

USN applications and services are established by integration of sensor network services into network infrastructure. They are applied to everyday life in an invisible way as everything is virtually linked by pervasive networking between users (including machine and human) and sensor nodes, and relayed through intermediate networking entities such as application servers, middleware entities, access network entities, and USN gateways. Integrating of the elements, USN applications and services can be used in many civilian application areas such as industrial automation, home automation, agricultural monitoring, healthcare, environment, pollution and disaster surveillance, homeland security, and military field.

Figure 2 shows elements of deploying USN applications and services to mitigate the climate change. They may cause negative impacts as well as positive impacts to the environment.

Figure 2 – Deployment elements of USN applications and services

Positive environmental impacts

USN is a key technology to mitigate the climate change by monitoring diverse environmental data and enabling energy consuming sources to be controlled according to the environmental data.

Sensor nodes can measure and deliver diverse environmental data such as:

- pressure, humidity, temperature, light, chemicals, strain and tilt, speed and acceleration, magnetic fields, vibrations, motion, metal detection, and sound.

The sensing parameters are used to trace the climate change and understand climate phenomena. The issues are how to deliver the sensed data and to manage, present and exploit the data to derive value-added information for countering the climate change. This section will briefly introduce examples of how USNs are applied to mitigate the climate change.

Direct climate change monitoring

USN applications and services provide direct monitoring for the acquisition of climate data. For example, marine environment monitoring and glacier status monitoring help trace continuous environmental changes.

In order to help counter the climate change, it is important to monitor the climate to verify if changes to the environment are caused by human influence or natural phenomena. Research in using sensor networks to monitor the climate has been studied for decades, and the technology and techniques have been recently developed into a viable solution for monitoring the climate change. With many experiments, it is proved that USN based monitoring system gives valuable data.

The marine environment monitoring of Figure 3 shows an example of direct environment monitoring. The data of the sensor nodes used to monitor real-time status of the  marine and glacier environment is transmitted to the local monitoring and management system.

Figure 3 – Marine and glacier environment monitoring example

Upper air current and atmospheric state monitoring is presented as another direct climate monitoring example which embeds slightly different features than wide area of climate monitoring shown in Figure 4. According to the altitude changes, temperature, humidity and flow of the atmosphere are the key information for understanding the climate change of a certain region.

Figure 4 – Upper air current and upper atmosphere state monitoring example

Monitoring and control of the GHG emission

USN applications can be developed to automatically monitor and limit power consumption levels. Various USN applications have the ability to monitor electric power consumption and air pollution to alert users when their systems exceed established thresholds.

A good example of this category of USN applications is a management system of various components of an urban infrastructure such as roads, sewerage, water, and gas lines. When USN systems sense a defect, the systems will activate corresponding maintenance systems to correct the malfunction. For example, a road management system captures road conditions and provides them to drivers with additional weather information. In addition, USN applications can help reduce GHG emissions caused by stop-and-go traffic through rerouting traffic to less congested routes thereby reducing GHG emissions.

Figure 5 – Management of city facilities example

Another example of monitoring energy and GHG emission is home and commercial building automation. Light bulbs can automatically control the appropriate brightness based on information from motion sensors and ambient light. Home appliances and other electronic gadgets can save energy when not in use. Controlling power consumption levels and GHG emissions of commercial building complexes are more complex than controlling the consumption and emissions levels of single family homes. However, the same type of automation equipment or similar concept of systems can be used in commercial buildings. Home and building monitoring servers are able to show the monitored power consumption levels to allow owners to adjust usage levels appropriately. These monitoring and control systems are known to reduce GHG emissions in average about 10%.

Figure 6 – Home GHG monitoring example

Indirect monitoring to learn climate features

There are many USN applications that allow indirect monitoring for the acquisition of climate data. This type of USN applications is essential in allowing researchers to analyse and understand the climate change. Understanding the climate change is the first step in developing strategies to deal with impending crises that could threaten global supplies for drinking, sanitation and irrigation.

The hydro watch USN application builds wireless sensor networks to more closely examine the water cycle. This is a good example of how USN applications can be used to understand climate phenomena. USN applications can be extensively deployed to monitor any environmental changes and help understand the cause of the change. The results of the collected data can be used to predict future changes.

Sensor nodes may also be installed in green houses or in open fields. The sensor network application monitors the agricultural environment and learns the plants’ habitat in order to help manage optimal growing conditions for plants.

Negative environmental impacts

As global awareness on the climate change arises, in the ICT sector, the concerns on environmental impacts of the electric & electronic products, restriction of the use of hazardous substances and eco-design are increasing. Also, the GHG emission by-products are generated from all life cycle of raw materials through materials processing, manufacturing, distribution, use, repair and maintenance, and disposal or recycling of products. USN is not an exception in this aspect although it can be used in wide area giving positive environmental impact.

Use of hazardous materials

The elements of USN contain physical equipment such as gateways, sensor nodes, sensors, and batteries. Especially, small sensor nodes mostly have battery-powered designs. Batteries contain heavy metals such as mercury, lead, cadmium, and nickel, which can contaminate the environment when batteries are improperly disposed. If the used sensor node cannot be collected, the electronics waste generated from USN physical equipment and certain metals can release hazardous elements in the ash produced by the combustion process. Therefore, the recovery of the polluted environment by electronics waste causes another GHG emission.

Indirect emission of GHG

USN applications and services will cause an environmental load in each life cycle phase. However, most of the environmental load is caused by using electric power in the use phase. Consuming electric power causes indirect GHG emission from power plants (e.g. thermoelectric power plant, etc.) where the GHG emissions were caused during electric power generation.

Requirements for deployment of USN applications and services for mitigating the climate change

USN applications and services are not free from GHG emission since it has positive and negative environmental impacts as specified in section 7.2 and 7.3. Yet, they have larger positive impact on mitigating the climate change in various areas so that it is important to environment-friendly deploy and utilize USN applications and services.

In addition, eco-design and eco-operations must be considered in sensor network gateways and other dedicated servers as well as sensor nodes.

Environmental friendly resources

Sensor nodes are designed and manufactured in small sizes with small memory and low processing power, and run on very limited power supplied by non-rechargeable batteries. This basic design principle of sensor nodes with small sizes and low processing power makes USN applications and services a good solution to pursue low carbon emissions. However, it still has many areas to consider such as materials of elements, batteries, resource recycling, etc. Especially, the use of solar batteries or other alternative energy-friendly energy sources must be taken into account.

Materials for elements

In case of using sensors in order to monitor the weather or the environmental information, the environmental load due to sensors has to be minimized. Generally, the GHG emission by products is generated from all life cycle from raw material acquisition to final disposal of products. In addition, environmental load is also caused by using the harmful raw material. Therefore, if the used sensor node can be collected, the GHG emission can be reduced by reusing or recycling, and even if the used sensor node cannot be collected, the environment load can be minimized if the sensor node is made of the environment-friendly material. Thus, elements of USN have the following considerations:

- To use environment-friendly materials for sensor node and related equipment.

- To use recyclable materials for sensor node and/or reusable sensor node.

- To avoid hazardous materials for sensor node and related equipment.

- To manage the location information of sensor nodes for collection.

Batteries

Batteries and energy resources containing hazardous materials have serious impact on the environment not only GHG emission.  The fact that sensor nodes are often used in mobile situation and require frequent maintenance of batteries can be a cause of unnecessarily large amount of GHG emission. That is, energy saving and harvesting using the environment-friendly resources, such as solar energy, are able to minimize the GHG emission. Thus, batteries for sensor node have the following considerations:

- To use environment-friendly or rechargeable batteries.

- To use high capacity battery for reduction electronic waste.

- To use alternative source of environment-friendly energy (e.g. solar energy, electromagnetic energy, thermal energy).

Energy efficiency

Figure 7 shows energy consumption of sensor node. The total energy used in the sensor node is calculated as the sum of the energy used in each part of the sensor node. Particularly, the energy is most used in communication. Energy used in calculation and other tasks is relatively small. Figure 7 shows that the overall energy efficiency can be drastically increased by designing low energy consumption for communications and by energy efficient operation of communications.

Figure 7 – Energy consumption for a typical sensor reported in [b-IEEE VTC].

This clause classifies three categories of energy efficiency which should be considered in not only wireless sensor networks but also wired sensor networks.

Energy efficient hardware setting

Sensor nodes mainly rely on small batteries for their life time, all programs must be implemented with small code size and cause minimal power consumption. Power-aware networking and data delivery are a pivot feature for the longevity of necessary nodes and batteries to reduce electronic waste. Thus, hardware setting for sensor node has the following considerations:

- To place efficient number of devices; density and network diameter differ by applications and services and redundant communication scope may cause unnecessary receiving power consumption, and too scarce deployment may cause unnecessary increasing of retransmission which consume a lot of transmission power.

- To consider radio power and interferences; especially in indoor environment this has many obstacles in radio transmissions.

Energy efficient protocol

For efficient energy consumption, diverse modes (e.g. sleeping, idle, hibernate, operation modes) and their efficient operations must be supported. The sampling rate for gathering data may be different for each and every application and service. Thus, protocol for sensor node and sensor network have the following considerations:

- To support diverse modes (e.g. sleeping, idle, hibernate, operation modes).

- To be implemented in small code size.

- To minimize sensing, calculation and communication.

- To equally consume energy on sensor nodes at the same networks.

- To support self-recovery, tolerant networks, and remote management (movement prevention for frequent maintenance).

Energy efficient applications and services

USN applications and services for different purposes are being developed such as climate monitoring system, home or building automation system etc. These existing USN applications and services create other new services by convergence and they can be used to the mitigate climate change. Thus, applications and services for USN have the following considerations:

- To reduce the operations of sensor node.

- To perform the processing load on server.

- To reuse already deployed USN applications and services. (when applicable)

- To develop USN applications and services for multi-use of the sensed data (e.g. database schema, API, USN middleware, etc.).

- To include the sensor network management function in USN applications and services for automatic checking and remote reset of the USN elements malfunction.

- To analyse application of USN applications and services for energy saving (e.g. electric lighting control, ventilation control, air conditioning and heating control, etc.)

NOTE – Energy saving should be carefully considered when USN applications and services apply in the facilities directly related to the people’s life (e.g. surgery room, intensive care unit, emergency room, incubator, etc.).

Operation conditions of GHG sensors

A GHG monitoring sensor network belonging to the WMO-GAW global network shall comply with the guidelines of the Global Atmosphere Watch (GAW) programme of the World Meteorological Organization (WMO).

A national GHG monitoring sensor network may have to be established by national regulations, domestic or international standards which may consist of a set of specifications to describe conditions for geographic locations, target GHGs, sensing frequencies, standard reference GHGs, calculation formulas, meter configurations, device positions, and so on. Practitioners should check them to deploy USN applications and services.

A local GHG monitoring sensor network also shall abide by any national regulation, domestic or international standard. If this requirement is not supported, monitoring results may not be certified and rejected by GHG management authorities.

Bibliography

[b-IPCC]	IPCC Working Group 1 Report (2007), Glossary of Terms used in the IPCC Fourth Assessment Report

[b-IPCC Guidelines] 2006 IPCC Guidelines for National Greenhouse Gas Inventories

[b-IEEE VTC]	Proc. of IEEE VTC (2003), Aggregation Tree Construction in Sensor Networks

_____________________