| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4230 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.DPI |  |  |  |
| Title: | H.248.DPI “Gateway control protocol: H.248 Support for deep packet inspection” (New): Input draft (Ed. 0.13) |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the editor’s input for draft new H.248.DPI “Gateway control protocol: H.248 Support for deep packet inspection”.

2.	Discussion

This document is a copy of TD 745R1/WP2 from the SG16 meeting in Geneva, Apr/May 2012. There are no changes to the document.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.DPI Ed. 0.0 | Initial proposal (TD-24) from WP 2/16 Shanghai meeting (2010-03) based on AVD-3856 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1003_Sha/AVD-3856.zip |
| H.248.DPI Ed. 0.1 | Output (TD 376 (WP2/16)) from ITU-T SG16 Geneva meeting, July 2010 Location: http://itu.int/md/T09-SG16-100719-TD-WP2-0376 |
| H.248.DPI Ed. 0.2 | Initial draft (AVD-4033) to WP 2/16 Research Triangle Park (USA) meeting (2010-11) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/AVD-4033.zip |
| H.248.DPI Ed. 0.3 | Output draft (TD-25) from WP 2/16 Research Triangle Park (USA) meeting (2010-11) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1011_Res/TD-25.zip |
| H.248.DPI Ed. 0.4 | Input (TD 456 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0456 |
| H.248.DPI Ed. 0.5 | Output (TD 456R1 (WP2/16)) from ITU-T SG16 Geneva meeting, March 2011 Location: http://itu.int/md/T09-SG16-110314-TD-WP2-0456R1 |
| H.248.DPI Ed. 0.6 | Input (AVD-4049) to WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/AVD-4049.zip |
| H.248.DPI Ed. 0.7 | Output draft (TD-37) from WP 2/16 Andover (USA) meeting (2011-07) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1107_And/TD-37.zip |
| H.248.DPI Ed. 0.8 | Input (TD 618 (WP2/16)) from ITU-T SG16 Geneva meeting, Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0618/ |
| H.248.DPI Ed. 0.9 | Output (TD 618R1 (WP2/16)) from ITU-T SG16 Geneva meet., Nov/Dec 2011 Location: http://itu.int/md/T09-SG16-111121-TD- WP2-0618R1 |
| H.248.DPI Ed. 0.10 | Input (AVD-4160) to WP 2/16 Geneva meeting (2012-02) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/AVD-4160.zip |
| H.248.DPI Ed. 0.11 | Input (TD 745 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0745 |
| H.248.DPI Ed. 0.12 | Output (TD 745R1 (WP2/16)) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0745R1 |
| H.248.DPI Ed. 0.13 | Input (AVD-4230) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4230.zip |
| H.248.DPI Ed. 0.14 | Output (TD-xx) from WP 2/16 Brisbane meeting (2012-09) Location: |
| H.248.DPI Ed. 0.15 |  |

CONTENTS

Page

1 Scope 6 2 References 7 3 Definitions 7 3.1 Terms defined elsewhere: 7 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 8 5 Conventions 8 6 Inspection rule pointer package 9 6.1 Properties 9 6.1.1 Pointer to rule conditions 9 6.2 Events 9 6.2.1 <Event Title> 9 6.3 Signals 10 6.3.1 <Signal Title> 10 6.4 Statistics 11 6.4.1 <Statistic Title> 11 6.5 Error Codes 11 6.6 Procedures 11 7 Inspection rule container package 11 7.1 Properties 11 7.1.1 Container for rule 11 7.2 Events 12 7.2.1 <Event Title> 12 7.3 Signals 12 7.3.1 <Signal Title> 12 7.4 Statistics 13 7.4.1 <Statistic Title> 13 7.5 Error Codes 13 7.6 Procedures 13 Appendix I Living List – MGC aspects of DPI policy rules 14 I.1 MGC-locally vs MGC-remotely generated DPI policy rules 14 Appendix II Living List – Examples of DPI and non-DPI Policy Rules 15 II.1 Examples of DPI and non-DPI Policy Rules 15 II.2 Discussion 16 II.2.1 Conventions 16 II.2.2 Non-DPI scenarios 17 II.2.3 DPI scenarios 20 Appendix III Living List – H.248 aspects for signalling DPI policy rules 29 III.1 Discussion of container-based rule signalling 29 III.2 Discussion of potential signalling methods 29 Appendix IV Living List – Discussion of Policy Specification Language support 34 IV.1 Discussion of Policy Specification Language Candidates 34 IV.2 SNORT Analysis 35 IV.2.1 Signalling 35 IV.2.2 Structure 35 IV.2.3 Addressing 35 IV.2.4 Actions 35 IV.2.5 Conditions 36 IV.3 Potential Issues 37 IV.4 Conclusions 38 Appendix V Living List – Relation to other ITU-T Recommendations on DPI 39

List of Tables

Page

Table 1 – "Non-DPI" Example I – "Packet size range policing" 17 Table 2 – "Non-DPI" Example II – "Flow-level byterate policing" 18 Table 3 – "Non-DPI" Example III – "Well-known port SIP message policing" 19 Table 4 – "Non-DPI" Example IV – "General IP 5-tuple policing" 20 Table 5 – "DPI" Example VI – "Media type/media format policing" 21 Table 6 – "DPI" Example VII – "Detect application X of an "application-agnostic" bearer" 22 Table 7 – "DPI" Example VIII: "Detect application X = 'BitTorrrent'" 23 Table 8 – "DPI" Example X: "Detect application X = 'Skype'" 24 Table 9 – "DPI" Example X: "Detect application X = 'Open Game Protocol'" 25 Table 11 – "DPI" Example XI: "Detect application X = 'audio channel in a multi-channel media application'" 26 Table 11 – "DPI" Example XII: "Detect & measure application X = 'greek Jabber traffic'" 26

List of Figures

Page

Figure 1 – Packet Inspection – Example of DPI, here layered architecture according OSI-BRM (X.200) and the example when DPI covers also Layer 2 7 Figure I.1 – Possible Roles of a Policy Control Server (with embedded MGC entity) - MGC-locally (A) vs MGC-remotely (B) generated DPI policy rules 14

Draft new ITU-T Recommendation H.248.DPI

Gateway Control Protocol:
H.248 support for deep packet inspection

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

Editor's note: initial proposal for scope -

The scope of this document covers (at least)

- the "policy specification language" (PSL) topic (as outlined in AVD-3855);

- possible relations with other ITU-T H.248.x-series Recommendations (e.g., work items on H.248.Packets, H.248.Filter (note: there might be Filter Terms for 'DPI')); and

- push/pull mode options for DPI policy rules (as typically required for distributed policy control systems; see also H.248.55).

Figure 1 – Packet Inspection – Example of DPI, here layered architecture according OSI-BRM (X.200) and the example when DPI covers also Layer 2

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.200]	ITU-T Recommendation X.200 (1994) | ISO/IEC 7498-1:1994, Information technology. Open Systems Interconnection. Basic reference model: The basic model.

[IETF RFC 791]	IETF RFC 791 (1981), Internet Protocol.

# 3	Definitions

This Recommendation defines the following terms:

## 3.1	Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

FIXTHIS

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Deep Packet Inspection (DPI): A method of packet filtering that applies policy (filter) rules using policy (filter) conditions (see Figure 1) based on protocol elements across multiple protocol layers (see e.g. reference model [ITU-T X.200]) in order to apply associated policy (filter) action(s) on the inspected packet.

The following non-standard terms, often used in literature, should be noted:

- "Shallow Packet Inspection" (SPI) = L3,4HI

- "Medium Depth Packet Inspection" (MPI) = L3,4HI  L4+HI

- "Deep Packet Inspection" (DPI) = L2HI  L3,4HI  L4+HI  L7PI = L2HI  L3,4HI  L4PI

3.2.2	DPI for packets according IETF-BRM protocol layering (abbreviated as DPIIETF-BRM): The IETF basic reference model, given by [IETF RFC 791], relates to the OSI-BRM without protocol layers L5 and L6. The DPIIETF-BRM is thus based on an absolute protocol layering model. There is DPIIETF-BRM in case of policy rules for packet inspection with policy conditions based on elements related to protocol layers above the transport layer.

{Proposal for an Editors notes:

- Above definition may be mapped on other protocol reference model using an absolute  numbering scheme. The DPIIETF-BRM definition may be thus exemplary.

- Complementary (or as alternative) might be a DPI definition developed for packets according a relative protocol layering scheme. E.g., inspection could be related to "distinct levels" of encapsulation, n, n+1, n+2....  Where n would provide the datum for some particular case.  These "levels" may or may not correspond to OSI layers.

- Another idea: the DPI function could be described by using a formal specification language (e.g., ABNF). E.g., above text indicates that the DPI functional behaviour may be expressed on the basis of policy rules ("and there would be already formal specification baseline available"). […]

Contributions are solicited.}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

| DPI | Deep Packet Inspection |
| --- | --- |
| LxHI | Layer x Header Inspection |
| LyPI | Layer y Payload Inspection |
| Lz+ | Layer(s) above z (e.g. L4+) |
| MPI | Medium depth Packet Inspection |
| SPI | Shallow Packet Inspection |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation if any>

# 6	Inspection rule pointer package

|  | Package Name: | Inspection rule pointer package |
| --- | --- | --- |
|  | Package ID: | irp (0x????) |
|  | Description: | This package allows the MGC to indicate to the MG DPI policy rules. The DPI policy rules and behaviour are out of scope of this Recommendation. Such rules could be e.g. based on "SNORT" as a typical policy rule specification language for DPI. […] |
|  | Version: | 1 |
|  | Extends: | None |

## 6.1	Properties

### 6.1.1	Pointer to rule conditions

|  | Property Name: | Pointer to rule conditions |
| --- | --- | --- |
|  | Property ID: | ptr (0x0001) |
|  | Description: | This property allows the MGC to indicate which rule conditions should be used for inspection of incoming packets by referring to a pointer to a data object which contains DPI policy rule conditions. The pointer is in a URI format which includes also a generic name (which may represent a globally or locally unique identifier). The applied pointer scheme is subject of an H.248 profile specification. E.g., the URI format could follow a URI scheme such as “file”, “ftp”, “https”, etc. NOTE – The concept of a “pointer to a local or remote data object” is also supported by [ITU-T H.248.9] and [ITU-T H.248.69] with concrete data object types “media”, “message” and “SIEVE script” respectively. |
|  | Type: | String |
|  | Possible values: | The string is of type "URI". [IETF RFC 3986] See: http://www.iana.org/assignments/uri-schemes.html |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 6.2	Events

### 6.2.1	<Event Title>

|  | Event Name: | <event name> |
| --- | --- | --- |
|  | Event ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |

#### 6.2.1.1	EventsDescriptor parameters

##### 6.2.1.1.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

#### 6.2.1.2	ObservedEventsDescriptor parameters

##### 6.2.1.2.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

## 6.3	Signals

### 6.3.1	<Signal Title>

|  | Signal Name: | <name> |
| --- | --- | --- |
|  | Signal ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Signal Type: | <type> |
|  | Duration: | <duration> |

#### 6.3.1.1	Additional parameters

##### 6.3.1.1.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

## 6.4	Statistics

### 6.4.1	<Statistic Title>

|  | Statistic Name: | <name> |
| --- | --- | --- |
|  | Statistic ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <type> |
|  | Possible values: | <values> |
|  | Level: | <Termination, Stream, Either> |

## 6.5	Error Codes

6.5.1	<Error Code Title>

|  | Error Code #: | <number> |  |
| --- | --- | --- | --- |
|  | Name: | <name> |  |
|  | Definition: | <definition> |  |
|  | Error Text in the Error Descriptor: |  | <error text to return> |
|  | Comment: | <comment> |  |

## 6.6	Procedures

<The procedures associated with the package>

# 7	Inspection rule container package

|  | Package Name: | Inspection rule container package |
| --- | --- | --- |
|  | Package ID: | irc (0x????) |
|  | Description: | <description> |
|  | Version: | <version> |
|  | Extends: | <extended Package ID and version> |

## 7.1	Properties

### 7.1.1	Container for rule

|  | Property Name: | Container for rule |
| --- | --- | --- |
|  | Property ID: | cfr (0x0001) |
|  | Description: | <description> |
|  | Type: | <type> |
|  | Possible values: | <values> |
|  | Default: | <value> |
|  | Defined in: | <Local, Remote, LocalControl, TerminationState, ContextAttribute> |
|  | Characteristics: | <ReadOnly, Read/Write> |

## 7.2	Events

### 7.2.1	<Event Title>

|  | Event Name: | <event name> |
| --- | --- | --- |
|  | Event ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |

#### 7.2.1.1	EventsDescriptor parameters

##### 7.2.1.1.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

#### 7.2.1.2	ObservedEventsDescriptor parameters

##### 7.2.1.2.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

## 7.3	Signals

### 7.3.1	<Signal Title>

|  | Signal Name: | <name> |
| --- | --- | --- |
|  | Signal ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Signal Type: | <type> |
|  | Duration: | <duration> |

#### 7.3.1.1	Additional parameters

##### 7.3.1.1.1	<Parameter Title>

|  | Parameter Name: | <name> |
| --- | --- | --- |
|  | Parameter ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <types> |
|  | Optional: | <yes/no> |
|  | Possible values: | <values> |
|  | Default: | <value> |

## 7.4	Statistics

### 7.4.1	<Statistic Title>

|  | Statistic Name: | <name> |
| --- | --- | --- |
|  | Statistic ID: | <(text ID), (binary ID (0x????)> |
|  | Description: | <description> |
|  | Type: | <type> |
|  | Possible values: | <values> |
|  | Level: | <Termination, Stream, Either> |

## 7.5	Error Codes

7.5.1	<Error Code Title>

|  | Error Code #: | <number> |  |
| --- | --- | --- | --- |
|  | Name: | <name> |  |
|  | Definition: | <definition> |  |
|  | Error Text in the Error Descriptor: |  | <error text to return> |
|  | Comment: | <comment> |  |

## 7.6	Procedures

<The procedures associated with the package>

Appendix I
Living List – MGC aspects of DPI policy rules

(This appendix does not form an integral part of this Recommendation)

{Editor's note: The text below should be integrated into the main document. Further contribution is invited on this issue}

## I.1	MGC-locally vs MGC-remotely generated DPI policy rules

Below Figure I.1 illustrates the two options of MGC-locally vs MGC-remotely generated DPI policy rules, which may impact the question related to PSL candidates at the H.248 interface.

Figure I.1 – Possible Roles of a Policy Control Server (with embedded MGC entity) - 
MGC-locally (A) vs MGC-remotely (B) generated DPI policy rules

Appendix II
Living List – Examples of DPI and non-DPI Policy Rules

## II.1	Examples of DPI and non-DPI Policy Rules

{Editor's note: this living list item intends to help progress the thinking related to H.248 signalling of policy rules and suitable grammar for rule specification. All examples here provide rule descriptions in a natural language as informal specification. A follow-up action might be the exercise in using a formal language towards the evaluation of appropriate policy specification language(s).
Further: it was observed that DPI policy rules and non-DPI policy rules are two separate, disjoint sets of rules. They are rather overlapping. E.g., DPI policy rules may contain e.g. non-DPI policy condtions, etc. This aspect may need consideration from PSL perspective.}

Overview of living list item II:

II.2	Discussion

II.2.1	Conventions

II.2.2	Non-DPI scenarios

II.2.2.1	Example I: "Packet size range policing"

II.2.2.2	Example II: "Flow-level byterate policing"

II.2.2.3	Example III: "Well-known port SIP message policing"

II.2.2.4	Example IV: "General IP 5-tuple policing"

II.2.2.5	Example V: "Extensive filtering"

II.2.3	DPI scenarios

II.2.3.1	Example VI: "Media type/media format policing"

II.2.3.2	Example VII: "Detect application X of an "application-agnostic" bearer"

II.2.3.2.1	Example VIII: "Detect application X = 'BITTORRRENT'"

II.2.3.2.2	Example IX: "Detect application X = 'SKYPE'"

II.2.3.2.3	Example X: "Detect application X = 'OPEN GAME PROTOCOL'"

II.2.3.2.4	Example XI: "Detect application X = 'audio channel in a multi-channel media 
					application'"

II.2.3.2.5	Example XII: "Detect & measure application X = 'greek Jabber traffic'"

II.2.3.3	Example XIII: "TCP attack detection"

II.2.3.4	Example XIV: "Remove invalid MIME attachments from Instant Messaging"

II.2.3.5	Example XV: "RTCP Block Type Filtering"

II.2.3.6	Example XVI: "Application-specific traffic handling"

II.2.3.7	Example XVII: "Abnormal packet size detection"

II.2.3.8	Example XVIII: "Delete old packets of application X"

## II.2	Discussion

### II.2.1	Conventions

We have to use a generic high-level format for (DPI & non-DPI) policy rules (see Figure 1) due to the pending question of appropriate H.248 syntax for signalling such rules. The generic format discriminates the three blocks of

    - condition(s),

    - action(s) and

    - identifier / name section (with priority / precedence indication due to possible multiple rules).

Note - This document just focuses on the condition/action part in order not to overload the discussion with too many aspects.

Figure 1 – Generic high-level format of a (DPI & non-DPI) Policy Rule

Such a generic format is actually independent of H.248, suited for all existing policy control and policy management protocols.

NOTE: A filter rule is just a dedicated type of a policy rule. In scope of this contribution the terms filter rule and policy rule are synonym (however, in general there are also other policy actions than filtering (of packets, flows, traffic, etc)).

Figure 2 recalls again the framework: policy rules are assigned to H.248 contexts, terminations (non-Root or Root) or/and streams. Policy rules may be signalled, but correspondent H.248 property values may be also provisioned. There's thus a signalling and management aspect (or policy control and policy management).

Figure 2 – General principle for (DPI) policy rules

### II.2.2	Non-DPI scenarios

We'd like to recall first some non-DPI policing scenarios.

#### II.2.2.1	Example I: "Packet size range policing"

This example illustrates packet size range policing:

Table 1 – "Non-DPI" Example I – "Packet size range policing"

| Policy Rule ("Packet size range policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L3 DA = (H.248 LD(A))?" AND { C2: "L3 PDU size > (H.248 pacs/M)?" OR C3: "L3 PDU size < (H.248 pacs/mpu)?"} | Then: A1: "discard IP packet" AND A2: "Update Statistic H.248 pacs/dp" |

This example supposes a wildcard ALL L4 port value (thus LD(P) = ALL; see H.248.39, cl. 6.11). The received H.248 IP packets must match a particular range concerning a given packet size distribution function, which is bounded by a maximum and minimum size value (using H.248.53 Packet size package). Violating packets are not silently discarded, rather counted by the pacs/dp statistic.

#### II.2.2.2	Example II: "Flow-level byterate policing"

This example illustrates flow-level byterate policing. The notion of 'flow' relates to the H.248 'flow', i.e. a traffic level above the H.248 stream (here: see clause 6.6.2/H.248.53).

Table 2 – "Non-DPI" Example II – "Flow-level byterate policing"

| Policy Rule ("Flow-level byterate policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L3 DA = (H.248 LD(A))?" AND C2: "L4 DP = (H.248 LD(P))?" AND C3: "L3 SA = (H.248 RD(A))?" AND C4: "L4 SP = (H.248 RD(P))?" AND { C5: "GBRA(Rp, Bp) = not OK?" OR C6: "GBRA(Rs, Bs) = not OK?"} | Then: A1: "discard IP packet" AND {/** if peak-rate violation A2,1: "Update Statistic H.248 tmanr/pvp" A2,2: "Update Statistic H.248 tmanr/pvo" } {/** if sustainable-rate violation A3,1: "Update Statistic H.248 tmanr/svp" A3,2: "Update Statistic H.248 tmanr/svo" } |

This example supposes incoming IP traffic on a local transport connection endpoint (= LD(A,P)) plus the inspection of the remote source transport address (= RS(A,P)), which is supposed to be equal here to the remote destination transport address (= RD(A,P)) due to a symmetry assumption.

A 2-stage policing shall be enforced (peak- and sustainable rate level).

The traffic policer conditions (for GBRA algorithm) are derived from tman v2 properties: e.g. the first list item values for flow 1.

Violating packets counted by the correspondent tmanr statistics, on packet and byte level.

#### II.2.2.3	Example III: "Well-known port SIP message policing"

This example is based on clause III.1.1/H.248.43 (see description):

Table 3 – "Non-DPI" Example III – "Well-known port SIP message policing"

| Policy Rule ("Well-known port SIP message policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If NOT: C1: "L3 DA = (H.248 LD(A))?" AND C2: "IP PCI 'protocol' = 'UDP' OR 'TCP'?" AND C3: "L4 DP = well known port for 'SIP' OR 'SIP-TLS'?" | Then: A1: "discard IP packet" |

This example uses compound policy conditions, i.e. a combination of multiple simple policy conditions (see also H.248.43). However, this aspect is minor (because every compound condition may be equally transformed in a number of simple conditions).

#### II.2.2.4	Example IV: "General IP 5-tuple policing"

The considered 5-tuple relates here to the 4-tuple of the IP transport connection endpoints plus the 1-tuple of protocol type. H.248.43 supports "general IP 5-tuple policing" by specifying a set of 5-tuples in parallel. The example here is based on clause III.1.3/H.248.43 (see description):

Table 4 – "Non-DPI" Example IV – "General IP 5-tuple policing"

| Policy Rule ("General IP 5-tuple policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| Incoming IP packet: If NOT: C1: "L3 SA = [101.0.*.0]?" /* wildcard IPv4 OR C2: "L4 DP = 23 OR 14 OR [1442 - 1490] OR 19999 OR 25000?" /* port range OR C3: "IP 'protocol' = 'FTP' OR 'TELNET' OR 'SMTP' OR 'TFPT' OR 'FINGER' OR 'SIP' OR 'SIP-TLS'?" | Then: A1: "discard IP packet" |
| Outcoming IP packet: If: C1: "L3 DA = "[12.8.3.0]/12?" /* wildcard AND C2: "L4 DP = 153 OR 155 OR 157 OR 159 OR 161 OR [732 - 789]?" /* port range AND C3: "IP 'protocol' = '…' OR '…' … OR '…'?" | Then: A1: "discard IP packet" |

The policy rule is of type "non-DPI" because the policy conditions are based on L3 & L4 PCI information elements only.

#### II.2.2.5	Example V: "Extensive filtering"

H.248.76 provides a tool for defining extensive set of filter rules per context.

### II.2.3	DPI scenarios

#### II.2.3.1	Example VI: "Media type/media format policing"

There are H.248 profile definitions for IP-to-IP gateways which support media type/media format policing. Such kind of policy rules may be addressed by specifying allowed SDP values (in the H.248 profile section on SDP information elements).

For instance, the profile could not allow a particular list of RTP payload type values (related to a specific RTP profile type (like e.g. 'AVP')). The H.248 MG must then inspect the RTP header field "payload/packet type" against correspondent values. Not supported media formats shall lead to the blocking of such RTP packets. That's an example of an implicit policy rule because not explicitly signalled over the H.248 interfaces, but enforced due to the successful registration of this profile by the MG.

Table 5 – "DPI" Example VI – "Media type/media format policing"

| Policy Rule ("Media type/media format policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L4+ protocol = RTP?" AND { C2: "RTP PT = NOT {…allowed values…}?" | Then: A1: "silently discard IP (= RTP) packet" |

#### II.2.3.2	Example VII: "Detect application X of an "application-agnostic" bearer"

Figure 3 illustrates the general framework. The created context may be e.g. transport protocol type aware, but application agnostic, according the H.248 signalling information (from MGC to MG) for "regular IP user plane processing".

Figure 3 – General framework for rule category of
"detect application X of an "application-agnostic" bearer"

Such a context could be then overlaid by a policy rule for the inspection of a particular application or a number of applications. Justified e.g. by security concerns, or to check whether users are really using that IP transport connection for the initially requested application.

The rule is here just denoted, but not detailed:

Table 6 – "DPI" Example VII – "Detect application X of an "application-agnostic" bearer"

| Policy Rule ("Detect application X of an "application-agnostic" bearer") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND { C2: "… transport protocol = …?" AND { C3: "'application' = X?" | Then: A1: "…" + possibly other actions |

It may be noted that the notion of "application" shall be not limited to (IP) application protocol types only, rather every kind of "application" which may be unambiguously detected based on correspondent conditions.

Just some random examples for 'X':

- BitTorrent

- Skype

- Open Game Protocol (OGP)

- media format (e.g. video layer)

The choosen application types related to peer-to-peer (P2P) filesharing (a), proprietary conversational service (b), multi-user distributed P2P (c) and e.g., an HTTP-based streaming service (d).

Concrete, detailed policy rules for above scenarios are not relevant for the discussion here (however provided in subclauses II.2.3.2.1 to II.2.3.2.5), but it may be imagined that the underlying policy conditions may be multifaceted, e.g.

- "simple" by searching for a particular bit pattern

- "complex" by executing a regular expression on each packet

- deterministic vs stochastic (e.g. heuristic) rule type

- stateless vs statefull

##### II.2.3.2.1	Example VIII: "Detect application X = 'BitTorrrent'"

This is a HTTP/TCP-based application, which may be roughly detected by looking for specific keywords (e.g. control commands).

Table 7 – "DPI" Example VIII: "Detect application X = 'BitTorrrent'"

| Policy Rule ("Detect application X = 'BitTorrrent") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'TCP'?" AND C3: "L4+ protocol type = HTTP" AND C4: "http control = GET request header" AND C5: "http message content contains element uploaded with a value > 0" AND C6: "…" | Then: A1: "…" + possibly other actions |

##### II.2.3.2.2	Example IX: "Detect application X = 'Skype'"

Example for a bidirectional, statefull DPI policy rule.

NOTE – The used rule itself illustrates just the principal proceeding, but is not necessarily sufficient for a real deployment. There are some policy rule proposals for detecting this application. This is just one out of many.

The details of the rule are minor. Relevant here is the fact that this application has an inherent session concept in terms of a state machine. The DPI policy rule here is related to that state model and associated to the session establishment phase.

Table 8 – "DPI" Example X: "Detect application X = 'Skype'"

| DPI Policy Rule ("Detect Skype session establishment") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| State S1: If: C1,1: "L4 protocol = UDP" AND C1,2: "L4 payload size = 18" AND C1,3: "3rd payload byte = 0x02" | Then: A1,1: "save source IP transport address" A1,2: "save destination IP transport address" A1,3: "save first two bytes of L4 payload" Comment: the FlowID is based here on the 5-tuple for the UDPoIP transport connection and locally stored. |
| State S2: If: C2,1: "FlowID in reverse direction = … (see A1,1 / A1,2)" AND C2,2: "L4 payload size = 11" AND C2,3: "first two bytes of L4 payload = … (see A1,3)" AND C2,4: "3rd payload byte: lower nibble = 7" | Then: none |
| State S3: If: C3,1: "FlowID in initial direction = … (see A1,1 / A1,2)" AND C3,2: "L4 payload size = 23" AND C3,3: "first two bytes of L4 payload = … (see A1,3)" AND C3,4: "3rd payload byte: lower nibble = 3" | Then: none |
| State S4: If: C4,1: "FlowID in reverse direction = … (see A1,1 / A1,2)" AND C4,2: "L4 payload size = 18" AND C4,3: "3rd payload byte = 0x02" | Then: A4,1: "report "successfully Skype detected" |

##### II.2.3.2.3	Example X: "Detect application X = 'Open Game Protocol'"

This is a UDP-based application, which may be simply detected by L4+ Header Inspection (L4+HI).

Table 9 – "DPI" Example X: "Detect application X = 'Open Game Protocol'"

| Policy Rule ("Detect application X of an "application-agnostic" bearer") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'UDP'?" AND C3: "L4-SDU 1st 32-bit-Word = '0xFF…F'?" AND C4: "L4-SDU 2nd 32-bit-Word = '"OGP"'?" | Then: A1: "…" + possibly other actions |

##### II.2.3.2.4	Example XI: "Detect application X = 'audio channel in a multi-channel media application'"

Chosen example: detect 2nd audio channel in a multi-channel AMR application and remove channel.

Table 11 – "DPI" Example XI: "Detect application X = 'audio channel in a multi-channel media application'"

| Policy Rule ("Detect application X = 'audio channel in a multi-channel media application'") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "…" AND C2: "…" AND C3: "Packet contains the hexadecimal string = "0x2321414d525F4D43312E300a"" AND C4: "next 32-bit word contains the hexadecimal string = "0x00000002"" | Then: A1: "Discard Audio Frame within SDU" A2: "Update Statistic" + possibly other actions |

##### II.2.3.2.5	Example XII: "Detect & measure application X = 'greek Jabber traffic'"

This is a XMPP/TCP-based application, which may be simply detected by looking for XML specific control elements.

Table 11 – "DPI" Example XII: "Detect & measure application X = 'greek Jabber traffic'"

| Policy Rule ("Detect & measure application X = 'greek Jabber traffic'") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'TCP'?" AND C3: "L4+ header type = XMPP stream header" AND C4: "XMPP message content contains XML element = "xml:lang='gr'" | Then: A1: "…" + possibly other actions |

For the following examples, the DPI policy rule indications are omitted.

#### II.2.3.3	Example XIII: "TCP attack detection"

There are existing H.248 profiles for IP-to-IP gateways which support TCP-based applications. Such a gateway may principally located at the edge of a security domain, and then requested to enforce some policy rules for well-known TCP attack scenarios.

This is not necessarily a DPI scenario ("the very majority of TCP threat scenarios is non-DPI"), but the correspondent policy conditions are typically "statefull" ("because the inspection may cover multiple TCP/IP packets and the MG may need to drive through the TCP connection state machine …").

#### II.2.3.4	Example XIV: "Remove invalid MIME attachments from Instant Messaging"

Similar another use case: a user may be subscribed for an Instant Messaging (IM) service, realized here as "MSRP-over-TCP" application. The IM service shall allow the transmission of "attachments", which may be encapsulated via MIME ("the low level details are not important here"). The user may be not allowed to issue via his IM client dedicated attachment types. The IM service may be "end-to-end", i.e. without a MSRP relay node. The H.248 MG may be thus requested for MIME attachment type inspection (because e.g. the only "IP hop" instance where all bearer traffic is routed across).

This example relates to a DPI policy rule because besides the TCP header verification, there might be policy conditions related to the

- MSRP header,

- MSRP body 'text',

- MSRP body embedded MIME attachment types,

- etc ("e.g., MIME itself is recursive, which allows to consider MIME-over-MIME …").

#### II.2.3.5	Example XV: "RTCP Block Type Filtering"

That's an example, which may come along with H.248.48 based network applications. E.g., provider A wants to block the distribution of own RTCP-based measurement data to other, peering provider partners.

Such DPI policy conditions for measurement data filtering may look like:

- RTCP report: PT = x1, BT = x2, Field = "..."

In other words, firstly packet type value is checked, then block type value, then a particular field within a block, and if all conditions are true, then the action may be to remove the correspondent metric or to reset the value to default, and to recalculate PCI on all affected protocol layers (e.g., UDP checksum recalculation).

#### II.2.3.6	Example XVI: "Application-specific traffic handling"

The notion of 'handling' shall indicate other possible action types like just "packet accept" or "packet discard" actions.

For instance, action types like

- create packet copy and …;

- sent alert (e.g., H.248 Notify to MGC);

- modify IP header DS/ToS/TC values ("H.248.52 like");

- etc.

#### II.2.3.7	Example XVII: "Abnormal packet size detection"

This example is similar to example I (clause 2.2.1), but the difference may be related to another protocol layer (> L4). E.g., multiple application layer PDUs may be assembled in a single IP packet.

The example may be generalized to "abnormal traffic detection", with correspondent policy conditions for specific traffic characteristics.

#### II.2.3.8	Example XVIII: "Delete old packets of application X"

The notion of 'old' shall be related to the IP header TTL value. The specific example here may illustrate that the condition

"IP PCI "TTL" < y?"

would be tight to a pre-condition of

"'application' = X?".

It may be observed that policy rules itself may be hierarchical, like e.g. conditions embedded in another condition, or an action may refer again to another policy rule, etc.

Appendix III
Living List – H.248 aspects for signalling DPI policy rules

## III.1	Discussion of container-based rule signalling

{Editor's note: there are different options of signalling policy rules across H.248. This living list item intends to help progress the thinking about such options.}

Overview of living list item I.3 – Evaluation of some principle signalling methods for policy rules:

1	Explicit rule signalling

1.1	Explicit rule in native H.248 syntax

1.2	Explicit rule encapsulated in container over H.248

2	Signalling of pointer to rule and rule MG-locally provisioned

3	Signalling of pointer to rule and rule MG-remotely provisioned

{Editor's note (from discussion at 2011-11 meeting): there might be different scopes for such a 'pointer', e.g. a reference to a entire rule or a reference to portions of a rule like just rule conditions part. The 2nd approach seems to be advantageous concerning the flexibility by combinations between rule actions and rule conditions, the 1st approach might be sufficient if there are only specific actions possible for particular rule conditions. The 2nd approach may also facilitate the support of two type of actions: a) actions based on H.248 protocol elements like to notify the MGC or to update an H.248 statistic, and b) actions defined by a PSL. Contributions are solicited.}

## III.2	Discussion of potential signalling methods

Figure III.1 recalls again the generic format used in AVD-3987. The format itself is also applicable for existing non-DPI based H.248 methods for signalling IP-IP gateway functions like filter rules according H.248.43, H.248.53 and H.248.76, but also applicable for describing functional behaviour like NAT-Traversal support (H.248.37), media inactivity detection (H.248.40), QoS marking (H.248.52), application data measurements (H.248.58), IP measurements (H.248.61), etc., even bearer-level ALG behaviour (H.248.78; "with the particular policy action in notifying the MGC").

Figure III.1 – Generic high-level format of a (DPI & non-DPI) Policy Rule

It may be also observed that there must not be necessarily explicit syntax for specific rule elements, e.g.

- Boolean operators in policy conditions may be

- implicit (i.e., part of the protocol semantic of an H.248 property) or

- explicit (e.g., H.248.43 'rules relationship' element); or

- Policy actions may be implicit (e.g., packet discard in H.248.43 & H.248.53) plus statistics.

Before evaluating correspondent protocol syntax for DPI rules, principle signalling methods should be considered (see Figure III.2). The figure outlines three examples for policy rules signalled between a policy decision (here MGC) and policy enforcement point (here MG).

Figure III.2 – Principle signalling methods for policy rules

The high-level principle behind each method is as follows.

The first option (1):

- the complete rule is carried by a single H.248 command.

The second option (2):

- the H.248 command carries a 'pointer' (which relates to an "identifier for a rule"),

- the rule shall be already available in a MG local data base,

- the identifier correlates rule and correspondent H.248 context/termination/stream.

The third option (3):

- as in (2), the H.248 command carries a 'pointer' (which relates to an "identifier for a rule"),

- the rule shall be already available in a MG remote data base,

- the MG issues a query to that data base for requesting the rule,

- the interface between MG and remote data base is non-H.248 ("protocol itself is out of scope of H.248 Recommendations"),

- the downloaded rule is assigned to H.248 context/termination/stream.

Methods (2) and (3) are potential candidates, but out of scope of this document. Method (1) is in scope of the discussion here. There are again sub-alternatives for method (1), see Figure III.3:

Figure III.3 – Two sub-options for the first method

The first sub-option (1.1):

- the rule is encoded in native H.248 syntax, i.e., based on (existing or new) H.248 properties, events, signals, and/or statistics, incl. error codes.

The second sub-option (1.2):

- the rule itself is specified in a non-H.248 language (note: such a language may be termed as policy specification language (PSL)),

- H.248 provides support of a "container" for carrying the rule,

- note: such a "container" could be e.g. already realized by a H.248 property.

The PSL itself would be out of scope of H.248 Recommendations. Some examples of existing PSLs:

- XML

- Perl (Compatible) Regular Expressions

- POSIX Regular Expressions

- JAVA script

- etc

Some observations:

  - the container principle may be actually also relevant for 
method 2 ("container-embedded rule configured over OAM interface in MG") and 
method 3 ("container-embedded rule download across a database interface to MG");

  - decision for single PSL not required, multiple PSLs may be principally supported;

  - parallel support of (1.1) and (1.2) is principally feasible;

  - parallel support of (1) and (2) or (3) is principally feasible;

  - technical goal for container-embedded rule: preferably a format which may be directly executed in processing logic of the policy enforcement path, i.e., trying to avoid transformation/conversion steps between rule formats;

  - the container principle may be also attractive, when supported by other, non-H.248 policy control protocols (e.g., COPS, Diameter);

  - the container principle may be in favour when trying to avoid the explicit specification of detailed rules in Q.3/16;

  - possible interactions in case of multiple rules (note: this is a general topic, independent of DPI rules), with the principle solution of rule ordering, precedence or priority assignment (as e.g. done in H.248.76);

  - others.

The introduced generic rule format from Figure III.1 may be further detailed by Figure III.4, see next.

Figure III.4 – A more detailed Policy Rule format, 
not all elements may be applicable to H.248 …

Some further observations (based on Figure III.4):

  - when specifying rules, it might be beneficial in providing separate syntax for conditions and actions, i.e., a syntactical element should not get any semantic, which would tightly couple a particular action to a condition;

  - thus separate syntax for conditions and actions, plus syntax for flexible combination of actions to conditions;

  - others

Appendix IV
Living List – Discussion of Policy Specification Language support

## IV.1	Discussion of Policy Specification Language Candidates

During the Research Triangle Park meeting (2010-12) progress was made in identifying different DPI scenarios. These scenarios provide a good set of requirements when considering the adoption of a policy specification method to be used by H.248 for DPI purposes.

H.248.DPI in clause I.3 currently contains a living list which highlights the possible mechanisms for signalling the policy rules to the MG. The options are:

1	Explicit rule signalling

1.1	Explicit rule in native H.248 syntax

1.2	Explicit rule encapsulated in container over H.248

2	Signalling of pointer to rule and rule MG-locally provisioned

3	Signalling of pointer to rule and rule MG-remotely provisioned

From an analysis of the DPI scenarios it can be determined that the breadth and complexity of the scenarios do not lend themselves to option "1.1 Explicit rule in native H.248 syntax". Using multiple properties and multiple Contexts/Termination would be problematic and overly complex.

This leaves options 1.2, 2 and 3 as a possible means to signal policy rules for the illustrated scenarios. In terms of signalling the Contributors believe that option 2 and 3 are essentially the same as they are simple pointers to an existing rule set. Therefore the choice is between carrying the rule set in a H.248 command or carrying a pointer to the rule set in a H.248 command.

In order to make this choice one must consider how the rule set is specified (i.e. what is the policy specification language (PSL)?) and what framework it is specified in. In order to determine an appropriate PSL the candidate PSLs must be compared to the scenario list to see if the functionality can be supported.

Where possible the Contributors favour an approach of trying to utilise an existing PSL for H.248.DPI rather than defining a new language. Considerable effort and expertise has already been invested in the specification of such languages and the Contributors believe that Q.3/16 would not necessarily be the appropriate place to specify a new PSL. Small additions or modifications may be needed in order to fit the PSL with H.248. Whilst the Contributors do not see the specification of a completely new PSL in H.248.DPI they do believe that preferentially a single PSL should be used by H.248.DPI. This would increase interoperability between implementations.

As mentioned by the H.248.DPI living list there are a number of existing PSLs that may be applicable for deep packet inspection, e.g. XML, Perl Regular Expression, POSIX regular expressions, JAVA script, etc. An analysis of these aspects shows that, whilst these could be utilised to create a filter to parse an input, none has been exclusively designed for the task of DPI.

There are other PSLs (both open source and proprietary) that are related to or exclusively designed for DPI. For an example of a proprietary system is Procera's DRDL (http://www.proceranetworks.com/products/drdl-technology.html). Many telecommunication vendors have their own PSLs for DPI. It is unlikely that a single vendor's implementation would be accepted thus adopting an open source approach has merit.

An example of an open source system is SNORT (http://www.snort.org/).

## IV.2	SNORT Analysis

On analysis of SNORT the we believe that it is an good candidate for the PSL for H.248.DPI. SNORT has many benefits:

- SNORT is open source and is already used in many products.

- There is a large base of existing rules that may be applied to address known vulnerabilities.

- There is a large address space for proprietary/custom rules (~1,000,000 IDs).

- There is an extensive community updating and developing new rules.

Thus we see greater value in the intellectual capital involved in the defined rule sets and community around them rather than the mechanics of the SNORT PSL itself.

However this is not to say that the SNORT PSL is itself insufficient. The SNORT PSL can be found in section 3 of the SNORT Manual (http://www.snort.org/assets/166/snort_manual.pdf ).

### IV.2.1	Signalling

SNORT is a text based PSL and thus lends itself to being potentially carried in H.248 commands as a string. It also defines a Snort Rule ID (sid) that allows a unique identification of each rule, which lends itself to being used as a pointer in a H.248 command. Thus SNORT would not prevent the signalling options 1.2, 2 and 3 (indicated above) from being supported.

### IV.2.2	Structure

Figure 1 / H.248.DPI outlines a generic high-level format of a policy rule, identifier/name part, condition part and action part.

The SNORT PSL provides several elements that enable clear identification and version handling. The Snort Rule (sid) as mentioned above allows a unique identification of SNORT rules. For version handling the Revision (rev) element can be used. The Reference (reference) element can provide further information regarding the definition of a rules. The Priority element (priority) may be assigned to the rule in order to prioritise one rule over another.

SNORT rules consist of a rule header and rule options. As indicated by the manual:

"Snort rules are divided into two logical sections, the rule header and the rule options. The rule header contains the rule's action, protocol, source and destination IP addresses and netmasks, and the source and destination ports information. The rule option section contains alert messages and information on which parts of the packet should be inspected to determine if the rule action should be taken."

So whilst the rule structure itself does not follow Figure 1 strictly it does allow conditions and actions to be specified.

### IV.2.3	Addressing

SNORT currently supports the TCP, UDP, ICMP and IP protocols. The IP address, port and source and destination (direction indicator) information is carried in the SNORT rule header. H.248 supports the sending of this information via Termination/Stream configuration and the Local and Remote Descriptors. Further investigation is needed to determine the mapping between SNORT and H.248.

### IV.2.4	Actions

SNORT defines a rich set of actions  (see section 3.2.1 / SNORT Manual), including:

- alert - generate an alert using the selected alert method, and then log the packet

- log - log the packet

- pass - ignore the packet

- activate - alert and then turn on another dynamic rule

- dynamic - remain idle until activated by an activate rule , then act as a log rule

- drop - block and log the packet

- reject - block the packet, log it, and then send a TCP reset if the protocol is TCP or an ICMP port  unreachable message if the protocol is UDP.

- sdrop - block the packet but do not log it.

Compared against the current example scenarios in H.248.DPI it can be seen that actions "discard packet", "silently discard packet" and "allow the packet to pass" are supported.

SNORT allows the logging and alerting of events. These have some similarity to H.248 Statistics and H.248 Event Reporting. Further investigation is needed to describe any potential mapping between SNORT and H.248 in this regard. One option may be the definition of H.248 specific rule types.

### IV.2.5	Conditions

SNORT defines many detection rule options. These are effectively grouped into three groups:

- Content Detection Rule Options

- Non-Payload Detection Rule Option

- Post-Detection Rule Options

The elements contained in these groups allow complex filtering behaviour to be specified.

H.248.DPI outlines a number of DPI scenarios. Below is a brief discussion of SNORT support:

Example VI: "Media type/media format policing"

The "content" element with associated content modifier allow would allow  a rule to be specified to look at the bits associated with the payload type field and to see whether they matched an allowed value/s.

Example VII: "Detect application X of an "application-agnostic" bearer"

The example in H.248.DPI indicates several different methods for determining applications. In the above example it can be seen that SNORT allows simple bit pattern searching. SNORT also allows complex regular expression matching utilising the "pcre" element (section 3.5.23 / SNORT Manual). Heuristic analysis is supported via multiple rules and dynamic rule specification. As alluded to in section IV.2 there is an active community defining rules for well known and emerging attack signatures.

Example VIII: "Detect application X = 'BITTORRRENT'"

SNORT has several elements that deal with searching and matching in HTTP bodies. I.e. "http_method" (section 3.5.13 / SNORT manual) and "http_client_body" (section 3.5.8 / SNORT manual).

Example IX: "Detect application X = 'SKYPE'"

SNORT allows a stateful based rule definition approach. It is possible that one rule triggers another. It is also possible to extract information via one rule and save it for use in another rule (i.e. through the use of the "byte_extract" (section 3.5.29 / SNORT Manual)).

Example X: "Detect application X = 'OPEN GAME PROTOCOL'"

SNORT may achieve this through simple pattern matching.

Example XI: "Detect application X = 'audio channel in a multi-channel media application'"

Whilst SNORT may allow the detection of the condition it is currently unclear whether it can support the removal of an audio frame within an SDU. SNORT does have the "replace" element that can be utilised for simple replacement of data (where length can be maintained). However removal of payload data has implications for checksums etc. Further investigation is needed.

Example XII: "Detect & measure application X = 'greek Jabber traffic'"

SNORT may achieve this through simple pattern matching.

Example XIII: "TCP attack detection"

SNORT allows the detection of TCP attacks. It has several elements dedicated to TCP checks, i.e. the elements "flags", "flow", "flowbits", "seq", "ack" and "window" from section 3.6 / SNORT Manual).

Example XIV: "Remove invalid MIME attachments from Instant Messaging"

As per example XI. SNORT support for selective removal of payload information is for further study.

Example XV: "RTCP Block Type Filtering"

As per example XI. SNORT support for selective removal of payload information is for further study.

Example XVI: "Application-specific traffic handling"

As discussed in section IV.2.4 above SNORT has several defined actions but also allows user defined actions. Further investigation would be needed into how H.248 actions would be triggered from a SNORT rule.

Example XVII: "Abnormal packet size detection"

The SNORT element "dsize" (section 3.6.7 / Snort Manual) allows the testing of packet payload size.

Example XVIII: "Delete old packets of application X"

The SNORT element "ttl" (section 3.6.2 / Snort Manual) allows the testing of the time to live for a packet.

## IV.3	Potential Issues

Section IV.2 highlights that technically SNORT is a good candidate PSL for H.248.DPI. However an issue is the status of the organisation responsible for open source SNORT PSL. The "Snort Community" open source effort is not recognised as an SDO by the ITU-T. I.e. it would not be possible to reference the work via A.5. This presents problems with normatively referencing their work. The company behind this effort is SourceFire and they are not a member of the ITU-T. There may be copyright/patent issues associated with this organisation. These issues would need to worked through.

## IV.4	Conclusions

Whilst SNORT is a strong candidate for a PSL for DPI unfortunately the issues in section IV.3 mean that it is not possible to utilise SNORT at this stage.

Appendix V
Living List – Relation to other ITU-T Recommendations on DPI

It might be beneficial to check potential relations with other work on DPI in ITU-T, for instance:

- SG13 Q.17 is working on framework and requirement related ITU-T Recommendations for DPI (= work items Y.dpifr and Y.dpireq).

Another objective should be the alignment of DPI related terminology.

__________________

= ASCII character string: "#!AMR_MC1.0\n" (see [IETF RFC 4867], the magic number for multi-channel AMR)

= 32 bit channel description field(see [IETF RFC 4867])