| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4255 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: Progress of H.248 package(s) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update clause 8, driven by editor’s notes.

List of proposed changes:

- move of terminology from H.248.TLSPROF to H.248.TLS), see also AVD-4258;

- update of TLS service negotiation package.

To H.248 package update:

History:

The original design (C-826; http://www.itu.int/md/T09-SG16-C-0826/en) did consider separate packages for basic capabilities (H.248 TLS capability package; tlsc) and negotiation specific aspects (H.248 TLS service negotiation control package; tlsn). Both packages were merged, primarily due to overlappings and redundant H.248 signalling elements.

The package design was also driven from MGC perspective due to multiple possible levels of involvement in TLS bearer service control, such as

- H.248 profiles without support of tlsc and tlsn: demanding for a pure MG autonomous behaviour concerning TLS service establishment. Such a scenario might be suitable in network environments with very limited TLS service support (e.g., remote TLS entities with unique TLS implementations).

- H.248 profiles with tlsc only support: the MGC could access to main TLS capabilities supported by a MG and used that information at the level of application control signalling. The MG would autonomously negotiate the TLS service with remote TLS entities, within the framework of supported TLS capabilities.

- H.248 profiles with support of both packages: the MGC has the chance in influencing the TLS service negotiation.

The different levels of MGC control options are still meaningful in our opinion. Though, the present contribution is firstly continuing with the single package approach.

Additional package elements proposed:

- H.248 events and signals for explicit support of the TLS handshake protocol.

It has to be noted that the work on the TLS package(s) is still not mature.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4232, the input document for this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP)

[ITU-T H.248.84]	Recommendation ITU-T H.248.84, Gateway control Protocol: NAT-traversal for peer-to-peer services

[ITU-T H.248.TCP]	Recommendation ITU-T H.248.TCP, TCP connection control package

# 3	Definitions

# 3	Definitions

{Editor’s note (2012-09): below terms are just moved from H.248.TLSPROF, without any modification.}

## 3.1	Terms defined elsewhere

{Editor’s note (2012-09): below terms are just moved from H.248.TLSPROF, without any modification.}

This Recommendation uses the following terms defined elsewhere:

3.1.1	TLS-client [IETF RFC 5246]: The application entity that initiates a TLS connection to a server.  This may or may not imply that the client initiated the underlying transport connection.  The primary operational difference between the server and client is that the server is generally authenticated, while the client is only optionally authenticated.

3.1.2	TLS-server [IETF RFC 5246]: The server is the application entity that responds to requests for connections from clients.  See also "client".

3.1.3	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): stateful forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a stateful TCP connection state machine.

NOTE 3 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	TLS Domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.2	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.3	TLS-profile: A selection of options from a set of TLS related parameters and procedures.

3.2.4	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.5	MG TLS profile: The TLS profile supported by the MG.

3.2.6	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

{Editor’s note (2012-09): the definition proposals in clause 3.2 are still under discussion .}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 8	TLS service negotiation control package

{Editor’s note (2012-04): the initially proposed two packages by C-826 were “merged” into a single package in this clause (“because the five properties of the “TLS capability package” are also part of the tlsn package” ). Still to do: 
- check/describe provisioning aspects,
- check descriptor level (TerminationState & LocalControl)
This package may be fundamentally used in scenarios

- MGC does not control bearer-path level TLS service negotiations, but MGC may audit supported TLS capabilities;

- MGC want to influence TLS service negotiations;

- others.

Contributions are solicited.}

|  | Package Name: | TLS service negotiation control package |
| --- | --- | --- |
|  | Package ID: | tlsn (0x####) |
|  | Description: | When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. This package provides control tools for the MGC for influencing bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. ### |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0001) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS-RFC (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the “0x”, and if needed, a leading “0” is added for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root |
|  | Characteristics: | Read/Write |

### 8.1.2	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0002) |
|  | Description: | This property indicates the cipher suites and their precedence for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sublist of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA. The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA will be coded into the first two characters, and the second value will be coded into the last two characters of the string. Thus each value in its hexadecimal representation will be converted into the double-hexdigit string. The “0x” will be omitted and if needed, a “0” is used for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). {Refer to http://www.iana.org/assignments/tls-parameters/tls-parameters.xml, other contributions are welcome. } |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root |
|  | Characteristics: | Read/Write |

### 8.1.3	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0003) |
|  | Description: | This property declares the list of compression methods and their preference for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root |
|  | Characteristics: | Read/Write |

### 8.1.4	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0004) |
|  | Description: | This property declares the support for the TLS session resumption in the MG |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported by the MG True Session Resumption shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root |
|  | Characteristics: | Read/Write |

### 8.1.5	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0005) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported by the MG |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported by the MG True Renegotiation shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.6	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0006) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. {Editor’s note: Do we need other criteria for a renegotiation trigger, like nbr fragments sent/rec., nbr octets sent/rec., etc?} |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.7	TLS-Endpoint Role

|  | Property Name: | TLS-Endpoint Role |
| --- | --- | --- |
|  | Property ID: | er (0x0007) |
|  | Description: | This property declares whether the MG shall act as a TLS-client or TLS-server. |
|  | Type: | Enumeration |
|  | Possible values: | “tcp-alike” (0x0000) The MG’s role shall depend on the TCP-role: If the MG acts as a TCP-client, then the MG shall act as a TLS-client as well. If the the MG acts as a TCP-server, then the MG shall act as a TLS-server aswell. “tls-client” (0x0001) The MG shall act as a TLS-client “tls-server” (0x0002) The MG shall act as a TLS-server |
|  | Default: | “tcp-alike”, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.8	TLS-Domain-Profile-Identifier

|  | Property Name: | TLS-Domain-Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0008) |
|  | Description: | This property identifies the TLS domain profile that shall be used by the MG to setup the TLS session |
|  | Type: | String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.9	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0009) |
|  | Description: | This property declares if the MG – if acting as a TLS-server – shall authenticate the client |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS-server, the client is not requested to be authenticated. True In case the MG acts as a TLS-server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.10	Autonomous bearer release indicator

|  | Property Name: | MG Autonomous Bearer Release Indicator |
| --- | --- | --- |
|  | Property ID: | abri (0x0010) |
|  | Description: | This property defines if a detected condition leading to the closure of the TLS connection will trigger an MG-autonomous release of the bearer connection of the related SEP as well. |
|  | Type: | Boolean |
|  | Possible values: | True On closure of the TLS connection the MG will autonomously release the bearer connection on the other related SEP as well. False The bearer connection of the other SEP is not affected by the closure of the TCP connection. |
|  | Default: | True |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	TLS session established

|  | Event Name: | TLS session established |
| --- | --- | --- |
|  | Event ID: | estab (0x0001) |
|  | Description: | This event indicates that the initial TLS-handshake has completed successfully and that application data may flow in both directions. This event is triggered when the Finish message has been sent and the Finish message has been received from the remote TLS-endpoint has been verified successfully. The event shall be set at stream level. |

#### 8.2.1.1	EventsDescriptor parameters

None

#### 8.2.1.2	ObservedEventsDescriptor parameters

#### None

### 8.2.2	TLS-Session Termination Alert

|  | Event Name: | TLS-Session Termination Alert |
| --- | --- | --- |
|  | Event ID: | sta (0x0002) |
|  | Description: | This event indicates the termination of the TLS session. It does not provide any statement regarding the state of the underlying TCP-connection. The event shall be set at stream level. |

#### 8.2.2.1	EventsDescriptor parameters

#### None

#### 8.2.2.2	ObservedEventsDescriptor parameters

##### 8.2.2.2.1	Session Termination Initiator

|  | Parameter Name: | Session Termination Initiator |
| --- | --- | --- |
|  | Parameter ID: | sti (0x0001) |
|  | Description: | Indicates which side initiated the termination of the TLS-session. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Session termination was initiated by the MG “remote” (0x0002) Session termination was initiated by the remote TLS-endpoint |
|  | Default: | NA |

##### 8.2.2.2.2	Session Termination Alert Description

|  | Parameter Name: | Session Termination Alert Description |
| --- | --- | --- |
|  | Parameter ID: | stad (0x0002) |
|  | Description: | Indicates the alert description for the termination of the TLS-session. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA |
|  | Default: | NA |

### 8.2.3	TLS-Session Warning Alert

|  | Event Name: | TLS-Session Warning Alert |
| --- | --- | --- |
|  | Event ID: | swa (0x0003) |
|  | Description: | This event indicates an abnormal condition for the TLS-layer that does not lead to the termination of the session. The abnormal condition may be detected on the local or on the remote TLS-endpoint. The event shall be set at stream level. |

#### 8.2.3.1	EventsDescriptor parameters

None

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### 8.2.3.2.1	Session Warning Initiator

|  | Parameter Name: | Session Warning Initiator |
| --- | --- | --- |
|  | Parameter ID: | swi (0x0001) |
|  | Description: | Indicates on which TLS-endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Abnormal condition detected by MG “remote” (0x0002) Abnormal condition detected by the remote TLS-endpoint |
|  | Default: | NA |

##### 8.2.3.2.2	Session Warning Alert Description

|  | Parameter Name: | Session Warning Alert Description |
| --- | --- | --- |
|  | Parameter ID: | swad (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA |
|  | Default: | NA |

### 8.2.4	Lower Layer Abort

|  | Event Name: | Lower layer abort |
| --- | --- | --- |
|  | Event ID: | lla (0x0004) |
|  | Description: | This event indicates that the layer below TLS has become unavailable, thus indicates the termination of the TLS session. The event shall be set at stream level. |

#### 8.2.3.1	EventsDescriptor parameters

None

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### None

## 8.3	Signals

8.3.1	Alert

|  | Signal Name: | Alert |
| --- | --- | --- |
|  | Signal ID: | al (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, or if the ad parameter is set to “close_notify(0)” the session will be terminated. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

8.3.1.1	Additional parameters

8.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

8.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

## 8.4	Statistics

### 8.4.1	Number of Received TLS-Fragments

|  | Statistic Name: | Number of Received TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recfrag (0x0001) |
|  | Description: | This statistics logs the number of TLS-fragments received by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.2	Number of Sent TLS-Fragments

|  | Statistic Name: | Number of Sent TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentfrag (0x0002) |
|  | Description: | This statistics logs the number of TLS-fragments sent by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.3	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0003) |
|  | Description: | This statistics logs the number of application data octets that are delivered to the layer above TLS. Received application data messages from the remote TLS endpoint are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.4	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0004) |
|  | Description: | This statistics logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 8.5	Error Codes

FIXTHIS …

## 8.6	Procedures

### 8.6.1	Determining TLS-Related Capabilities

The MGC can determine the properties of the MG TLS profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr).Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS-versions supported by the MG without any specific sequence.

The Supported Cipher Suite property may include the cipher suite TLS_NULL_WITH_NULL_NULL (“0000”), which is used as the initial cipher suite during the first handshake for the TLS-session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS-session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

{Editor’s note: TLS-resumption use cases to be identified.}

### 8.6.2	Mode of Operation

The instantiation of a TLS endpoint is indicated to the MG by the “m=” line in the Local and Remote Descriptor. The protocol being used is defined in [IETF RFC 4572] for the media agnostic case or in the media related specification (e.g. [IETF RFC 4975] for MSRP).

With the creation of a TLS endpoint in the MG the underlying TCP endpoint is created as well implicitly. The SDP attribute “setup” must be set to “actpass” in the Local and Remote Descriptor to indicate the TCP-proxy mode, refer to [ITU H.248.84] and to [ITU H.248.TCP].

### 8.6.3	TLS-Endpoint Role

The TLS-Endpoint Role (er) property controls whether the MG shall act as a TLS-client or a TLS-server. Usually per TLS-endpoint this role is coupled to role of the TCP-layer, however use cases might exist where the role in the TLS-layer and the TCP-layer may differ.

At creation time of a TLS endpoint in the MG the TCP connection setup role and the TLS-Endpoint Role (er) might not be defined yet, as normally the termination is created in the MG before the client/server role negotiation on the signalling plane is completed (offer/answer). In such a case the TCP connection setup role “notset” will be used (refer to [ITU-T H.248.TCP]) and the TLS-Endpoint Role (er) will be set to “tcp-alike”.

Once the TCP connection setup role is set to a value different from “notset” in the MG, the TCP connection setup will be triggered in the MG, and at completion the TLS session will be established autonomously in the MG according to the value of the property TLS-Endpoint Role (er).

### 8.6.4	TLS-Profile Handling

For different TLS domains different behaviours of the TLS implementation may be required. For each TLS domain those requirements are defined by a “TLS domain profile”. Generally the TLS domain profile must be provisioned on the MG. The property TLS-Domain-Profile Identifier (dpid) is used to refer to a TLS domain profile. This property must be aligned between the MG and the MGC, as the MGC will use it to indicate to the MG which TLS domain profile will be applicable for a creation of a TLS endpoint in the MG.

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as “MG TLS profile”.

On creation of a TLS endpoint in the MG the intersection of the applicable TLS domain profile and the MG TLS profile will be build. This intersection is called the TLS endpoint profile. It defines the TLS related parameter and procedures the MG will use to establish the TLS session for the endpoint.

Figure 1 illustrates the relation between the profiles.

Figure 1: Intersection of the TLS domain profile 1 and the MG TLS profile

8.6.4.1	TLS-profile handling: TLS version

The list of TLS versions used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. This list is supposed to be provisioned per dpid on the MG.

In case the MGC provides the property TLS Versions (tlsv), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG is acting as a TLS-client, the highest TLS versions as defined by the TLS-endpoint profile are included into the initial TLS-handshake message (ClientHello). In case the MG is acting as a TLS-server, the MG shall select the highest TLS-version that is present in the list of TLS-versions offered from the TLS-client and in the TLS Versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS-handshake is regarded as failed.

{Editor’s note: Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

8.6.4.2	TLS-profile handling: Cipher suites

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of cipher suites are provisioned per dpid on the MG.

In case the MGC provides the property Chipher Suites (cs), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS-client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS-server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS-version.

8.6.4.3	TLS-profile handling: Compression methods

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of compression methods are provisioned per dpid on the MG.

In case the MGC provides the property Compression Methods (cm), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle than the negotiation for the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS-version.

8.6.4.4	TLS-profile handling: Client authentication

The MG’s TLS server implementation must support the client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this setting is provisioned per dpid on the MG.

In case the MGC provides the property Client Authentication Required (car), the MG will use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

8.6.4.5	TLS-profile handling: Self-signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute a=fingerprint. If present in the Remote Descriptor, the client certificate will be self-signed and will be verified against the fingerprint.

8.6.4.6	TLS-profile handling: signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). This list of root certificates is supposed to be configured per dpid on the MG.

8.6.4.7	TLS-profile handling: Server authentication

In case the TLS endpoint in the MG is configured as a TLS server, the MG must provide the server certificate to the client as defined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). Different TLS domains may require different server certificates, because the list of accepted root certificates may differ between TLS domains.

8.6.4.8	TLS-profile handling: Session resumption

FIX THIS

8.6.4.9	TLS-profile handling: Renegotiation support

The TLS implementation in the MG must provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this value is provisioned per dpid on the MG.

In case the MGC provides the property Support of Renegotiation of the Security Context (srsc), the MG will use this value instead.

{Editor’s note: Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

The MG will negotiate the cipher suite according to the Cipher Suites (cs) property. In case the MG is acting as a TLS-client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined in the Cipher Suites property. In case the MG is acting as a TLS-server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS-version.

The negotiation of the compression methods follows the same principle than the negotiation for the cipher suite. The Compression Methods (cm) property provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS-version.

### 8.6.5	Completion of the initial TLS handshake

### The TLS handshake will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

### The initial TLS handshake is completed when the following both conditions are true:

### The TLS-Finish message has been sent, using the negotiated set of security parameters

### The TLS-Finish message has been received and has been successfully verified

### At this point in time TLS application data may be sent and received using the negotiated security parameter. The event TLS session established (estab) can be used to notify the MGC accordingly.

### The event estab does not provide any statement about the state of the other SEP of the context.

### 8.6.6	Security Context Renegotiation

The behaviour for the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, and the Renegotiation Period (rp) property. In case the TLS-endpoint profile disables renegotiation, the MG will never initiate a renegotiation. If receiving a renegotiation request from the remote TLS-endpoint, the MG will respond with a no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS-version.

If renegotiation shall be supported, the MG will accept and process a renegotiation request from the remote TLS-endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to 0, no renegotiation will be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS-handshake completed successfully. On expiration of the timer the MG will trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS-version. In case the MG receives a trigger for the renegotiation from the remote TLS-endpoint while this timer is running, the MG will cancel and restart this timer.

{Editor’s Note: Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.}

### 8.6.7	TLS-Session Resumption

FIXTHIS …

### 8.6.8	Statistics

FIXTHIS …

### 8.6.9	TLS-Session Termination

### The trigger for a TLS session termination might come from different sources:

### The TLS session is terminated by the remote TLS endpoint by sending a close_notify alert or a fatal alert;

### The underlying TCP connection is terminated

### A non-recoverable error condition has been detected in the TLS application, e.g. the authentication of a received TLS segment has failed;

### The MGC indicated the closure of the TLS session by the signal Alert;

### The release of the bearer connection of the other SEP triggers the release autonomously in the MG;

### In case the MG acts in an application aware mode, the application part in the MG may indicated the release of the TLS session;

### 8.6.9.1		TLS alert

### In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour on the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a close_notify was received the TLS endpoint will generate a close_notify on its own and the TLS session is closed. The MG will autonomously initiate the release of the TCP connection as well. If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). In this case the parameter Session Termination Initiator (sti) will be set to “remote” and the parameter Session Termination Alert Description (stad) will contain the value from the AlertDescription as received in the TLS alert message.

### In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.2		TCP connection closed

In case the underlying TCP connection is closed, e.g. due to a received TCP-FIN or TCP-RST the MG will autonomously complete the closure of the TCP session if needed.

If requested the MGC will be notified by the event Lower Layer Abort (lla).

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.3		Non recoverable error condition detected

### In case an error condition is detected locally in the TLS layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to “fatal” and the AlertDescription is provided accordingly.

If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). The parameters Session Termination Alert Description (stad) will contain the same values as sent to the remote TLS endpoint, and the parameter Session Termination Initiator (sti) will be set to “local”.

### In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.4		Signal alert received from MGC

In case the signal alert is received and the parameter Alert Level (al) is set to “fatal” or if the parameter Alert Description (ad) is set to “close_notify”, the TLS session closure is initiated by the MG by sending a TLS alert with the same parameter as received from the MGC.

The alert signal does not generate the notification TLS-Session Termination Alert (sta) in return.

Independent of the value of the property MG Autonomous Bearer Release Indicator (abri) the other SEP will not be affected by the alert signal. If desired the MGC needs to release the bearer connection on that SEP separately as well.

8.6.9.5		MG autonomous release initiated on related SEP

The TLS enabled SEP might receive the indication that the bearer connection of the related SEP is released. If such an indication is detected the termination of the TLS session is initiated by sending a TLS alert (close_notify) to the remote TLS endpoint. The closure of the underlying TCP connection is initiated by the MG autonomously.

The event TLS-Session Termination Alert (sta) is not reported to the MGC in this case, as the MGC has the option to be notified by the related SEP about the release of the bearer connection.

8.6.9.6		Release indication received from TLS application

### In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session will be closed autonomously by sending a TLS alert (close notify) and the underlying TCP connection is closed autonomously by the MG as well. The property MG Autonomous Bearer Release Indicator (abri) will have no effect, as it will depend on the application if the bearer connection of the related SEP needs to be released as well.

### {Editor’s Note: In this case the behaviour of the application may be controlled by a similar property than tlsn/abri. This would need to be clarified in the application related specification.}

### {Editor’s Note:

### Observation: There are a lot of commonalities to H.248.TCP:

### Bearer connection state “established” and “closed” are reported by event notifications

### MGC may trigger the release of the bearer connection by a signal (“close”)

### The property MG Autonomous Bearer Release Indicator (abri) allows a “MGC-strictly controlled” behaviour as well as a”MG-autonomous” behaviour. The property  is present in both packages, indeed this indicator is bearer type independent

### Release procedure differs only in minor protocol dependent details (TCP: half closure; TLS: “duplex” closure)

### Thus defining a bearer protocol independent package to control the bearer connection setup and release might be an option. Such a packet could be the base for other bearer connection protocols as well like SCTP, DTLS, etc. Such a package could simplify interworking functions between different bearer connection protocols.}

### 8.6.10	Example for the TLS profile concept

This section provides examples for the TLS profile concept.

Table 1 depicts an example for a TLS domain profile. The profile is partitioned into three parts: part one describes the TLS client related properties, part two describes the TLS server related properties, and the last part covers the properties which are common to both TLS server and TLS client. Additionally for some profile properties like the cipher suite the a distinction is made between items that must be supported and items that are optionally allowed to be supported.

| Scope | Property | Example |
| --- | --- | --- |
| Client related profile entities | Mandatory list of TLS versions | TLS1.1 |
|  | Optional list of TLS versions | TLS1.2 |
|  | Mandatory list of cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | Optional list of cipher suites | TLS_RSA_WITH_3DES_EDE_CBC_SHA |
|  | Mandatory list of compression methods | NULL |
|  | Optional list of compression methods | DEFLATE, LZS |
|  | Extensions |  |
| Server related profile entities | Mandatory list of TLS versions | TLS1.1, TLS1.2 |
|  | Optional list of TLS versions |  |
|  | Mandatory list of cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | Optional list of cipher suites |  |
|  | Mandatory list of compression methods | NULL |
|  | Optional list of compression methods | DEFLATE, LZS |
|  | Extensions |  |
| Common profile entities | List of root certificates |  |
|  | Client authentication required | yes |
|  | List of supported client certificate types | Signed, self-signed |
|  | Renegotiation support | yes |
|  | Resumption support | no |
|  | Extensions |  |

Table 1 –Example of a TLS domain profile

Table 2 depicts an example for a MG profile. It also shows which properties can be audited on the root termination by the MGC.

| Property | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlsn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlsn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlsn/cm |
| Client authentication required | yes | tlsn/car |
| Resumption support | no | tlsn/ssr |
| Extensions |  |  |

Table 2 –Example of a MG TLS profile

The following table 3 shows an example how the intersection of the TLS domain profile and the MG TLS profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. This table does not indicate  which properties can be provided by the MGC.

| Property | TLS Domain profile | MG TLS profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B Provisioned per TLS domain profile |
| MG certificate |  | List of certificates, one certificate per TLS domain profile | Provisioned per TLS domain profile |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |

Table 3 –Example of an intersection between a TLS domain profile and a MG TLS profile

{Editor’s Note: A clear view on the MG certificate is still missing. It is used during the TLS handshake, thus it is a part of the “set of TLS related parameter and procedures” (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the MG TLS profile. The MG certificate is network element specific and TLS-domain specific.}

FIXTHIS …

...

[End Proposed Correction]

____________________