| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4260 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLSPROF: Framework and layout of this Recommendation (clause 1) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update scope and summary sections.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4233, the input document for this meeting.

Change proposal #1:

[Begin Proposed Correction]

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

This Recommendation provides guidelines on the use of secured TCP bearer traffic according the Transport Layer Security (TLS) technology in H.248 profiles.  These guidelines may be used by other Standards Developing Organizations (SDOs) when defining their H.248.1 profiles in support of TLS.

# 1	Scope

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two TCP endpoints. This Recommendation

- describes example use cases;

- defines the basic requirements to secure the bearer path connection between an MG and a remote endpoint through TLS; and

- references suitable H.248 signalling capabilities in terms of H.248 packages as defined by other H.248.x-series Recommendations and

- defines a protocol solution in style of a generic profile (which covers usage information of  package(s) and procedures) as a guideline for final profile specifications.

Scope and purpose of this Recommendation may be illustrated (Figure 1):

Figure 1 – Scope, structuring principle and framework of this Recommendation

The primary audience of this Recommendation are therefore authors of H.248 profile specifications, which aim to support a particular network use case with TLS encrypted bearer traffic.

This Recommendation is organized as follows:

- examples use cases are collected in Appendix II of [ITU-T H.248.TLS];

- principal requirements are subject of clauses  6 to 8, categorized in capabilities with regards to the mode of operation of a MG, TLS profile concepts and TLS protocol specific requirements;

and

- profile specification guidelines in clause 9 (which follows the profile structuring of the profile template according to [ITU-T H.248.1]).

[End Proposed Correction]

____________________