| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4261 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal for clause 1 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clause 1.

Discussion

There are two electronic comments in clause 1, pending from the last meeting. It looks like that the second one was just a copy & pasted. The proposal tries to resolve the first comment.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4234, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 1	Scope

The scope of this Recommendation covers performance monitoring in RTP networks with the involvement of H.248 entities. Performance monitoring as such is a network service across multiple network entities which are located in user, control and/or management planes. The scope of this Recommendation is limited to the user/control/management plane interfaces of H.248 entities. The notion of performance monitoring is used to emphasize a call/session dependent service, which is basically in contrast to the legacy performance management capability, which is a pure management plane function [ITU-T M.3010, ITU-T M.60].

Performance monitoring could be considered as a network overlay function.

This Recommendation provides (and other aspects):

- an overview of the existing “entire H.248 toolkit” for RTCP extension report (XR) support, i.e., the set of available H.248 packages in that area;

- an overview of available H.248 packages for non-XR related performance metrics, typically subject of RTCP basic report (SR, RR) capabilities;

- exemplary network use cases with different H.248 MG types (primarily physical-to-RTP gateways, media-agnostic IP-to-IP gateways, RTP-topology-aware RTP-to-RTP gateways);

- collection of functional requirements in order to motivate associated profile guidelines;

- classification of performance metrics (such as application-level versus transport-level metrics) in order to indicate applicability or not for H.248 MGs;

- principal H.248 MG tasks of

  - measurements of RTP traffic,

  - reporting of measurement data via RTCP XR reports,

  - collection of RTCP XR carried measurement data,

  - loopback of RTP (in order to relocate a measurement point),

  - filtering of RTCP XR reports,

  - modification of RTCP XR reports and/or

  - reporting of measurement data via H.248 statistics to the MGC

- control of such tasks via H.248 that could be addressed in a H.248 profile, i.e.

  - the identification of appropriate H.248 packages

  - the detailed specification of package usage

  - the example call-dependent procedures

- the association to other related H.248.x Recommendations such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.LOOPB] and [ITU-T H.248.RTPTOPO]

- control of measurement methods (such as interval, cumulative and alert type of measurements)

- usage of H.248.47-based conditional reporting [ITU-T H.248.47]

The Recommendation also discusses the purpose of performance monitoring (and therefore possible motivation of a use case). For example:

- capture of Grade-of-Service (GoS) related data in order to evaluate network conditions;

- capture of Quality-of-Service (QoS) and Quality-of-Experience (QoE) related data in order to evaluate service conditions (e.g. as part of service level agreements);

- capture of billing/charging related data;

- capture of data as input for routing algorithms;

- capture of data as input for network node selection algorithms;

- capture of data as input for centralized servers as support of network operations;

- capture of data as part of online tests;

- etc.

## 1.1	Framework and layout of this Recommendation

Scope and purposes may be summarized by a diagram: Figure 1 illustrates the framework of this Recommendation.

Figure 1 – Scope, structuring principle and framework of this Recommendation

The primary audience of this Recommendation are authors of H.248 profile specifications, which aim to support a particular network performance monitoring use case, and the consideration of a specific location and/or type of an H.248 MG.

Performance monitoring support could basically affect three areas of an H.248 profile:

- basic protocol capabilities (such as connection model);

- packages and their detailed usage; and/or

- call-dependent procedures.

This Recommendation provides guidelines for these profile areas. Typically to define a H.248 profile there are several steps that are followed, according to Figure 1, given by

- requirements stage: firstly selecting use case(s) and the identification of required MG behaviour in terms of measurements, reporting, filtering, etc. capabilities;

- protocol stage: secondly, addressing the necessary templates of the indicated profile areas and recommended input; and

- profile stage: thirdly, incorporating “guidelines” in an actual profile specification and adapting the templates to the concrete specification setting.

This Recommendation is organized as follows:

- examples use cases are collected in Appendix I;

- principal MG behaviour is defined in clause 8 by considering the set of requirements behind such MG roles (and the used prescriptive language for requirements is outlined in clause 5);

- an technology overview of existing H.248 packages is in scope of clause 6;

and finally

- guidelines for the three profile areas are subject of clause 8 (which follows the profile structuring of the profile template according to [ITU-T H.248.1]).

[End Proposed Correction]

____________________

“Not sure this text is needed. I guess all this is trying to say is that Sr/RR were defined before the XR work.“