| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4262 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal for clause 3.2 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clause 3.2 and a new Appendix.

Discussion and conclusions

This contribution addresses two disjoint kind of editor’s notes:

- Missing definitions for filtering and loopback points

- Discussion of application and transport level performance metrics (see also AVD-4264), as well as the request for concrete examples

To 1:

- Initial definitions are proposed, based on H.248.76 and H.248.LOOPB

- see below

To 2:

- a new Appendix is proposed as a central place for example performance metrics

- an initial list of concrete examples is added to that Appendix

- relation to H.248.58 may be also clarified (see also AVD-4264)

- note: the protocol stack based classification scheme of clause 7, and their associated terminology in clause 3.2.1 and 3.2.6, should not be overstressed; for instance, a detailed classification (as now used in new Appendix III) might be helpful when measurement cost factors would be developed based on information elements as outlined in Figure 2, but a coarse classification might be already sufficient for the requirements discussion in clause 8.

Attachment to terminology discussion:

Figure A reflects our understanding of (ITU-T) terminology.

Figure A – Packet handling with loopback path options

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4234, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

## 3.2	Terms defined in this Recommendation

3.2.1	application level metrics: a protocol stack based classification of a performance metric. Such a performance metric uses protocol information elements from the application layer and possibly from lower layer protocols (NOTE 1). Some application level metrics are furthermore coupled to the termination of the application protocol (NOTE 2) and/or the location of user equipment (NOTE 3).

NOTE 1 - Example: an H.248 IP-IP MG would be typically configured for a “media-aware” mode in order to have sufficient information about the carried application traffic with regards to the application level performance metric.

NOTE 2 - The termination of the application protocol correlates typically with RTP topology “RTP endsystem”.

NOTE 3 - Example: the category of “QoE” based performance metrics as application level metrics.

3.2.2	collection point (CP) (adapted from clause 7.6.2/[ITU-T H.248.71]): An CP is the physical or logical point which records/collects remote measurement data, as received via a user plane reporting interface (here via RTCP report packets).

3.2.3	filtering point (FP) (adapted from filter definition of [ITU-T H.248.76]): Location of policy rule enforcement with purpose on filtering information. Such specific policy rules are often simply termed as filter rules. In the context of this Recommendation,

- the filter rule conditions would be primarily related to protocol control information elements related to RTP and RTCP header fields (e.g., payload/packet type, block type), but may also be based on carried measurement data; and

- the filter rule actions would generally cover forwarding, blocking, counting or modification of data units at the levels of RTP/RTCP packets, reports down to individual measurement data.

NOTE – Specification and syntax of such policy rules are out of scope of this Recommendation.

3.2.4	loopback point (LP) (adapted from [Draft ITU-T H.248.LOOPB]): Location of a loopback function. In the context of this Recommendation, a loopback function comprises a copy and forward operation at the level of protocol data information, with primary scope on “IETF Media Loopback” (according clause 3.2.3/[ITU-T H.248.LOOPB]).

NOTE 1 – A loopback function is different in comparison to a redirect function, which does not copy any protocol data.

NOTE 2 – The loopback function is called “loopback mirror” in [b-IETF LOOPBACK].

{Contributor’s note (2012-09 meeting; the “H.248 Media Loopback” does relate to a redirect function only. There might be also use cases, in the context of performance monitoring, for such a loopback function. E.g., for offline test procedures. Hm? The primary scope is the IETF media loopback  …}

3.2.5	Subject to profile specification: When used this term indicates that the H.248 profile template section requires further specification by a profile specification.

3.2.6	transport level metrics: a protocol stack based classification of a performance metric. Such a performance metric could be realized solely on protocol information elements from the transport layer and lower layer protocols.

NOTE 1 - Example: an H.248 IP-IP MG in “media-agnostic, transport-aware” mode could still support measurements for transport level metrics

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix III

Example performance metric types

(This appendix does not form an integral part of this Recommendation)

## III.1	Background

Possible classification scheme for performance metrics for RTP traffic are described in clause 7. The protocol stack based method uses the terms application and transport level metrics (see clauses 3.2.1 and 3.2.6). This appendix provides some typical examples for each category.

There are examples which may be only MG-locally generated and reported via H.248, metrics which may be reported via RTCP, and examples for both reporting options.

## III.2	Transport level metrics

## III.2.1	RTP-agnostic transport level metrics

Example metrics:

- all four statistics according [ITU-T H.248.61] concerning octet and packet based IP traffic volume (for RTP sessions)

H.248 Network package:

- statistics os and or, see clause E.11.4/[ITU-T H.248.1]

### III.2.2	RTP-aware transport level metrics

That kind of transport level metrics are RTP-aware due to consideration of RTP header information TS, SN and/or RTCP data.

Example metrics:

- jitter buffer nominal delay [b-IETF rtcp-xr-jb]

- jitter buffer maximum delay [b-IETF rtcp-xr-jb]

- jitter buffer high water mark and jitter buffer low water mark [b-IETF rtcp-xr-jb]

- Sum of Burst Durations [b-IETF rtcp-xr- burst-gap-loss]

- Packets lost in bursts [b-IETF rtcp-xr- burst-gap-loss]

- … [b-IETF rtcp-xr- burst-gap-loss]

- Sum of Squares of Burst Durations [b-IETF rtcp-xr- burst-gap-loss]

- round trip delay [IETF RFC 3611]

H.248 RTP package:

- all five statistics (ps, pr, pl, jit and delay), see clause E.12.4/[ITU-T H.248.1]

H.248 Received RTCP package:

- all five statistics (rps, ros, rpl, rcpl and rjit), see clause 7.4/[ITU-T H.248.71]

## III.3	Application level metrics

### III.3.1	RTP-agnostic application level metrics

Example metrics:

These examples are based on the pure application data stream at receiver side (to be cross-checked again):

- Unimpaired seconds [b-IETF rtcp-xr-concsec]

- Concealed seconds [b-IETF rtcp-xr-concsec]

- Severely Concealed Seconds seconds [b-IETF rtcp-xr-concsec]

- Unimpaired seconds [b-IETF rtcp-xr-concsec]

Following example relates to fax-over-IP using [ITU-T T.38] in T.38-over-RTP transport mode and an H.248 IP termination enabled with the IP FAX package [ITU-T H.248.2]:

- Pages Transferred defined by H.248 statistic ipfax/pagestrans; that’s an application level metric due to data object ‘page’, and RTP-agnostic due to independence of RTP header information

Statistics of RTP application data package [ITU-T H.248.58] (again due to RTP payload relation only (“application data”), but independence of RTP header information):

- RTP payload octets sent and

- RTP payload octets received

### III.3.2	RTP-aware application level metrics

The measurement function must be here media format aware, based on the RTP payload type codepoint, in contrast to the examples of clause III.3.1.

Example metrics:

- MOS value (attributed by MOS type and calculation method) [b-IETF rtcp-xr-qoe]

- signal level [IETF RFC 3611]

- noise level [IETF RFC 3611]

- residual echo return loss [IETF RFC 3611]

- R factor [IETF RFC 3611]

- external R factor [IETF RFC 3611]

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.xnq]	Draft Recommendation ITU-T H.248.xnq (2005), Gateway control protocol: Extended Network Quality Metrics Packages for Next Generation Networks (NGNs).
Contained in document AVD-2773:
WP2/16 Rapporteur Meeting (Geneva, 28 November - 2 December 2005), 
http://wftp3.itu.int/av-arch/avc-site/2005-2008/0511_Gen/AVD-2773.zip

[b-ITU-T H.323]	Recommendation ITU-T H.323 (2009), Packet-based multimedia communications systems.

[b-IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[b-IETF RFC 3435]	IETF RFC 3435 (2003), Media Gateway Control Protocol (MGCP) Version 1.0.

[b-IETF rtcp-xr-qoe]	IETF draft-ietf-xrblock-rtcp-xr-qoe (201X), RTCP XR Blocks for QoE Metric Reporting.

[b-IETF rtcp-xr-jb]	IETF draft-ietf-xrblock-rtcp-xr-jb (201X), RTCP XR Report Block for Jitter Buffer Metric Reporting.

[b-IETF rtcp-xr- burst-gap-loss]	IETF draft-ietf-xrblock-rtcp-xr-burst-gap-loss (201X), RTCP XR Report Block for Burst/Gap Loss metric Reporting.

[b-IETF rtcp-xr- concsec]	IETF draft-ietf-xrblock-rtcp-xr-concsec (201X), RTCP XR Report Block for Concealed Seconds metric Reporting.

[b-IETF rtcp-xr- burst-gap-loss]	IETF draft-ietf-xrblock-rtcp-xr-burst-gap-loss (201X), RTCP XR Report Block for Burst/Gap Loss metric Reporting.

[End Proposed Correction]

____________________