| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4265 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Progress of clause 8 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clause 8.

Discussion

There are a couple of pending editors notes in clause 8, and there are also some subclauses with just some initial text. This contribution continues the list of requirements.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4234, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 8	Roles provided by MG

The overall network service of performance monitoring may be structured in a set of capabilities, which could be individually or assigned in combinations to an H.248 MG. The various roles of a MG may be illustrated by a gateway model concerning the processing of incoming RTP and RTCP packets (see Figure 3). The purpose of this clause is the characterization of such roles and the specification of associated requirements (see clauses 8.1ff).

Figure 3 – RTCP-based performance measurements – H.248 IP-to-IP border gateway/router – Unidirectional, ingress only RTP/RTCP processing model

The following separate functions are identified for an MG with an (IP, IP) connection model (Figure 3):

- Generation of local measurements (MPL);

- Reporting of local measurements (RPL,H.248) via H.248 towards MGC;

- Reporting of local measurements (RPL,RTCP) via RTCP towards remote terminals, remote H.248 MGs, remote RTP endsystems, or other RTP entities;

- Collection of remote measurements (CPR,H.248);

- Reporting of remote measurements (RPR,H.248) towards MGC;

- Loopback of incoming RTP/RTCP traffic (LPRTP) back to a remote RTP endsystem [purpose: the remote RTP endsystem may then provide a local measurement point of a remote sink, i.e., loopback allows to relocated MPs];

- Filtering of remote measurements (FPOut) in an outgoing direction, from provider A towards B (which are transported via RTCP reports); and

- Filtering of remote measurements (FPIn) in ingress direction, from provider B (which are transport via RTCP reports).

## 8.1	H.248 MG as general measurement point (MP)

MG capability related requirement:

R-8.1/1:	The MG is required to support local performance measurements.

Technology-dependent requirements related to metric types which are particularly defined for RTP traffic:

NOTE 1 - Subsequent requirements consider “grouped performance metrics” at the top-level of H.248 packages or IETF RFC documents.

R-8.1/2:	The MG is required to execute measurements for the performance metrics as defined by the RTP package [ITU-T H.248.1].

R-8.1/3:	The MG is required to execute measurements for the performance metrics as defined by the RTP application data package [ITU-T H.248.58].

R-8.1/4:	The MG is required to execute measurements for the performance metrics as defined by the Received RTCP package [ITU-T H.248.71].

R-8.1/5:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block types 1 to 7 [IETF RFC 3611].

NOTE 2 – There are two reporting options via H.248 (RPH.248): [ITU-T H.248.30] or [ITU-T H.248.48], see clause 8.5.

R-8.1/6:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block type 8 [IETF RFC 5093].

R-8.1/7:	The MG is required to execute measurements for the performance metrics as defined by other RTCP XR block types.

## 8.2	H.248 MG as location-dependent measurement point (MP)

There are two location specific aspects:

- a performance metric itself could be associated to the source, destination or interim node of an end-to-end RTP session (such as RTP packet rate at RTP sender side, as audio signal level at RTP receiver side or RTP packet delay variation observed at an interim RTP mixer); and

- the particular location of an H.248 MG within the (RTP) IP network topology.

### 8.2.1	Location-independent requirements

Some principal requirement could be derived:

R-8.2.1/1:	The MG is required to support at least transport level measurements.

NOTE 1 – This is the very minimum requirement, applicable for almost all MG scenarios (related to RTP topologies, related to metric types according clause 3.2.6).

R-8.2.1/2:	The MG is required to support transport level metrics, independent of the supported H.248 connection model (“all IP” or “IP-to-non-IP”) and independent of RTP topologies.

### 8.2.2	Location and metric type specific restrictions

Some performance metrics are only applicable for particular locations in a network topology. Especially application level metrics are tied to the source/sink of application level traffic, the location of the final consumer of the service (e.g., the type of metrics related quality of experience).

R-8.2.2/1:	The MG is prohibited to support application level metrics in such a scenario due to possible violation of the metrics semantic.

NOTE 1 – This requirement is consistent with [IETF draft monarch], see clause 3.2 “Location of RTP Monitors”.

R-8.2.2/2:	The MG is required to notify the MGC of such a violation, if enforced by the MGC for such kind of measurements, by e.g. an error code.

## 8.3	H.248 MG as collection point (CP)

The collection function is based on incoming RTCP data.

R-8.3/1:	The MG is required to collect remote measurement data via incoming RTCP reports.

R-8.3/2:	The MG is required to identify remote RTP nodes (via the H.248 RTCP source description package according [ITU-T H.248.71]) in the bearer path.

## 8.4	H.248 MG as reporting point at IP bearer interface (RPRTCP)

This interface is used for the exchange of measurement data and information for the production of measurements via RTCP reports.

R-8.4/1:	The MG is required to send RTCP basic reports (SR and/or RR)..

R-8.4/2:	The MG is required to receive RTCP basic reports (SR and/or RR).

R-8.4/3:	The MG is recommended to send RTCP extension reports (XR; NOTE 1).

R-8.4/4:	The MG is recommended to receive RTCP extension reports (XR; NOTE 1).

NOTE 1 - This requirement could be further detailed to the block type (BT) level or below the BT level down to individual metrics.

## 8.5	H.248 MG as reporting point at H.248 interface (RPH.248)

This reporting capability is tightly coupled to the H.248 signalling capability according individual H.248 packages:

R-8.5/1:	The MG is required to support H.248 statistics according the RTP package [ITU-T H.248.1].

R-8.5/2:	The MG is required to support H.248 statistics according the RTCP extended performance metrics package [ITU-T H.248.30].

R-8.5/3:	The MG is required to support H.248 statistics according the RTCP XR Block Reporting package [ITU-T H.248.48].

R-8.5/4:	The MG is required to support H.248 statistics according the RTP application data package [ITU-T H.248.58].

R-8.5/5:	The MG is required to support H.248 statistics according the Received RTCP package [ITU-T H.248.71].

## 8.6	H.248 MG as filtering point (for RTCP traffic) (FP)

Example scenarios with filtering support are illustrated in Appendix I, see use cases 2, 3, 4, 6 and 7. The specific filter capabilities are basically given by the particular filter rule conditions and filter rule actions (see clause 3.2.3). The level of conditions and actions is useful for the derivation of filter requirements.

Identification requirements, hierarchically ordered:

R-8.6/1:	The MG is required to support filter rule conditions related to the identification of RTP/RTCP traffic as such (e.g., related to the usual 5-tuple for an RTCP control flow (under the condition of separate IP transport connections for RTP and RTCP)).

R-8.6/2:	The MG is required to support filter rule conditions related to the identification of RTCP packets based on the RTCP packet type (PT) codepoint.

R-8.6/3:	The MG is required to support filter rule conditions related to the identification of RTCP report block structures based on the RTCP block type (BT) codepoint.

R-8.6/4:	The MG is required to support filter rule conditions related to the identification of a individual metric within an RTCP report block, based on the report block structure.

Action execution requirements:

a) action type “forwarding”:

R-8.6/5:	The MG is required to support unmodified forwarding of information entities according requirements R-8.6/1-4, as default action.

b) action type “blocking”:

R-8.6/6:	The MG is required to support blocking of the entire RTCP traffic.

NOTE 1 – This action may be subject of “abnormal traffic handling”, but may violate end-to-end RTCP protocol applications.

R-8.6/7:	The MG is required to support blocking of specific RTCP packet types.

NOTE 2 – This action may relate to the discard of an entire RTCP packet or the removal of an individual RTCP packet unit in case of a compound RTCP packet.

R-8.6/8:	The MG is required to support blocking of specific RTCP block types.

NOTE 3 – The identified RTCP block would be entirely removed from an RTCP XR packet.

c) action type “modification”:

R-8.6/9:	The MG is required to support the modification of an individual measurement value, i.e., the carried value of a specific metric type in an RTCP packet. Any concrete modification operation (such as a default value setting, value resetting, setting of a specific value) is out of scope of this Recommendation.

NOTE 4 – Such an action is typically requested by a network operator, which do not want to disclose his own measurement results to peering partners.

All above actions should be executed in a protocol correct manner, i.e., any modified RTP/L4/IP packet should be afterwards syntactically correct, which may imply the adaptation of length, padding and/or checksum information.

## 8.7	H.248 MG as loopback point (for RTP/RTCP traffic) (LP)

There are multiple loopback options:

R-8.7/1:	The MG is required to support a LP in "native H.248" mode, i.e. due to a StreamMode property value LoopBack setting.

R-8.7/2:	The MG is required to support a LP according [b-IETF LOOPBACK] in "encapsulated packet loopback" mode.

R-8.7/3:	The MG is required to support a LP according [b-IETF LOOPBACK] in "direct loopback" mode.

R-8.7/4:	The MG is required to support a LP according [b-IETF LOOPBACK] in "media loopback" mode.

## 8.8	Support of specific RTP topologies

The relation between RTCP services and RTP topologies in H.248 entities is described in [ITU-T H.248.RTPTOPO]. Performance monitoring based on RTCP XR belongs to supplementary RTCP services which are basically an overlay function and hence independent of RTP topologies, whereas performance monitoring using RTCP basic reports (such as SR, RR) would depend on RTP topologies.

R-8.9/1:	The MG is required to support ###.

[End Proposed Correction]

____________________