| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4267 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.82: Discussion of IETF document on "Reactions to Signaling from ECN Support for RTP/RTCP" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses IETF draft: “Reactions to Signaling from ECN Support for RTP/RTCP”, related to congestion controls on top of RFC 6679 ECN for RTP-over-UDP.

Discussion

After completion of RFC 6679 by IETF AVTCORE, work on complementary congestion controls is proceeded by WG TSVWG, such as

IETF draft: “Reactions to Signaling from ECN Support for RTP/RTCP”
<draft-carlberg-tsvwg-ecn-reactions-02.txt>

The work raises the question whether there would be any impact on H.248.82 or not? There are basically two aspects:

- Consideration of ETS calls (which is also subject of Appendix II/H.248.82)?

- “Congestion reaction” by H.248 MGs, i.e., when a MG would have to support a sender-driven or receiver driven congestion control, and if there would be information exchanged between the MGC and the MG embedded congestion control (e.g., due to configuration or reporting purposes)?

Figure 1 tries to capture the scope of this IETF document. There are several items evaluated.

Figure 1 – Scope of draft-carlberg-tsvwg-ecn-reactions-02.txt

It is not necessary to discuss the various items in detail in this contribution, we may rather focus on above two mentioned questions.

With regards to 1 – ETS calls:

- relates to clause 5.3 in [draft-carlberg-tsvwg-ecn-reactions-02.txt] and

- relates to clause II.2.3 in [Draft H.248.82]

- both clauses are inline concerning the assumption of ECN-disabled ETS calls (“ECN exemption”)

- draft-carlberg consider further a more fine granular “level of exemption (not just an on/off mechanism)

- draft-carlberg provides some high level quantitative estimations based on simulations, whereby Appendix II/H.248.82 makes only qualitative statements

- both documents are apparently inline concerning their principal propositions

Conclusion: an informative link could be added to Appendix II, see below proposal.

With regards to 2 – MGC-controlled congestion reaction behaviour:

- this subject is so far out of scope of H.248.82

- the configuration of specific congestion behaviour might be in responsibility of management plane functions (and then out of scope of H.248)

- however, it might be not excluded that there are also use cases for MGC-controlled behaviour in future

- but it is too early at this stage, just due to the premature status of congestion control discussions for RTP traffic (“and there are a couple of proposals around”)

Conclusion: not any action; if at all, then a possible future work item for a revision of H.248.82.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4239, the input draft to this Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

### II.2.3	Example 3: Network with "sparse ETS traffic"

A network situation with a low portion of ETS traffic, e.g., when “ETS traffic is much less than non-ETS traffic”.

Evaluation:

The ECN-disabled ETS calls could benefit from the ECN-enabled non-ETS calls due to a potential positive effect of more available transport capacity from non-ETS traffic and a reduced overall congestion level (because the congestion control could reduce the consumed bitrate by non-ETS calls).

NOTE – This assessment is inline with the conclusion outlined by [b-IETF ecn-reactions], clause 5.3 on “Alternative Reaction for Emergency Communications”.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Bibliography

[b-IETF ecn-reactions]	IETF draft-carlberg-tsvwg-ecn-reactions (2012), Reactions to Signaling from ECN Support for RTP/RTCP.

[End Proposed Correction]

____________________