| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4272 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.1 Media Descriptor versus RTP multiplexing models – Some initial observations |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

The document provides a discussion of multiplexing models for RTP traffic. Such extended IP protocol stacks are in scope of H.248 MGs in case of some emerging services (like telepresence or WebRTC) and need to then described in H.248 signalling as part of the media descriptor. It is yet unclear whether existing H.248 protocol means would be sufficient, or if syntactical or/and semantical changes would be required.

The topic as such is rather complex due to the number of aspects and solution options. This contribution is therefore just an initial evaluation, rather already a comprehensive study. We could imagine that a dedicated work item might be finally beneficial for documentation purposes.

Content:

1	Introduction	3

1.1	Important references	3

1.2	Important terminology	4

2	Discussion	7

2.1	Motivation for RTP multiplexing: Telepresence, WebRTC & Conferencing	7

2.2	Identifier spaces (in user, control and application plane)	8

2.3	RTP protocol stack identifier space and basic levels of multiplexing	9

2.4	WebRTC media configurations: conventions used	10

2.5	Multiplexing levels and major media configuration options (WebRTC)	11

2.5.1	Scope and terminology	11

2.5.2	Use case #1: fully multiplexed by both local and remote endpoints	13

2.5.3	Use case #2: local = fully unmultiplexed, remote = fully multiplexed	14

2.6	SDP control elements for describing (WebRTC) media configurations	15

2.7	Mapping of multiplexed, multimedia media configurations from SIP to H.248	16

2.7.1	Problem statement	16

2.7.2	Mapping option I: “Single-Stream, Bundle-aware, SDP described”	17

2.7.2	Mapping option II: “Multi-stream, Bundle-agnostic, TerminationState-Property controlled”	17

2.7.3	Mapping option III: “Multi-stream, Bundle-agnostic, MGC specified LD transport addresses”	17

2.7.4	Mapping option IV: “Single-stream, Bundle-agnostic, ReserveGroup Property controlled”	17

2.7.5	Mapping option V: “Multi-stream, Bundle-agnostic, MuxDescriptor controlled”	17

2.7.6	Mapping option VI: others	17

3	Conclusions	18

Annex A – Example signalling	18

A.1	Example SIP-embedded SDP Offer for WebRTC	19

A.2	Use case #I: MG local = fully unmultiplexed, MG remote = fully multiplexed	20

A.3	Use case #II: MG local = RTP/RTCP multiplexed, MG remote = fully multiplexed	22

A.4	Use case #III: MG local = fully multiplexed, MG remote = fully multiplexed	24

Attachment – Useful terminology from Draft Y.2770 on DPI	26

# 1	Introduction

Multiplexing models for RTP isn’t a new thing, rather already indicated in the first RFC on RTP in 1996 (RFC 1889). A number of multiplexing options was discussed in the last 15 years, however, the very majority of RTP applications didn’t apply “multiplexing”. The native (unmultiplexed) mode could be regarded as a single IP transport connection for a single media type, e.g. video traffic over a single UDP connection, characterized as a single RTP session.

Multiplexing models in H.248 are an inherent protocol capability since day one, given by the hierarchy of flow/stream/termination concept for bearer connection endpoints or the H.248 Multiplex Descriptor.

And there were (in 2008) also additional multiplexing options discussed for support of multiplexed RTP, such as by AVD-3360 for a new draft H.248.RM “RTP Multiplexing Package” (http://wftp3.itu.int/av-arch/avc-site/2005-2008/0801_Seo/AVD-3360.zip), here in the context of the 3GPP CS domain.

Recent activities for resuming the design on RTP multiplexing options are driven by new services (e.g., [4, 5]) which are fairly extending native multimedia models, as well as by the ultimate goal in minimizing the complexity of NAT traversal solutions.

## 1.1	Important references

It isn’t the purpose of this document in recapitulate all the ongoing technology discussions on RTP multiplexing, the reader should be rather familiar with (Status: 2012-09) following IETF documents (list might be not exhaustive) from four IETF working groups:

[1]	Guidelines for using the Multiplexing Features of RTP
IETF AVTCORE
Draft: draft-westerlund-avtcore-multiplex-architecture 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multiplex-architecture-02.txt

[2]	Multiplexing RTP Data and Control Packets on a Single Port
IETF MMUSIC
RFC 5761

[3]	“BUNDLE”
= Multiplexing Negotiation Using SDP Port Numbers 
IETF MMUSIC
Draft: draft-ietf-mmusic-sdp-bundle-negotiation
URL: http://tools.ietf.org/wg/mmusic/draft-ietf-mmusic-sdp-bundle-negotiation/

[4]	“RTP for WebRTC”
 = Web Real-Time Communication (WebRTC): Media Transport and Use of RTP
IETF RTCWEB
Draft: draft-ietf-rtcweb-rtp-usage
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

[5]	“RTP for Telepresence”
 = RTP Usage for Telepresence Sessions
IETF CLUE
Draft: draft-lennox-clue-rtp-usage
URL: http://tools.ietf.org/id/draft-lennox-clue-rtp-usage-04.txt /

Complementary:

[6]	Source-Specific Media Attributes
IETF MMUSIC
RFC 5576

[7]	SDP Grouping Framework
IETF MMUSIC
RFC 5888

[8]	Media multiplexing with RTP subsessions
IETF AVTCORE
Draft: draft-ejzak-avtcore-rtp-subsessions
URL: http://tools.ietf.org/id/draft-ejzak-avtcore-rtp-subsessions-01.txt

[9]	Multiple RTP Sessions on a Single Lower-Layer Transport
IETF AVTCORE
Draft: draft-westerlund-avtcore-transport-multiplexing 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-transport-multiplexing-03.txt

[10]	Multiple Media Types in an RTP Session
IETF AVTCORE
Draft: draft-westerlund-avtcore-multi-media-rtp-session
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multi-media-rtp-session-00.txt

[11]	RTP Considerations for Endpoints Sending Multiple Media Streams
IETF AVTCORE
Draft: draft-lennox-avtcore-rtp-multi-stream
URL: http://tools.ietf.org/id/draft-lennox-avtcore-rtp-multi-stream-00.txt

[12]	Using Simulcast in RTP sessions
IETF AVTCORE
Draft: draft-westerlund-avtcore-rtp-simulcast 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-rtp-simulcast-01.txt

[13]	Cross Session Stream Identification in the SDP
IETF RTCWEB
Draft: draft-alvestrand-rtcweb-msid
URL: http://tools.ietf.org/id/draft-alvestrand-rtcweb-msid-02.txt

[14]	H.248.1v4 Living List: New Appendix V – H.248 Streams “Examples for valid media format suites”
ITU-T SG16
Draft: AVD-4241
URL: see http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/1209_Bri.html

## 1.2	Important terminology

Present work on multiplexed RTP did lead to a revision, modification, extension till the semantical overloading of some RTP related terms. Primarily the terms “media stream” and “RTP session” are used in many slightly different flavours. We are trying to avoid that terms, but when using, we think that following definitions from [1] might be quite useful (“despite the fact that they are not yet fully unambiguous”):

All that terms are either new or extensions/revisions of RFC 1889/3550 terms:

Endpoint:  A single entity sending or receiving RTP packets.  It may

be decomposed into several functional blocks, but as long as it

behaves a single RTP stack entity it is classified as a single

endpoint.

Comment: an endpoint relates to RTP topology “RTP endsystem” in our understanding (see RFC 5117, H.248.RTPTOPO).

Media Source:  The source of a stream of data of one Media Type, It

can either be a single media capturing device such as a video

camera, a microphone, or a specific output of a media production

function, such as an audio mixer, or some video editing function.

Sending data from a Media Source may cause multiple RTP sources to

send multiple Media Streams.

Media Stream:  A sequence of RTP packets using a single SSRC that

together carries part or all of the content of a specific Media

Type from a specific sender source within a given RTP session.

Comments:

- we need to distinguish between “IETF media stream” and “H.248 media stream”

- 1:1 relationship between “media stream” and “SSRC” due to “single SSRC”, right?

- May use different media streams the same SSRC value? The definition allows both interpretations …

RTP Source:  The originator or source of a particular Media Stream.

Identified using an SSRC in a particular RTP session.  An RTP

source is the source of a single media stream, and is associated

with a single endpoint and a single Media Source.  An RTP Source

is just called a Source in RFC 3550.

Comments:

- 1:1 relationship between “RTP source” and “SSRC”, right?

- May use different RTP sources the same SSRC value? The definition allows both interpretations …

Media Sink:  A recipient of a Media Stream.  The endpoint sinking

media are Identified using one or more SSRCs.  There may be more

than one Media Sink for one RTP source.

Multiplex:  The operation of taking multiple entities as input,

aggregating them onto some common resource while keeping the

individual entities addressable such that they can later be fully

and unambiguously separated (de-multiplexed) again.

RTP Session:  As defined by [RFC3550], the endpoints belonging to the

same RTP Session are those that share a single SSRC space.  That

is, those endpoints can see an SSRC identifier transmitted by any

one of the other endpoints.  An endpoint can receive an SSRC

either as SSRC or as CSRC in RTP and RTCP packets.  Thus, the RTP

Session scope is decided by the endpoints' network interconnection

topology, in combination with RTP and RTCP forwarding strategies

deployed by endpoints and any interconnecting middle nodes.

Comment: this is an extension of the RFC 3550 RTP session concept in our understanding. The RTP session is NOT any global concept across all involved RTP entities (such as endsystems, mixers, translators) of a particular communication application.
The RTP session is rather a local concept, given by the perspective of a particular endpoint.

Examples: a two-party call (= point to point topology) between A and B, and with different multiplexing models of each endpoint may lead to the effect that e.g. A perceives a single RTP session whereas B observes multiple RTP sessions for the same communication service.

Note: we may agree to such an “RTP session” concept (given that our interpretation is inline with [1]).

RTP Session Group:  One or more RTP sessions that are used together

to perform some function.  Examples are multiple RTP sessions used

to carry different layers of a layered encoding.  In an RTP

Session Group, CNAMEs are assumed to be valid across all RTP

sessions, and designate synchronisation contexts that can cross

RTP sessions.

Source:  Term that should not be used alone.  An RTP Source, as

identified by its SSRC, is the source of a single Media Stream; a

Media Source can be the source of mutiple Media Streams.

SSRC:  An RTP 32-bit unsigned integer used as identifier for a RTP

Source.

(tbd: The terms "SSRC multiplexing" and "session multiplexing" are

confusing, with unclear historical meanings; they need to be removed

from this document in the interests of clarity)

It might be beneficial to illustrate the multiplexing hierarchy of the defined terminology by [1] (Figure 1 illustrates the traffic source side only):

Figure 1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”)

NOTE: The terminology in [1] should be further improved. We found already a couple of different interpretations.

Importation clarifications required (by IETF):

- Exact relationship between entities (e.g., 1:1, 1:N, etc)

- Exact identifier of an entity („are there any identifiers, and if, which one, given by an n-tuple of …“)

It could be concluded that the evaluation of RTP multiplexing model demands for refined RTP terminology as originally introduced by RFC 1889, but the terminology update work by IETF is still not consistent and stable.

# 2	Discussion

## 2.1	Motivation for RTP multiplexing: Telepresence, WebRTC & Conferencing

We’d like to refer to WebRTC as an exemplary new service with RTP multiplexing considerations. AVD-4274 may be used as a rough introduction of WebRTC in context of H.248 gateways. Figure 2 illustrates an example network scenario: a WebRTC call between two WebRTC endpoints in different network domains (with possible different NAT and NAT-T architectures, different media format usages, different RTP multiplexing support, etc).

Figure 2 – WebRTC use case

Reference: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

The details of WebRTC (or Telepresence) service are out of scope of this document because not relevant for the RTP multiplexing discussion.

## 2.2	Identifier spaces (in user, control and application plane)

The data objects (such as e.g. a protocol data unit at a particular protocol layer), with communication information as carried over the network or/and processed by involved network entities, are referred by identifiers. Figures 3 provides a rough view on WebRTC, from perspective on a WebRTC gateways.

Figure 3 – Identifier spaces (in user, control and application plane) at the example of WebRTC

It may be noted that there are multiple, different identifier spaces in use, primarily due to multiple different protocols used (at different interfaces). Secondly, in scenarios with open application programming environments (such as with browsers in WebRTC) due to additional consideration of API specification (e.g., the common work on WebRTC by IETF RTCWEB and W3C WEBRTC imply a correlation and mapping between “WebRTC data objects” at the IP network interface (IETF) and the browser (W3C)).

There are consequently mappings between different identifier spaces, e.g. between SIP-and-H.248 at MGC level and H.248-and-IP stack at MG level.

Such identifier mappings are well known capabilities, but demand for special attention in case of RTP multiplexing models (see later).

Figure 4 outlines the legacy mapping model, as applied by H.248 and IP bearer traffic, between the identifier spaces of H.248 and IP protocol stacks. The illustration is based on the examples of [15], the discussion of “valid media format suites” in the context of H.248.

Figure 4 – H.248 identifier space and media stream/flow model

There is (was?) a 1:1 correlation between an H.248 flow component (media flow or control flow) and IP traffic flows in case of unmultiplexed RTP traffic.

## 2.3	RTP protocol stack identifier space and basic levels of multiplexing

Figures 5 remembers again the RTP protocol stack identifier space and possible multiplexing options. E.g., at the level of

- IP transport port (usually a UDP port, but possibly also a DCCP or TCP port);

- RTP payload/packet type codepoint; or

- SSRC.

Figure 5 – IP protocol stack identifier space (in case of RTP traffic)

The example in Figure 5 shows two RTP sources (identified by separate SSRC values) with RTCP support. There might be multiple media formats in (serial or/and parallel) use per RTP source, and multiple RTCP report types. Both RTP sources share the same UDP transport endpoint.

## 2.4	WebRTC media configurations: conventions used

We’d like to discuss some example multiplexing use cases in clause 2.7. For a concise description of the examples we are reusing an illustration method as proposed by an IETF draft for RTP usage in WebRTC, see [4]. Figures 6 provides a copy of two multiplexing examples as discussed in [4].

Figure 6 – IETF conventions, as introduced by [4]

Such an IETF convention is useful for highlighting traffic directionality, media sources and sinks, media types, media formats per media source/sink, scope of RTP sources and sinks, mapping to IP transport connections, and the endpoint internal mappings (such as the WebRTC PeerConnection instance in Figure 6).

We are using that IETF convention in an extended manner in clause 2.7.

## 2.5	Multiplexing levels and major media configuration options (WebRTC)

### 2.5.1	Scope and terminology

Documents [1] to [5] indicate that there are many different multiplexing models in view (rather than just a single option) in case of WebRTC, - and the support of different WebRTC endpoint implementations as well as support of interworking to non-WebRTC environments generates a plethora of possible protocol stack combinations between two “RTP endpoints” (such as a WebRTC terminal and an H.248 MG).

In order to reduce complexity and to limit the scope of this initial discussion, we’d like to derive some basic multiplexing options. This is the prime purpose of this clause.

Figures 7 summarizes the multiplexing levels and major media configuration options as considered by WebRTC:

- using a single L4 port for RTP and RTCP packets is denoted as transport multiplexing;

- using a single SSRC for multiple media types is denoted as media multiplexing;

- there might be multiple media types (the legacy understanding of multimeda);

- there might be multiple media formats per media type (due to support of multiple encodings (codecs));

- for each format/type combination could be principally multiple “media streams/tracks” (due to multiple media sources and/or sinks); and

- the option of asymmetry between communication endpoints (due to asymmetrical communication services, but also other reasons).

Figure 7 – Multiplexing levels and major media configuration options (WebRTC)

The legacy notion of “multimedia” becomes varied. Figure 8 provides a visualization attempt related to the three “multi media” dimensions in above list (i.e. bullet items 2 to 4):

Figure 8 – Visualization of the three “multi-media” dimensions

The protocol layer related multiplexing options transport and media multiplexing could be applied in each combination. Table 1 proposes a correspondent naming scheme.

Table 1 – Naming of multiplexing options related to transport and media multiplexing

| Multiplexing options | Transport (NOTE 1) | Media (NOTE 2) | Name |
| --- | --- | --- | --- |
| a) | No | No | Fully unmultiplexed |
| b) | No | Yes | Media type multiplexed |
| c) | Yes | No | RTP/RTCP multiplexed |
| d) | Yes | Yes | Fully multiplexed |
| NOTE 1 – Single L4 port for RTP media flow(s) and RTCP control flow(s). NOTE 2 – Single L4 port for multiple RTP sources. There’s a single SSRC per RTP source. |  |  |  |

Interworking between different multiplexing methods should be fundamentally possible, leading to 16 combinations in a point-to-point communication scenario. The next two sub-clauses illustrating two (of 16) possible use cases.

### 2.5.2	Use case #1: fully multiplexed by both local and remote endpoints

Figure 9, left hand side, depicts use case #1, a scenario between two WebRTC endpoints A and B. Both endpoints apply a fully multiplexed mode. There are the three media types audio, video and text. The identifiers in the symbols of traffic sources using following naming scheme: XYZ with X for denoting the endpoint (here: A or B), Y for media type and RTCP specific traffic (here: A(udio), V(ideo), T(ext), R(TCP)) and Z for numbering multiple traffic components of the same type (here: 1, 2, …).

Figure 9 – Two example use cases

There are two issues with the illustration scheme as used in Figure 9:

- if level ‘UDP’ relates to an IP transport connection, then what means ‘RTP’?

- we’ve added RTCP as self-contained traffic flows besides the three media types, though, RTCP is tightly coupled to RTP media traffic, hm?

As a consequence, in the H.248 use cases in clause 2.7, we will explicitly denote an RTP source (related to an SSRC) and indicate RTCP explicitly for each media type.

Note to fully multiplexed mode:

The various traffic components related to RTP media and RTCP, which are all sharing a single L4 transport component, could be unambiguously identified by the 2-tuple of {PT, SSRC}. This implies a careful assignment of PT codepoints (if from dynamic value range).

However, the IETF is apparently deprecating the PT codepoint in multiplexing models, see [1], “Appendix A.  Dismissing Payload Type Multiplexing”. Emphasizing the solely usage of the SSRC as main identifier for demultiplexing at the level of media multiplexing.

### 2.5.3	Use case #2: local = fully unmultiplexed, remote = fully multiplexed

Figure 9, right hand side, depicts use case #2: the local endpoint (here B) is fully unmultiplexed and the remote side A fully multiplexed.

There are two major observations from use case #2:

1st Such a combination of totally contrary multiplexing configurations is feasible (under the precondition of careful usage of identifier spaces).

2nd The concept of a “RTP session” becomes really a local concept (“endpoint perspective”), rather than a global idea across all involved RTP endsystems (incl. mixer, translator, etc) within a single communication application. This aspect was already introduced in clause 1.2. For instance in use case #2:

- view by A: a single RTP session

- view by B: four RTP sessions

Use case #2 is again a valid network scenario. Each endpoint A and B is just using a different set of identifiers for demultiplexing again the individual traffic components.

## 2.6	SDP control elements for describing (WebRTC) media configurations

Figure 7 did summarize the major multiplexing levels and aspects, leading to the question, which signalling elements are available or in development for describing multiplexed media configurations? The most important application control protocol is SIP in IP networks and H.248 entities. Hence we need to look at SDP elements. Table 2 summarizes SDP control elements according our understanding.

Table 2 – SDP control elements for describing media configurations

NOTE 1: the “Bundle Framework” [3] is still work in progress, at least not fully consistent concerning terminology (e.g., “RTP session” concept).

NOTE 2: how SDP “a=ssrc:” would and could be used by H.248 is still open, primarily due to a) distributed resource management considerations between MGC and MG, and b) the character of a “local source” property, which doesn’t really fit on the LD/RD concept  in H.248. See also clause A.2.

SDP is slightly differently use by SIP and H.248, though, we could imagine that some of above SDP elements might be also used for controlling “multiplexed” H.248 RTP bearer traffic.

## 2.7	Mapping of multiplexed, multimedia media configurations from SIP to H.248

### 2.7.1	Problem statement

There are apparently already all necessary SDP tools available for describing any RTP multiplexing model in SIP-level SDP Offer/Answer messages. At least for WebRTC (also for Telepresence?). Leading to the open question how such multiplexed, multimedia media configurations could and would be mapped on H.248 bearer traffic entities (flow/stream/termination)?

Let’s consider the most complex scenario of a “fully multiplexed local endpoint” in order to study possible H.248 solutions.

What are the basic mapping options? Any multiplexed, multimedia media configuration may be abstracted as a set of multiple, associated traffic flows (in order to avoid confusion with overloaded terms such as media stream; using here a generic term flow as e.g. defined in Y.2770 on DPI).

A single flow instance (as identifiable at the MG IP bearer interface) could be then theoretically mapped on:

- a single termination (and then considering single-stream-per-termination structures);

- a single stream (and then multiple-stream-per-termination structures);

- an H.248.1 (media/control) flow (leading to multiple flow components per stream, i.e., multiple-flow-per-stream structures);

- a flow instance defined by a correspondent filter rule according H.248.79, H.248.DPI;

- an H.248 multiplexing structure (using the Mux Descriptor)?

- an H.248 media group (using ReserveGroup)?

- others?

We’d like to start in evaluating a few mapping options. The problem statement must take into account further aspects, at least:

- the (H.248) requirement: reservation and allocation of a single local destination IP transport address (= LD(A,P)) for all multiplexed media and control flow components

- hence, e.g., any mapping on multi-stream-per-termination structures implies a correlation of all stream-individual LD(A,P) addresses

- MGC resource control options of wildcarding versus fully specified LD(A,P) address values

- MGC-embedded SDP mapper function: using SIP level SDP elements (see Table 2) also at the H.248 interface or not? E.g., the SDP “a=group:BUNDLE” element might be used (“MG bundle aware”) or not (“MG bundle agnostic”), dependent on mapping options ...

Following SIP-to-H.248 mapping options (by the MGC) are studied:

    - option I: “single-stream-per-termination structure” in bundle aware mode using embedded “SDP bundle information elements”

Possible protocol solution: possible extension of SDP usage rules in H.248 required (H.248.1, clause 7.1.8.1.1.1 Rules)
Brief discussion: probably an H.248.1 Version 4 item

    - option II: “multi-stream-per-termination structure” in bundle aware mode, but a new termination-level property for “bundled resource control” across all streams

Possible protocol solution: New H.248 property at TerminationState level
Brief discussion: compatible to existing H.248.1 protocol versions

    - option III: “multi-stream-per-termination structure” in bundle agnostic mode

Possible protocol solution: existing H.248 capabilities
Boundary condition: MGC fully specified LD(A,P) resources
Brief discussion: compatible to existing H.248.1 protocol versions, but all existing H.248 profiles prefer the approach of MG local management of LD(A,P) resources, hence, this concept would be an extension …

    - option IV: “single-stream-per-termination structure” in bundle agnostic mode, using the H.248 media grouping concept

Possible protocol solution: usage of ReserveGroup property 
Brief discussion: there seems to be at least two fundamental issues, firstly the temporal condition due to the requirement for multiple media flows, all active in parallel (and not sequentially), and secondly the need of an explicit identifier for each media group element

    - option V: H.248 Mux descriptor

Possible protocol solution: TBD
Brief discussion: TBD

### 2.7.2	Mapping option I: “Single-Stream, Bundle-aware, SDP described”

Bundle-aware mapping: entire “bundle” mapped on a single stream endpoint (SEP).

TBD (to be done).

### 2.7.2	Mapping option II: “Multi-stream, Bundle-agnostic, TerminationState-Property controlled”

See example in clause A.4.

A new H.248 package would be proposed for defining such a property.

### 2.7.3	Mapping option III: “Multi-stream, Bundle-agnostic, MGC specified LD transport addresses”

Bundle-agnostic mapping: each “m=” line mapped on a dedicated SEP.

Additional aspect: single local LS transport address!? (via CHOOSE …or …)

TBD (to be done).

### 2.7.4	Mapping option IV: “Single-stream, Bundle-agnostic, ReserveGroup Property controlled”

H.248 media groups: clarify whether ReserveGroups allows multiple parallel media flows per stream (or just serial usage).

TBD (to be done).

### 2.7.5	Mapping option V: “Multi-stream, Bundle-agnostic, MuxDescriptor controlled”

TBD (to be done).

### 2.7.6	Mapping option VI: others

There might be other options, hm?

# 3	Conclusions

Several multiplexing models for RTP traffic were introduced. Multiplexing may occur at different levels at the IP protocol stack. Correspondent mapping on H.248 bearer models isn’t straightforward, rather multiple options might exist. Some options may demand for new H.248 signalling elements. Some options may be achieved by (mis)using SDP element in order to force “multiplexing” across H.248 stream/termination entities.

This contribution is far away from a comprehensive evaluation. The purpose is rather to point out problem statements, mapping aspects and to study first examples.

____________________

Annex A – Example signalling

This Annex illustrates some example signalling messages, starting with a generic SDP Offer for SIP-controlled WebRTC in clause A.1.

The subsequent clauses provide example H.248 signalling, labelled as use cases I to ….  It should be noted again that there is not any 1:1 relation between the use cases of this Annex and the mapping options of clauses 2.7.x, because this Annex is also covering all the different multiplexing levels according Table 1, whereas the H.248 mapping options of clauses 2.7.x are only focusing on the worst case of the “fully multiplexed” option of the MG-local bearer connection endpoint.

For brevity, only abstracted media sections of the descriptions are given.

## A.1	Example SIP-embedded SDP Offer for WebRTC

Table A.1 – Example SIP-embedded SDP Offer

| SIP/SDP encoding (shortened SDP media description) | Comments: |
| --- | --- |
| v=0 o=... s=... t=... a=group:BUNDLE audio video text m=audio <UE_A_port_audio_rtp> RTP/XYZ <list of audio media formats> c=IN IP4 <UE_A_IP_addr_audio_rtp> a=rtcp:<UE_A_port_audio_rtcp> IN IP6 <UE_A_IP_addr_audio_rtcp> … a=mid:audio a=rtcp-mux … a=ssrc:<ssrc-id> <attribute>:<value> … m=video <UE_A_port_video_rtp> RTP/XYZ <list of video media formats> c=IN IP4 <UE_A_IP_addr_video_rtp> a=rtcp:<UE_A_port_video_rtcp> IN IP6 <UE_A_IP_addr_video_rtcp> … a=mid:video a=rtcp-mux … a=ssrc:<ssrc-id> <attribute>:<value> … m=text <UE_A_port_video_rtp> RTP/XYZ <list of text media formats> c=IN IP4 <UE_A_IP_addr_text_rtp> a=rtcp:<UE_A_port_text_rtcp> IN IP6 <UE_A_IP_addr_text_rtcp> … a=mid:text a=rtcp-mux … a=ssrc:<ssrc-id> <attribute>:<value> … | NOTE: SDP information elements with respect to - media format details - send/receive control - NAT-T support (i.e., ICE/STUN) - media encryption (i.e., SRTP) are omitted. |

Characteristics:

- SDP block contains three media descriptions (“m=” line) for audio, video and text

- all three media (types) are grouped via bundle (“a=group:BUNDLE”)

- RTP/RTCP transport multiplexing is used (“a=rtcp-mux”)

- RTP media multiplexing uses explicitly the SSRC codepoint (for identification/demultiplexing) (via “a=ssrc”)

## A.2	Use case #I: MG local = fully unmultiplexed, MG remote = fully multiplexed

Characteristics

- communication service:

- WebRTC

- two media types audio and video

- two media formats for audio

- single media format for video

- RTCP used (for both media types)

- remote endpoint A:

- fully multiplexed mode

- there is a single UDP transport connection endpoint

- local endpoint, the H.248 MG:

- H.248 IP-IP MG type, acting as RTP translator

- fully unmultiplexed mode

- thus, not any “SDP bundling” required at H.248 interface

- there are four UDP transport connection endpoints

- explicit assignment of SSRC values for support of RTP media multiplexing by remote endpoint A

- there’s dedicated stream used per media type, thus a dual-stream-per-termination structure

NOTE to SDP “a=ssrc:”: this SDP attribute belongs to the category of source-specific properties (RFC 5576), hence if used at H.248 it would be a Local Source (LS) capability, rather than belonging to the LD or RD (see above diagram, based on H.248.1 conventions)! Leading to the question on the appropriate Descriptor within the Stream Descriptor, e.g. Local or Remote Descriptor, or any other Descriptor?

This is an open question in our understanding. We are using the LD, - if the Remote Destination requires this SDP attribute for RTP packet demultiplexing -, in below examples.

| Use case #I: | H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- | --- |
| MG remote = fully multiplexed, MG local = fully unmultiplexed | Media{ Stream=1{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_RTP1> m=audio <MG_UDP_port_UDP1> 8 13 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=audio <A_UDP_port_UDP1> RTP/AVP 8 13 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 } }, Stream=2{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_RTP1> m=video <MG_UDP_port_UDP3> 96 a=rtpmap:96 VP8/90000 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=video <A_UDP_port_UDP1> RTP/AVP 96 a=rtpmap:96 VP8/90000 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 } } } | Contributor’s note: There is still an inconsistency regarding the association of an IP interface (L3 network connection endpoint) and the corresponding model instances on “A” and on “MG”. * Within current Local descriptors the MG’s RTP session instance “RTP1” is interpreted as uniquely associated to one IP address <MG_IP_addr_RTP1>. This raises the question whether the concept of an “RTP session” is really needed on MG level. Perhaps not. * But related to the “A” side, within the Remote descriptors A’s PeerConnetion instance “PeerC1” is interpreted as uniquely associated to one IP address <A_IP_addr_PeerC1>. Note 1: Here, the a=rtcp-mux attributes seem to be redundant to the a=rtcp attributes, which already indicate the remote RTCP ports from MG’s point of view. Thus, should we remove the a=rtcp-mux attributes? Or should we remove the a=rtcp attributes instead? |

## A.3	Use case #II: MG local = RTP/RTCP multiplexed, MG remote = fully multiplexed

Characteristics

- communication service:

- same as use case #I

- remote endpoint A:

- same as use case #I

- local endpoint, the H.248 MG:

- H.248 IP-IP MG type, acting as RTP translator

- RTP/RTCP multiplexed mode

- thus, not any “SDP bundling” required at H.248 interface

- there are two UDP transport connection endpoints due to shared ports by RTP and RTCP

- explicit assignment of SSRC values for support of RTP media multiplexing by remote endpoint A

- there’s dedicated stream used per media type, thus a dual-stream-per-termination structure

NOTE to RTP/RTCP multiplexed: H.248.1 assumption = “RTP and RTCP flows are always mapped into a single H.248 stream”. See also [ITU-T H.248.57], clause 1.1. Thus a dual-flow-per-termination structure in this case.

| Use case #II: | H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- | --- |
| MG remote = fully multiplexed, MG local = RTP/RTCP multiplexed | Media{ Stream=1{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=audio <MG_UDP_port_UDP1> 8 13 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=audio <A_UDP_port_UDP1> RTP/AVP 8 13 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 } }, Stream=2{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=video <MG_UDP_port_UDP2> 96 a=rtpmap:96 VP8/90000 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=video <A_UDP_port_UDP1> RTP/AVP 96 a=rtpmap:96 VP8/90000 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 } } } | Contributor’s note: As compared to Use Case #I, I replaced the MG’s “RTP session” “RTP1” with an IP connection endpoint “IP1”. Additionally, I replaced the MG’s “Audio RTP Source” and “Video RTP Source” with “Audio Synchronisation Source” and “Video Synchronization Source”, respectively. After all, these synchronisation sources do not only transmit RTP packets but also RTCP packets. Note 1: See UC #x1. Note 2: With the a=rtcp-mux attribute in Local the MGC could instruct the MG to apply local RTP/RTCP multiplexing when allocating transport endpoint resources for the requested new MG stream endpoint. The a=rtcp attribute could not be used as is for this purpose, if the transport endpoint address resource mangement is on MG. [“a=rtcp:$ IN IP4 $” alone, without additional “a=rtcp-mux” would not imply MG has to use same transport endpoint for RTCP as for RTP. An additional info element like new LocalControl property would have to be used with semantic “allocate just one transport endpoint for RTP and RTCP”] |

## A.4	Use case #III: MG local = fully multiplexed, MG remote = fully multiplexed

Characteristics

- communication service:

- same as use case #I

- remote endpoint A:

- same as use case #I

- local endpoint, the H.248 MG:

- H.248 IP-IP MG type, acting as RTP translator

- fully multiplexed mode

- TerminationState-Property controlled multiplexing (so called mapping option II, see clause 2.7.2
=> consideration of new H.248 bundle package (bundlepkg)

- thus, not any “SDP bundling” required at H.248 interface

- single UDP transport connection endpoint

- explicit assignment of SSRC values for support of RTP media multiplexing, also now for local endpoint, beside remote endpoint A

NOTE to SDP “a=ssrc:”: we are using now also the RD for the SSRC-based demultiplexing in incoming direction.

| Use case #III: | H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- | --- |
| MG remote = fully multiplexed, MG local = fully multiplexed | Media{ TerminationState{ bundlepkg/bundle{streams=[1,2]} ; Note 3 }, Stream=1{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=audio <MG_UDP_port_UDP1> 8 13 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=audio <A_UDP_port_UDP1> RTP/AVP 8 13 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 a=ssrc:… } }, Stream=2{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=video <MG_UDP_port_UDP1> 96 a=rtpmap:96 VP8/90000 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=video <A_UDP_port_UDP1> RTP/AVP 96 a=rtpmap:96 VP8/90000 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 a=ssrc:… }}} | Contributor’s note: As compared to Use Case #II replace “RTP Session Multiplexing” with “Media Description Bundling”, because (according to my understanding) no two (or more) RTP sessions are actually multiplexed onto the same local transport endpoints, but different media description associated (RTP + RTCP) packet flows are multiplexed onto the same local transport endpoint. Note 1: See UC #I. Note 2: See UC #II. Note 3: Here, on termination level (within the TerminationState descriptor) I could imagine a new package property requesting the MG to apply “bundling” when creating the stream endpoints listed as property value. This raises many questions, though. Such a TerminationState “bundle” property would probably always be set already at the point in time a new SEP is created. What exactly is the relationship to RTP/RTCP multiplexing (to the a=rtcp-mux attributes in the Local descriptors)? |

# Attachment – Useful terminology from Draft Y.2770 on DPI

Ref.: T09-WTSA.12-C-0030!!MSW-E.docx

3.1.3	flow  [IETF RFC 5101]: A set of IP  packets passing an observation point in the network during a certain time interval. All packets belonging to a particular flow have a set of common properties. Each property is defined as the result of applying a function to the values of:

1) one or more packet header fields (e.g., destination IP address), transport header fields (e.g., destination port number), or application header fields (e.g., RTP header fields [b-IETF RFC 3550]).

2) one or more characteristics of the packet itself (e.g., number of MPLS labels, etc.).

3) one or more of fields derived from packet treatment (e.g., next hop IP address, the output interface).

A packet is defined as belonging to a flow if it completely satisfies all the defined properties of the flow.

This definition covers the range from a flow containing all packets observed at a network interface to a flow consisting of just a single packet between two applications. It includes packets selected by a sampling mechanism.

NOTE – The above numbered listed items indicate flow properties in the categories of (1) “Protocol Control Information (PCI) of packets”, (2) “Protocol Data Unit (PDU) properties of packets” and (3) “Local packet forwarding information”.

3.2.16	flow descriptor (also known as flow level conditions): Set of rule conditions that is used to identify a specific type of flow (according clause 3.1.3) from inspected traffic.

NOTE 1 – This definition of flow descriptor extends the definition in [b-ITU-T Y.2121] with additional elements as described in clause 3.

NOTE 2 – For further normative discussion of the flow descriptor as used in this Recommendation, see Annex A.

3.2.17	IPFIX flow identifier (IPFIX flow ID): The set of values for the IPFIX flow keys, which is used in conjunction with the flow descriptor to identify a specific flow.

3.2.18	IPFIX flow key: Each of the information elements of the flow descriptor that is used in IPFIX-based flow identification processes (according to [IETF RFC 5101]).

NOTE – The IPFIX flow key definition is semantically consistent with the flow key definition specified in IPFIX [IETF RFC5101]. The only difference between the two terms is that the definition in this document is scoped to the flow descriptor.

Annex A/Y.2770:  Specification of flow descriptor

A.1	Protocol syntactical perspective

The flow descriptor relates to a data structure (data object), which may be modelled as k-Tuple (see Figure A.1). The data structure consists of k information elements (IE) (NOTE 1). The value of k is variable and greater than zero, but constant for a particular flow. The information elements are the ones as contained in the IANA IPFIX registry. There is a value associated to each information element. The association is typically mathematical equality (‘=’), but other mathematical relations are not excluded.

NOTE 1 – The IETF IPFIX information elements may be attributed as “key field” or “non-key field”.

Figure A.1 – The flow descriptor (flow level conditions) from protocol syntactical point of view

The flow level descriptoras a k-tuple represents consequently a list of k “name-value pairs” (NVP); here a sequence of “< IE  value >” pairs).

A.2	Specifying information element values

In the flow level conditions, the value of an IE may be:

- fully specified

Full specification represents the case of a complete name-value setting.

- not specified

“Not specified” represents the case when there is not yet any value assigned to an IE.

- overspecified or

Overspecification indicates there are multiple possible values for a specific IE.

- underspecified.

Underspecification indicates wildcarding (e.g., all possible values, or choose value).

A.3	Relation between flow descriptor, IPFIX flow identifier and IPFIX flow key

The example in Figure A.2 provides a 5-tuple flow descriptor and contains 5 IFPIX flow keys. In order to identify a particular flow, the flow descriptor imposes some conditions on the values of these flow keys as defined in clause A.2: the first flow key IE x1 is “fully specified”, the second flow key IE x2 is “overspecified”, while the others IEs are “not specified”, as displayed in the part a) of the Figure A.2.

Figure A.2 – Flow Descriptor, IPFIX Flow Identifier and IPFIX Flow key example

Note that the flow descriptor doesn’t impose conditions on the IPFIX flow keys only: indeed, in some circumstances, flow descriptors on non flow key might be required, for example when a condition of the TCP flags of the first packet of the flow is required. The fundamental difference between the flow descriptor and the IPFIX flow identifier in the example Figure A.2 is that the flow descriptor contains a “superior than” condition on the IE x2, (“IE x2 > value y2”), while the IPFIX flow identifier contains the observed value for the IE x2, i.e., value yy2. The IPFIX flow identifier is composed of the set of observed values for the flow keys, once the DPI functional entity processed the packets and classified them into a flow.

Note that if the exported information (e.g., via an IPFIX flow record) contains each IEs along with the associated observed values, and whether or not the IE is an IPFIX flow key, then there is no need to assign a specified IPFIX flow identifier, as the IPFIX flow identifier is the sum of all this information.

____________________

Native multimedia model: e.g. as defined for initial SIP conference systems or H.320, H.323 or H.324 multimedia, i.e. a single audio/video/text/data associated with single input/output devices.

Roughly, the lower the number of IP transport connections per (multimedia) application, the lower the complexity and cost of NAT traversal, and the better of some QoS figures in the user/control plane (such as connectivity and communication establishment delay).

Note: N = 0 indicates “Flow-independent ”

Similar to other structures like AVP (<attribute name, value>), parameter-value pair (<parm=value>), etc.