| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4274 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | New work item: H.248.WEBRTC "Guidelines on the use of H.248 capabilities for Web-based Real-Time Communication Services in H.248 Profiles" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes the start of a new work item (called H.248.WEBRTC) with regards to the support of web-based real-time communication services in H.248 profiles.

1	Introduction

There’s an attempt to standardize web-based real-time communication services by the two SDOs IETF and W3C, see e.g. http://tools.ietf.org/wg/rtcweb/charters.

This is primarily a browser-to-browser based conversational service, but there are also already some scenarios identified with demand for gateway support, e.g. due to interworking with existing devices for conversational services.

2	WEBRTC H.248 gateways – Motivation

2.1	Example network use cases

2.1.1	Point-to-point communication

Figures 1 to 3 illustrate three prominent example use cases. There are further ones. All three examples are characterized by a two-party call (between A and B) and that party A is always using WebRTC browser environment.

Figure 1 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a brower-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two WebRTC IP domains, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture by the two WebRTC IP domains, etc.

Fig. 2 depicts an interworking scenario with NGN/IMS networks:

Figure 2 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed to be not fully identical to WebRTC endpoints (in terms of … see clause 2.2).

Fig. 3 outlines the general request in supporting of still existing PSTN/ISDN infrastructures.

Note – Both use cases #2 and #3 are indicated in Internet-Draft “RTP for WebRTC” (draft-ietf-rtcweb-rtp-usage), see clause A.4.2. Gateway / Protocol Translator.

Figure 3 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. H.324, H.320).

All three use cases share common H.248 capabilities and contain additional interworking specific requirements.

2.1.2	Multipoint communication

Multiparty calls are in scope of WEBRTC and may demand for “RTP mixer”, as e.g. provided by media servers. Such a network scenario may hence related to H.248 profile types as defined for media servers (aka media resource processors).

2.2	Example capabilities of WEBRTC H.248 gateways

Following table highlights for use case #2 potential functional areas which may require support by WEBRTC H.248 gateways:

| Area | WEBRTC | IMS |
| --- | --- | --- |
| Audio codecs | G.711, Opus | AMR, AMR-WB |
| Video codecs | VP8, H.264, H.263, H.261 | H.264 |
| Media security | DTLS-SRTP; under consideration: SDES | SDES, MIKEY (TICKET) |
| Media routing and NAT Traversal | ICE | Latching |
| Multiplexing | Bundle and multiple SSRC | native |
| RTP profile: | … | … |
| RTCP usage: | ... | ... |
| Congestion control in IP bearer path: | different options discussed | ECN |
| Media component ‚data‘ (and transport protocol stack) |  |  |
| ID management | … | … |
| others |  |  |

NOTE: the table is not complete, and may be also not up to date due to the ongoing progress on WebRTC. However, the important point is the expected difference between WebRTC and IMS endpoints.

3	Scope of work item: H.248.WEBRTC

The type of H.248.x-Recommendation should be basically a “profile guideline” document, such as H.248.81, H.248.TLSPROF and H.248.RTCPPROF, due to the expectation that WEBRTC support will lead to different H.248 “WEBRTC profiles”.

Following items are in scope:

- Use case examples with H.248 WebRTC gateway involvement

  - as H.248 IP-IP gateway

  - as H.248 IP-PSTN gateway

- Use case specific requirements for H.248 WebRTC gateways

- NAT-T support by H.248 WebRTC (e.g. NAT-T profile based on H.248.50/H.248.37 capabilities)

- WebRTC IP bearer traffic: H.248 WebRTC gateway support of “Media Transport and Use of RTP (draft-ietf-rtcweb-rtp-usage)” capabilities

- RTP topologies for H.248 WebRTC gateways (relation to H.248.RTPTOPO)

- Complementary services by H.248 WebRTC gateways, such as due to policy/QoS based frameworks (e.g., traffic policing, QoS marking), performance monitoring, etc.

- Usage of media security (relation to H.248.77)

- Usage of congestion control (relation to H.248.82 or other congestion controls)

- others

It looks like that all required “H.248 tools” are already existing for WEBRTC support, however, it may be not excluded that the H.248.WEBRTC work item may identify new H.248 protocol support (due to the still ongoing work in IETF/W3C on WEBRTC standardization). E.g., list item #4 relates to “RTP multiplexing” (see discussion by AVD-4272).

4	Proposal

It is proposed to accept this new work item for Q.3/16.

Annex A
Proposed new ITU-T Recommendation H.248.WEBRTC

| Question: | 3/16 | Proposed new ITU-T Recommendation | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.248.WEBRTC "Guidelines on the use of H.248 capabilities for Web-based Real-Time Communication Services in H.248 Profiles" |  |  |  |  |
| Base text: | Contribution AVD-4274 |  | Timing: | 2013 |  |
| Editor(s): | Albrecht Schwarz, ALCATEL-LUCENT, Albrecht.Schwarz@alcatel-lucent.com |  |  |  |  |
| Scope: |  |  |  |  |  |
| Major items for a new draft H.248.WEBRTC - there should be at least following aspects considered: - Use case examples with H.248 WebRTC gateway involvement - Use case specific requirements for H.248 WebRTC gateways - NAT-T support by H.248 WebRTC (e.g. NAT-T profile based on H.248.50/H.248.37 capabilities) - WebRTC IP bearer traffic: H.248 WebRTC gateway support of “Media Transport and Use of RTP (draft-ietf-rtcweb-rtp-usage)” capabilities - RTP topologies for H.248 WebRTC gateways (relation to H.248.RTPTOPO) - Complementary services by H.248 WebRTC gateways, such as due to policy/QoS based frameworks (e.g., traffic policing, QoS marking), performance monitoring, etc. - Usage of media security (relation to H.248.77) - Usage of congestion control (relation to H.248.82 or other congestion controls) |  |  |  |  |  |
| Summary |  |  |  |  |  |
| Web-based real-time communication is a standardized service by the IETF and W3C (called WEBRTC or RTCWEB service) and defines a particular protocol suite for IP-based communication in web browser environments. This application is related to multimedia conversational services which covers typical service components such as telephony, conferencing instant messaging, etc. This Recommendation – describes example use cases with H.248 WEBRTC gateway involvement; – identifies H.248 capabilities for such gateways; and – provides guidelines for the specification of H.248 profiles for dedicated H.248 WEBRTC gateway types. |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| - Telepresence (due to media transport) - H.248.37, H.248.50, H.248.52, , H.248.53, , H.248.77, H.248.82, H.248.RTPTOPO - IETF RTCWEB documents |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Possibly with IETF, W3C, 3GPP. |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Alcatel-Lucent |  |  |  |  |  |

_______________________