| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4277 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Brisbane, AUS, 24 - 28 September, 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.Prio: Generalization of Appendix III |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an attempt in generalizing the text of Appendix III, independent of the general editor’s note. We still think that such kind of information would be useful for the H.248 community, provides consulting oriented information and may relax resistances with regards to protocol upversioning.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 794 (WP 2/16), the output draft from last Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix III

Stepwise introduction of H.248.1 Version capabilities

{Editor’s note: This Appendix is temporarily contained in this Supplement for reference purposes whilst this Supplement is being developed. It will be removed at the time of Approval.}

Any H.248 interface (or Control Association) may be either operated with our without a profile (which may be termed as profile-full and profile-less modes). The profile mode is controlled via the ServiceChangeProfile parameter. The applied H.248 protocol version would be specified in the profile-full mode (see clause 6.3 in the profile definition template Appendix III/[ITU-T H.248.1]).

The real goody of profile-full mode and their H.248 profile specifications is the fact that individual H.248 protocol elements may be gradually supported (or not). Thus, the stepwise introduction of H.248.1 version-specific capabilities could be easily achieved when using a profile specification. It may be reminded that the purpose of the profile versioning concept itself inherently aims support of gradual capability upgrading.

The stepwise introduction method may be illustrated at the example of a profile with H.248.1 Version 2 support, the request of IEPS indicator introduction and the question of upgrading towards V3. Figure III.1 depicts the two basic up versioning strategies, either full (I) or just partial (II) support of V3 capabilities.

Figure III.1 – H.248.1 protocol version support – Upversioning strategies

The IEPS call indicator based signalling solution for priority traffic services would imply support of following two V3 capabilities:

a)	V3 compliant H.248 message encoder/decoder; and

b)	signalling property IEPS call indicator.

Qualitative evaluation aspects:

Capability (a) might be a minor delta because there is not any significant syntactical upgrade from V2 to V3 (with regards to the H.248.1 core protocol).

Capability (b) might be similar effort, comparable to the support of a new package with a single property.

Some observations:

- A stepwise introduction of H.248.1 V3 might be an alternative, pragmatic way forward and also feasible from specification point of view because not violating protocol/profile specifications.

- A stepwise introduction of H.248.1 V3 might be a smaller step in this example (see items (a) and (b)), in contrast to the full protocol version upgrade.

Warning: any upversioning strategy should of course compliant against [ITU-T H.248.1].

[End Proposed Correction]

____________________