| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4294 |  |
| --- | --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): | Question 2 | Brisbane, 24 - 28 September 2012 |  |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |  |
| Source: | GNU Gatekeeper Project |  |  |  |  |
| Title: | Document Facility Call Transfer Method |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |

Summary

I propose to document the long-standing practice for call transfers using the Facility message with reason callForwarded in the H.323 Implementers Guide. It is important to make implementers aware of this practice, because very few implementations support H.450.2 and this is the most widely supported mechanism for call transfer.

# Introduction

Proposed text to be added to the H.323 Implementer Guide

Use of Facility to transfer established calls

According to H.225.0 (v7) clause 7.2.2.16, calls can be forwarded using a Facility message with reason callForwarded. Many implementations also allow forwarding calls that have already been established in the same manner, effectively implementing a lightweight call transfer.

Given the complexity and low adoption of H.450.2 this is the most widely supported mechanism for call transfers and implementations should consider adding support for it.

This is a non-standard procedure, but documented here since the technique is widely implemented and interoperability issues sometimes arise due to the fact that some devices lack support.

__________________