| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4303 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | NEC |  |  |  |
| Title: | F.TPS-Reqs: Acceptable angles for eye contact |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution makes a comment on the section 2 (Eye contact) of TD898rev1(GEN/12) to COM16-LS300.   SG12 replied that the tolerable angles are 1.2° in horizontal direction and 1.7° in vertical direction. These angles indicate high sensitivity of human eyes on eye-direction of other persons.  But technically those angles are difficult to implement in video conference systems, especially for persons having edge seats of video conference room.  Q5/16 should not mislead end-users on telepresence experience using this term, eye contact.  It is suggested that the definition should be loosened.   An alternate proposal is to use gaze awareness instead of eye contact.

# Introduction

SG12 introduced an article of QoMEX2010, and the article indicated that the tolerable angles were 1.2° in horizontal direction and 1.7° in vertical direction.  The 1.7° is about 6cm-up (or down) on a screen at 200cm far from the screen.

Telepresence systems usually employ large displays (e.g. height is 70cm) and cameras are placed on the upper-edge of displays.  In this case, gap between “eye-position of a person in a screen” and “a camera located top of the screen” is about 35cm and the angle is 10°.  This means that today’s telepresence systems can not provide the tolerable angle suggested by SG12.

Note: There are several special ways to place camera in the line of gazing (e.g. to use half-silvered mirror and/or translucent screens), but they are not practical solutions.

Natural sense of mutual eye contact is not possible to provide by video conference systems, but the industry has used this term, eye contact, with implicitly loosened definition.  As for one-way eye contact, all video conference systems can provide it when a person gazes at a lens of the camera capturing the person.  In this case, however, the person is not watching the other person in a screen at the point of time, so mutual eye contact is not provided.

# Discussion

## Views on eye contact

The table below classifies views on eye contact.

|  | A telepresence system |  |  |
| --- | --- | --- | --- |
|  | Seat position enabling eye contact |  | Seat position NOT enabling eye contact |
|  | Mutual eye contact | One-way eye contact |  |
| Eye-contact as a term for non-verbal communication | Face-to-face meeting | - - - | - - - |
| Eye-contact (zero degree) |  | Broadcasting only when gazing a camera | - - - |
|  | [Experimental video conferencing] using half-silvered mirror, etc. | [Video conferencing] only when gazing at a camera (not a screen) |  |
| Tolerable eye-contact | Video conferencing with a gap from gazing direction of a person captured |  | - - - |

As a general term, eye contact is non-verbal communication and done only by gazing eyes mutually and simultaneously.  This is basically mutual eye contact.

As for broadcasting, TV viewers feel eye contact when a performer gazes a lens of TV-camera.  This situation is one-way eye contact.  Video conference systems can also provide this one-way eye contact, only when a person watching a lens of camera instead of watching a remote person in a screen.  This situation is not watching mutually and simultaneously.

Tolerable eye contact is a draft term focused on video conferencing and it is considered for the industry which wishes to use this term, eye contact, for telepresence systems.  The “tolerable” here means a wider gap of eye-direction between “a person” and “a remote person in a screen”, if end-users can talk without severe discomfort.

The alternative solution for the tolerable eye contact is to employ gaze awareness instead.  The gaze awareness here indicates a well-managed system of eye-direction (gazing-direction) to end-users.

Furthermore, eye contact is not possible for all of participants.  It depends on camera position and also seat positions (e.g. edge seats of a row).

As the draft definition of “telepresence” includes this term, eye contact, Q5/16 needs to review it.

# Proposal

Proposal A-1:

The term, eye contact is used for video conferencing, by defining it as “tolerable eye contact” which allows larger angles* on gazing-directions between two persons communicating.

A note should be added that eye contact (even tolerable eye contact) depends on seat positions.

*: The specific angles are TBD, but “up to 10 degrees” as an example.

Proposal A-2 (alternate):

Video conference systems including telepresence systems are not able to provide even tolerable eye contact to all of participants.  In order NOT to mislead end-users on eye contact, the industry should stop using this term, eye contact.  But “gaze awareness” is used instead, when any outstanding features of telepresence systems are described.

Proposal B

User02* of F.TPS-Reqs is proposed to be a Living List item of F.TPS-Reqs until Q5/16 reaches a final consensus on the eye contact.

*: The telepresence system should provide a natural eye contact experience.

Proposal C:

Apart from the above issue on eye contact, it is proposed to add a definition of gaze awareness to section 3 (definition) of F.TPS-Reqs.

Gaze awareness: To be aware of each direction of gaze of captured persons by watching eyes of those persons in a screen.

# Supplement (as a reference)

The followings are charts which introduced briefly in the last Q5 meeting in May/2012.

The study was done by NEC in 1967 for video “phone” systems that is a case for facing each other.

Reference:

The picture-phone system, 2C Video telephone station set, page 277

Vol.50   Feb/1971  Number 2, The Bell Systems Technical Journal

__________________