| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4313 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2, 12/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Using H.323 as a stepping stone to H.325 functionality |  |  |  |
| Purpose: | Discussion |  |  |  |

# Introduction

H.325 provides architecture to support distributed and media rich collaboration environments, where there are many H.325 enabled devices providing many applications and services. The user is free to co-ordinate activities utilizing this pool of applications and services provided by these devices to fulfil business or personal needs. This is a generational step beyond the current monolithic device/ protocol stack of H.323 or SIP.

There is a growing need for more ad-hoc distributed architectures with the growing concept of BYOD (bring your own device). Employees are increasingly bringing their own mobile devices (“Container”) to the workplace and using these devices to access the company owned and controlled resources (“Applications”). The monolithic and archaic cubical based PBX model is giving way to the more agile mobile, contact anywhere, style models.

Technology advancements in the last few years has seen more low cost powerful mobile devices and associational technology such as NFC Near field communication increasing the opportunity and reducing the cost of providing H.325 type technologies.

Over the same period there has been a reduction in contributions within ITU-T SG16 WP2 on H.325 due in part to the global recession.

The purpose of this document is to explore how H.323 can be extended to provide H.325 style functionality as a method to demonstrate the H.325 value proposition.

# Discussion

This document proposes discussion points on how to utilise H.323 to perform similar functionality to H.325 for the purpose of creating a working environment to promote H.325 development.

# Emulating H.325 functionality in H.323

After some ad-hoc discussions at the last ITU meeting in Geneva, it was discussed how H.323 can be used to promote and demonstrate the advantages of H.325.

Suggested topics for discussion.

- Associating H.323 devices and their capabilities

- Collecting and advertising those capabilities

- Invoking those capabilities

- Ensure backwards compatibility with existing H.323 systems

Some other considerations are:

- Media Encryption

- NAT/FW Traversal.

## Associating H.323 devices and their capabilities

Within this call scenario the assumptions are.

- No Fast Connect

- H.245 Tunnelling between associated H.323 devices.

Some proposals on how H.323 devices may associated.

- GRQ discovery.

A H.323 device advertises for other H.323 devices via H.460 Feature Identifier in a requiredFeature elements of the featureSet in a multicast GRQ.  Supporting devices respond in the GCF by providing H.225 AliasAddress(es) contained in a H.460 featureParameter of a supportedFeature element of the featureSet.

- Near field communication.

The a handheld H.323 device is waved over another H.323 device to retrieve H.225 AliasAddress(es). For further discussion.

- Manually by entering H.225 AliasAddress information into the device.

The H.323 devices being associated may be on 2 distinct H.323 networks so it is suggested that the enquiring H.323 devices use the H.225 AliasAddress to call the associated device using existing H.225 procedures. The SETUP PDU shall set h245Tunneling to true and contain a conferenceGoal set to callIndependentSupplementaryService. The SETUP PDU shall also contain a requiredFeature element containing a H.460 feature to advertise the H.235 type feature, within the feature shall contain a H.460 featureParameter to present a human readable device identifier (ie John Smith’s IPad) and a H.460 featureParameter indicating this is request for the devices’ capabilities. For security purposes, the device receiving the request may prompt the user for authorisation to associate. Upon approval, the receiving device generates a securityToken. If not approved or request time has lapsed a RELEASECOMPLETE is sent.

The newly associated H.323 device returns a CONNECT PDU containing the H.460 Feature identifier as a supportedFeature element in the featuresSet field with a H.460 featureParameter containing the securityToken. The h245Tunneling is set to true and the h245Control element contains the devices H.245 terminalCapabilitySet. Once received the originating H.323 device sends a RELEASECOMPLETE to the associated device to release the call.

## Collecting and advertising associated device capabilities

Once the terminalCapabilitySet is received by the originating device, the capabilities are merged with the devices capabilities. The user may be prompted to select which capability to use from the associated device and only those selected are merged.  Details of the H.225 aliasAddress, securityToken and capabilityTableEntryNumber are stored with the associated capability.

The device may place a SETUP PDU with conferenceGoal of callIndependentSupplementaryService to the associated device at any time. In the SETUP PDU shall include the securityToken in a H.460 featureParameter and a H.460 featureParameter identifying a request for availability. These shall be contained in a H.460 Feature element in the requiredFeature field. It may receive back a FACILITY or CONNECT PDU with a genericMessage containing a H.460 Feature element which includes an enumeratedParameter listing the capabilityTableEntryNumber and availability.

When a call is placed the merged capability list is advertised as per normal H.245 procedures in the originating devices’ TCS.

## Invoking associated device capabilities

When invoking a capability, a FACILITY PDU is sent to the associated device containing a H.460 featureParameter requesting an OLC/OLCAck for the provided capabilityTableEntryNumber. The responding OLC/OLCack shall be contained within an h245Control element of a FACILITY PDU. The OLC/OLCack is passed through to the remote device. When an OLC/OLCack is received from a remote device it shall be forwarded onto the associated H.323 device via a FACILITY PDU with the OLC/OLCack contained in the h245Control element.

## Ensure backwards compatibility with existing H.323 systems

The H.225 Call Signalling and H.245 Media Control should appear to the remote system as being identical to any other H.323 device.

# Special Considerations

## Media Encryption

For Further study.

## NAT/FW Traversal.

For Further study.

___________________