| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4314 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia |  |  |  |
| Title: | H.460.24amd2 Improving support for a multiplexed media mode for point to point media |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This Contribution proposes a new Amendment with modifications to ITU-T H.460.24 to better support a multiplexed media mode.

# Discussion

The content of the document was last presented at the SG16 meeting 5/2012 in Geneva as C.854. The amendments were approved at the meeting. Several experts’ comments documented in the Q2/16 meeting report highlighted suggested alterations:

[Excerpt TD 504/WP2]

It was noted that the proposed changes to the ASN.1 would break backward-compatibility with the current H.460.24 text. The new “multiplexID” should be added after the extension marker.

It was noted that a recent addition to H.245 wherein the OLCAck might contain a different payload type value than in the OLC may affect these procedures and needs to be considered by the contributor.

It was agreed to create a new work item as an amendment to H.460.24 or a revised H.460.24 with a plan to consent at the next SG16 meeting.

# Conclusion

The ASN.1 Syntax of H.460.24 Annex B has been corrected to support backwards compatibility.

The recent addition to H.245 Annex T has no effect on the proposed procedures. The procedures are limited only to the transmission of a 4-btye multiplex header on the RTP packets. All procedures with respect to RTP and RTP PayloadType value are beyond the scope of this document and shall be consistent with H.245.

_________________

Proposed Changes to H.460.24 Annex A

…

## A.1	Notification of direct connectivity probing

Endpoints supporting this annex and being notified of media strategy indicator 7 (same NAT/FW) shall include in the GenericInformation field of the ITU-T H.245 OLC and responding ITU-T H.245 OLCAck, a packet encoding rules (PER) encoded parameter with the object identifier (OID) as specified in Table A.1. The messageContent shall contain an array of string representations of the channel unique identifier (CUI) and the alternate RTP and RTCP address and port and optionally, where supported, the [ITU-T H460.19] MultiplexID. The identifiers shall be in accordance with Table A.2.

The channel unique identifier (CUI) shall be an endpoint-generated identifier for the purposes of identifying, authenticating and allocating received direct connectivity RTCP probe packets.

Table A.1 – OID for same NAT generic message

| Object identifier value | OID name |
| --- | --- |
| { itu-t (0) recommendation (0) h (8) 460 24 AnnexA (1) } | H.460.24 OLC Same NAT probe |

Table A.2 – ITU-T H.245 generic parameters

| Identifier | Indication | Raw type | Encoded type |
| --- | --- | --- | --- |
| 0 | Channel unique identifier | IA5String | octetString |
| 1 | Media channel IP address and port number | TransportAddress (as defined in ITU-T H.245) | octetString |
| 2 | Media control channel IP address and port number | TransportAddress (as defined in ITU-T H.245) | octetString |
| 3 | [ITU-T H460.19] MultiplexID | unsigned32 | unsigned32Max |

The RTP and RTCP IP address and port and optional MultiplexID in the generic parameters shall mirror the values in the media channel and media control channel parameters contained in the same [ITU-T H.245] OLC. The optional MultiplexID shall be the same value as supplied in multiplexID field of the [ITU-T H460.19] Traversal Parameters of the same OLC.  The CUI shall be used solely to verify the source of received probe packets.

…

A.2	Procedure to probe direct connectivity

…

Once direct connectivity has been verified either by sending or receiving the reply RTCP probe packet, the endpoint shall transmit a genericIndication message to the responding endpoint containing the same OID shown in Table A.1. If both endpoints transmit and receive a genericIndication message containing the OID from Table A.1, then each endpoint can assume that bidirectional connectivity has been verified. At this point, each endpoint shall stop sending media and control to the supplied ITU-T H.245 OLC/OLCAck address and begin to stream media and control to the verified alternate or detected RTP/RTCP address and port. Where a [ITU-T H.460.19] MultiplexID has been supplied, the media and control stream shall be in the format specified in [ITU-T H.460.19] clause. 7.3.2.  The RTP channel shall be established in the same order, even if the channel is unidirectional, as the successful RTCP probe packet, that is, the successful RTCP probe packet receiver shall wait for the first RTP packet from the RTCP probe packet sender to enable it to assign the detected IP address and port to send media.

…

The receipt of the genericIndication message from both endpoints shall also indicate to the ITU-T H.460.19 server that media and control for the associated session will be transmitted directly.

Where a [ITU-T H.460.19] MultiplexID has been supplied, all direct media and control stream shall be in the format specified in [ITU-T H.460.19] clause. 7.3.2.

…

## A.3	Probe packet format

The probe packet shall be constructed as an Application-Defined RTCP packet in accordance with clause 6.7 of [IETF RFC 3550] with the settings specified in Table A.3. Where a [ITU-T H460.19] MultiplexID has been supplied, the RTCP packet shall be in the format specified in [ITU-T H460.19] clause. 7.3.2.

Proposed Changes to Annex B

…

## B.2	Procedure to probe for direct connectivity

…

The gatekeeper shall transmit a genericRequest message containing the OID of Table B.2 and a generic parameter as per Table B.3 containing an encoded AlternateAddresses element. The element shall contain a sequence of AlternateAddress fields containing the detected apparent source addresses for media and control and session id for each session requiring direct media probing. Where [ITU-T H.460.19] clause.7.2  Multiplex media mode is employed, the AlternateAddress element shall also contain the same MultiplexID as the associated outbound multiplex media streams. The sessionCUI field shall be omitted. Once the endpoint has received the genericRequest, it shall send at least 3 RTP/SRTP/RTCP/SRTCP probe packets (refer B.3 for format) to the corresponding media/control addresses for each session from the associated local media/control addresses for that media session.

…

Upon receipt of the genericResponse, the gatekeeper initiates a genericRequest to the other endpoint with an encoded AlternateAddress element containing a sequence of AlternateAddress fields containing the detected apparent source addresses for media and control, session id and received sessionCUI for each session requiring direct media probing of the first NAT/FW. Where [ITU-T H.460.19] clause.7.2 Multiplex media mode is employed, the AlternateAddress element shall also contain the same MultiplexID as the associated outbound multiplex media streams. The second endpoint upon receiving the genericRequest, shall send at least 3 RTP/SRTP/RTCP/SRTCP probe packets (refer B.3 for format) to the corresponding media/control addresses for each session from the associated local media/control addresses with the received sessionCUI for that media session. Once the probe packets have been sent the endpoint shall respond back to the gatekeeper via a genericResponse message containing the OID of Table B.2. The AlternateAddress element shall be omitted.

...

## B.3	Probe packet format

For RTP/SRTP the probe packet shall have a payload type equal to the media payload type, empty payload size and sequence number starting at some arbitrary value and incrementing by one for each new probe packet.

The RTCP/SRTCP probe packet shall be constructed as an Application-Defined RTCP packet in accordance with clause 6.7 of [IETF RFC 3550] with the settings specified in Table B.4. Where a [ITU-T H460.19] MultiplexID has been supplied, the RTCP packet shall be in the format specified in [ITU-T H460.19] clause 7.3.2.

…

## B.4	Annex B ASN.1 code

MEDIA-TRAVERSAL {itu-t(0) recommendation(0) h(8) 460 24 2 asn1-module(1)} DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

TransportAddress

FROM MULTIMEDIA-SYSTEM-CONTROL -- See Rec. ITU-T H.245;

AlternateAddresses ::= SEQUENCE

{

addresses		SEQUENCE OF AlternateAddress,

...

}

AlternateAddress ::= SEQUENCE

{

sessionID	  INTEGER(0..255),

sessionCUI   IA5String                 OPTIONAL,

rtpAddress	  TransportAddress	  OPTIONAL,

rtcpAddress	  TransportAddress	  OPTIONAL,

...,

multiplexID   INTEGER(0..4294967295)  OPTIONAL

}

END

_____________________