| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4320 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Cisco Systems, Inc. |  |  |  |
| Title: | Proposed Revised Recommendation H.460.2 |  |  |  |
| Purpose: | Proposal |  |  |  |

Proposal

This contribution incorporates the agreed text records in the H.323-Series Implementor’s Guide into the current ITU-T Recommendation H.460.2.  We propose to revise this recommendation and, simultaneously, removing the associated text from the Implementor’s Guide.

ITU-T Recommendation H.460.2 Number Portability interworking between H.323 and SCN networks

Summary This Recommendation describes the procedures and the signalling protocol for Service Provider and Location Portability, while interworking with SCN (using SS No. 7 signalling) interfaces in a H.323 network. Service Provider Portability allows subscribers to keep their existing phone numbers/addressing scheme even when changing from one service provider to another without changing their location, and without changing the nature of the service offered. Location Portability allows subscribers to retain their existing phone numbers/addressing scheme even when moving from one location to another. This Recommendation makes use of the "Generic extensibility Framework" specified in ITU-T H.323 version 4.

---

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database at http://www.itu.int/ITU-T/ipr/.

ITU  2013

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

-

CONTENTS

Page

1	Scope	7

2	Normative references	7

3	Abbreviations and acronyms	7

4	Interworking description	8

4.1	Messages and signalling	8

4.2	ISUP-to-H.225 Ingress Interworking	9

4.3	H.225-to-ISUP Egress Interworking	10

5	H.225.0 Generic Data Usage	12

6	Description of ASN.1 types and fields	12

Annex A	14

Appendix I	16

I.1	Introduction	16

I.2	Informative references	16

I.3	Coding the RegionalParameters Sequence	16

ITU-T Recommendation H.460.2

Number Portability interworking between H.323 and SCN networks

# 1	Scope

This Recommendation specifies the Number Portability interworking procedures between H.323 and SCN networks.

# 2	Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T H.323 version 4 (2009), Packet-based multimedia communications systems.

[2]	ITU-T H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[3]	ITU-T Q.769.1 (1999), Signalling System No. 7  ISDN user part enhancements for the support of number portability.

[4]	ITU-T Q-series Supplement 3 (1998), Number Portability – Scope and capability set 1 architecture. { Editor’s Note: Withdrawn! }

[5]	ITU-T Q-series Supplement 4 (1998), Number Portability – Capability set 1 requirements for service provider portability (All call query and Onward routing). { Editor’s Note: Withdrawn! }

[6]	ITU-T Q-series Supplement 5 (1999), Number Portability – Capability set 2 requirements for service provider portability (Query on release and Dropback). { Editor’s Note: Withdrawn! }

[7]	ITU-T X.680 (2008), Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation.

[8]	ITU-T X.681 (2008), Information technology – Abstract Syntax Notation One (ASN.1): Information object specification.

[9]	ITU-T X.691 (2008), Information technology – ASN.1 encoding rules – Specification of Packed Encoding Rules (PER).

[10]	ITU-T T.35 (2000), Procedure for the allocation of ITU-T defined codes for non-standard facilities.

# 3	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

ARJ		RAS Admission Reject

ARQ		RAS Admission Request

ASN.1		Abstract Syntax Notation one

DN		Directory (or Dialled) Number

IAM		Initial Address Message

ISUP		Integrated Services Digital Network User Part

LRJ		RAS Location Reject

LRQ		RAS Location Request

NoA		Nature of Address indicator

NP 		Number Portability

NPSI		Number Portability Status Indicator

NRN		Network Routing Number

PSTN 		Public Switched Telephony Network

RAS		Registration, Admission, and Status

SCN		Switched Circuit Network

TON		Type Of Number

# 4	Interworking description

Number Portability in the SCN is specified in [3]-[6].

There are several schemes in ITU-T Q.769.1 [3] used for transporting the parameter.

•	Separate Directory Number Addressing Method: clause 6.1/Q.769.1.

•	Separate Network Routing Number Addressing Method: Annex B/Q.769.1.

•	Concatenated Addressing Method: Annex A/Q.769.1.

If the query returns no NRN, then the Called Party Number parameter in the IAM message contains the Directory Number (dialled-digits). The IAM may contain the Number Portability Forward Information parameter (see Annex E/Q.769.1).

The following information needs to be conveyed across H.323 network to support Number Portability.

1)	Network Routing Number (NRN).

2)	Directory Number (DN).

3)	NP Status indicator (NPSI).

4)	Regional Parameters (This parameter may be used to carry national specific or Regional Number Portability standards specific information).

## 4.1	Messages and signalling

It is proposed that the NP parameters listed above be transported as follows in the H.225.0/RAS messages using the generic extensibility framework:

1)	The genericData parameter (containing the NumberPortabilityInfo sequence) should be used in the H.225.0 Call Signalling (Setup) or RAS (ARQ and LRQ) messages. The genericData parameter identifies Number Portability feature and contains the NumberPortabilityData. The NumberPortabilityInfo of the NumberPortabilityData contains the appropriate fields populated.

2)	The ISUP NRN is always carried in the H.225 Called Party Number IE with its TypeOfNumber (TON) set to National (significant) Number.

3)	A new NumberPortabilityInfo ASN.1 definition is introduced to transport the directory number (DN), NPSI, NRN and regionalParams parameters.

4)	A concatenated ISUP NRN and DN are carried concatenated in the Called Party Number IE and the NumberPortabilityInfo parameter.

5)	When a Gatekeeper receives an ARQ or LRQ and determines that the destination number is ported out of the network and it may wish to invoke number portability Query on Release (QoR) procedures (as specified in Annex C/Q.769.1). In such cases, the Gatekeeper must respond with ARJ or LRJ that contains a reject reason of genericDataReason. The Gatekeeper should include the genericData of the ARJ/LRJ that contains the NumberPortabilityGenericData with the numberPortabilityRejectReason. The numberPortabilityRejectReason now will have a value of qorPortedNumber (=1). This maps to the ISUP release cause value = #14 (QoR: ported number) as specified in Addendum 1/Q.850.

## 4.2	ISUP-to-H.225 Ingress Interworking

| NP Parameters in ISUP → | H.225 and NumberPortabilityInfo (inside genericData) Parameters in Setup, ARQ, LRQ → |
| --- | --- |
| NPSI | NumberPortabilityInfo.numberPortabilityData.addressTranslated (if NPSI = query not done, it is not necessary to create a NumberPortabilityInfo parameter; it will be necessary for some downstream switch to do a query) (if NPSI = query done, set addressTranslated = TRUE) |
| Network Routing Number (NRN) in Network Routing Number parameter, DN in Called Party Number parameter | a) The NRN is mapped as follows: Called Party Number (Setup) or destinationAddress (ARQ, LRQ messages) (TON=National(significant)Number) and NumberPortabilityInfo.numberPortabilityData.routingAddress (Setup, ARQ, LRQ messages). (portabilityTypeOfNumber = routingNumber) b) The DN is mapped as follows: NumberPortabilityInfo.numberPortabilityData.portedAddress |
| Directory number (DN) in Called Directory Number parameter, NRN in Called Party Number parameter | a) The NRN is mapped as follows: Called Party Number (Setup) or destinationAddress (ARQ, LRQ messages) (TON=National(significant)Number) and NumberPortabilityInfo.numberPortabilityData.routingAddress (Setup, ARQ, LRQ messages). (portabilityTypeOfNumber = routingNumber) b) The DN is mapped as follows: NumberPortabilityInfo.numberPortabilityData.portedAddress |

| Concatenated NRN + DN in Called Party Number parameter | Called Party Number (Setup) or destinationAddress (ARQ,LRQ) (TON = National(significant)Number) and NumberPortabilityInfo.numberPortabilityData.routingAddress (Setup, ARQ, LRQ). (portabilityTypeOfNumber = concatenatedNumber) |
| --- | --- |
| Regional Parameters | NumberPortabilityInfo.numberPortabilityData.regionalParams |

If an NPSI indicating that a query has been done is present in the IAM, or if an NRN is present in the IAM, the genericData parameter (containing the NumberPortabilityInfo sequence) should be used in the H.225.0 Call Signalling (Setup) or RAS (ARQ and LRQ) messages. The genericData parameter identifies Number Portability feature and contains the NumberPortabilityData. The NumberPortabilityInfo of the NumberPortabilityData contains the appropriate fields populated as shown in the table above.

The portabilityTypeOfNumber field indicates the TypeOfNumber (TON) of the NRN or DN present in the IAM message.

If the NoA of the Routing Number in the IAM message is set to either Routing Number or Concatenated Number, then the routingAddress in NumberPortabilityInfo shall be populated with the Routing Number and its typeOfAddress is set to routingNumber or concatenatedNumber respectively.

If the NoA of the Routing Number in the IAM message is set to National(significant)Number, then the interworking function may not populate the routingAddress in NumberPortabilityInfo.

If the Gatekeeper performs an NP query, it shall provide the NumberPortabilityInfo as a genericData parameter in ACF and LCF messages as specified above. The ingress interworking function, receiving this info in ACF shall send it in Setup message and the egress interworking function shall send it in the IAM message.

NOTE  Egress interworking is specified in 4.3.

If the ingress interworking function sends the NumberPortabilityInfo in the ARQ message and the Gatekeeper does not return it in the ACF message, then the H.225.0 Setup message shall contain the NumberPortabilityInfo as sent in the ARQ message.

## 4.3	H.225-to-ISUP Egress Interworking

| H.225 and NumberPortabilityInfo (inside genericData) parameters in Setup. | Parameters to compose IAM according to the chosen addressing method (see ITU-T Q.769.1) |
| --- | --- |
| NumberPortabilityInfo.numberPortabilityData.addressTranslated | NPSI (if supported) |
| NumberPortabilityInfo.numberPortabilityData.routingAddress | Network Routing Number |
| NumberPortabilityInfo.numberPortabilityData.portedAddress | Directory Number (DN) |
| NumberPortabilityInfo.numberPortabilityData.routingAddress (portabilityTypeOfNumber = concatenatedNumber) | Concatenated NRN, DN |
| NumberPortabilityInfo.numberPortabilityData.regionalParams | Regional Parameters |

The egress interworking function shall compose the IAM message using the addressing method in operation at that interface using the LNP parameters that are provided by the Setup message.

# 5	H.225.0 Generic Data Usage

Generic extensibility framework shall be used to specify the Number Portability feature interworking with RAS and Annex G/H.225.0 as described below.

Data Specification

NumberPortabilityID: Identifies the Number Portability feature using the standard field of GenericIdentifier with a unique integer value.

| Generic Extensibility Type | Fields | Field name | Value |
| --- | --- | --- | --- |
| GenericIdentifier |  | standard | 2 |

NumberPortabilityData: This is the data sent in ARQ/LRQ/H.225.0 Setup messages and Annex G/H.225.0 Access Request messages to notify or communicate the number portability information. It is an EnumeratedParameter with unique identification using the standard field and the content is a raw field consisting of the ASN.1 PER encoded NumberPortablityInfo as specified in the ASN.1 notation in Annex A.

| Generic Extensibility Type | Fields | Field name | Value |
| --- | --- | --- | --- |
| EnumeratedParameter |  |  |  |
| GenericIdentifier | id | standard | 1 |
| Contents | content | raw | ASN.1 PER encoding of the NumberPortabilityInfo |
| Parameter Cardinality |  |  | Once and only Once |

NumberPortabilityDescriptor: This is a FeatureDescriptor used for feature negotiation using the generic extensibility framework.

| Generic Extensibility Type | Fields | Field name | Value |
| --- | --- | --- | --- |
| GenericIdentifier | id | standard | NumberPortabilityID |

# 6	Description of ASN.1 types and fields

NumberPortabilityInfo – Allows specification of parameters needed by ISUP-H.323 Number Portability interworking functions. Currently, the fields are used in the context of the dialled number or E.164 address, while in future other AliasAddress fields such as the email-ID, h323-ID may be used in portability services.

numberPortabilityRejectReason – Identifies the reason for rejection of ARJ/LRJ to invoke cases such as Number Portability Query On Release.

addressTranslated – This field is set to TRUE, if an NP query has been made.

portedAddress – This field contains the original Address (E.164 number or dialled digits or the AliasAddress) in the context of Number Portability.

routingAddress – This field contains the routing Address (E.164 number or dialled digits or the AliasAddress) in the context of Number Portability. This may be populated only when the NoA of NRN in IAM message is set to routing number or concatenated number. In other cases, it may optionally be used. If this field is present, the egress interworking function shall use it to compose the NRN in the IAM message.

regionalParams – This field may be used to carry Regional Number Portability Standards specific or National specific Information. The regionalData field contains the country specific information encoded as per the regional standard.

typeOfAddress – If the typeOfAddress field is present in the fields of type PortabilityAddress, it qualifies the typeOfNumber present in the AliasAddress of PortabilityAddress including email-ID, h323-ID etc. The typeOfAddress field, if present, has precedence over typeOfNumber, if any, present in the aliasAddress.

portabilityTypeOfNumber – Identifies the type of NP used, such as the portedNumber or the routingNumber or concatenatedNumber.

publicTypeOfNumber – This field identifies the numbering plan in public networks/E.164 address. This field shall be chosen when the number is not a routing or ported number.

partyTypeOfNumber – This field identifies the numbering plan in private networks/numbers. This field shall be chosen when the number is not a routing or ported number.

Annex A

H.460.2 Message Syntax (ASN.1)

This annex shows the ASN.1 syntax for the Number Portability feature.

The t35CountryCode element shall identify the country, as described in Annex A/T.35. The t35Extension element shall contain a country code extension that is assigned nationally, unless the t35CountryCode is binary "1111 1111", in which case this field shall contain the country code found in Annex B/T.35. VariantIdentifier is assigned nationally to identify specific national variants.

In order to comply with this Recommendation, a description of the national use and how to encode that usage in the regionalParams field should be provided to ITU-T SG 16. The description of the national use will be published as Appendix I.

NUMBER-PORTABILITY DEFINITIONS AUTOMATIC TAGS ::=
BEGIN

IMPORTS

PublicTypeOfNumber,

PrivateTypeOfNumber,

AliasAddress

FROM H323-MESSAGES

NumberPortabilityInfo ::= CHOICE

{

numberPortabilityRejectReason		NumberPortabilityRejectReason,

numberPortabilityData SEQUENCE

{

addressTranslated		NULL OPTIONAL,

portedAddress 			PortabilityAddress OPTIONAL,

routingAddress			PortabilityAddress OPTIONAL,

regionalParams			RegionalParameters OPTIONAL,

...

},

...

}

NumberPortabilityRejectReason ::= CHOICE

{

unspecified			NULL,

qorPortedNumber		NULL,

...

}

PortabilityAddress ::= SEQUENCE

{

aliasAddress 			AliasAddress,

typeOfAddress 			NumberPortabilityTypeOfNumber OPTIONAL,

...

}

NumberPortabilityTypeOfNumber ::= CHOICE

{

publicTypeOfNumber			PublicTypeOfNumber,

privateTypeOfNumber		PrivateTypeOfNumber,

portabilityTypeOfNumber		PortabilityTypeOfNumber,

...

}

PortabilityTypeOfNumber ::= CHOICE

{

portedNumber			NULL,

routingNumber			NULL,

concatenatedNumber		NULL,

...

}

RegionalParameters ::= SEQUENCE

{

t35CountryCode			INTEGER(0..255),

t35Extension			INTEGER(0..255),

variantIdentifier		INTEGER(1..255) OPTIONAL,

regionalData			OCTET STRING,

...

}

END

Appendix I

Number Portability within the United States

## I.1	Introduction

The United States, as with many other countries, has a variant of the ITU-T SS No. 7 specifications. As a result, it is not possible to perform number portability within the United States without also carrying additional information within the regionalParams sequence.

## I.2	Informative references

T1.113-1995, Signalling System No. 7, ISDN User Part, January 1995.

## I.3	Coding the RegionalParameters Sequence

Systems that perform number portability shall include the regionalParams element of the numberPortabilityData sequence. The variantIdentifier shall be omitted. The regionalData element shall contain the Jurisdiction Information Parameter (JIP) as specified in T1.113-1995.

Endpoints shall assume that if the variantIdentifier is present, the coding of the field is not in accordance with this appendix.

__________________

____________________