| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4326 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE, China Telecom |  |  |  |
| Title: | H.VSMarch: Proposed updates on the function of stream adjustment |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes some updates on the capability of VAU to support stream adjusting dynamically in mobile visual surveillance system to the ITU-T Rec. H.VSMarch.

# Introduction

The architecture of mobile visual surveillance system introduces a new unit VAU which implements the communication between M_CU and the units defined in ITU-T Rec. H.626. For real-time Audio/Video acquisition, VAU transfers the media stream from MDU and/or PU to M_CU. Though mobile network is not always in stable status and will lead to impact on transmission such as packet loss, the video/audio quality will be affected in result. So there is requirement for VAU to support stream adjusting dynamically according to the network status.

# Discussion

Firstly, in clause 7.3.2, it needs to give some description on the function components of VAU.

Secondly, to guarantee decent quality of video/audio in M_CU when the mobile network environment is bad, VAU can process the stream delivered from MDU and/or PU before it is transferred to M_CU: VAU adjusts the key frames interval and bit rate of the media stream, according to the packet loss rate reported by M_CU. And then VAU transfers the adjusted media stream to M_CU. This mechanism provides an approach to implement the adaptation adjustment of media stream based on network status:

- When the status of mobile network becomes worse, VAU reduces the key frames interval to increase the number of key frames and decrease the loss rate of key frames; besides, VAU decreases the bit rate accordingly to adapt the network status.

- When the status of mobile network turns better, VAU increase the key frames interval gradually to normal value and increases the bit rate appropriately based on the current bandwidth.

# Conclusion

# It’s proposed to accept the following content.

Start proposed content

# Functional Architecture

## Functional Model

## Functional Architecture Framework

The high-level functional components are described in Figure 1.

Figure 7-1: Components for mobile visual surveillance

- M_CU: Mobile Customer Unit, such as software clients running on mobile phones used by the visual surveillance end users.

- MSP: Mobile Service Portal, MSP provides the whole access portal for mobile surveillance users, including user authentication portal, PU list portal, etc. MSP provides service management functions for the end users through communication with SCU, such as user authentication, information querying, programme for surveillance content, synchronization of information with SCU, etc.

- VAU: Video Access Unit, VAU is the key unit to implement the communication between M_CU and the units defined in ITU-T Rec. H.626. The main functions of VAU include obtaining management information from SCU, processing the video acquisition request from M_CU, creating connection with MDU and/or PU, distributing and transcoding multimedia data, transferring commands of PTZ control, and dynamically adjusting the media stream according to network status, etc.

## Functional Entities

### MSP

Mobile Service Portal (MSP) is the application server for mobile visual surveillance system.

MSP is used to communicate with SCU to provide service management for M_CU, such as authentication, PU list search and obtaining, video data search, information synchronization with SCU, etc.

### VAU

Video Access Unit (VAU) is the key unit to implement the control between mobile terminals and other units defined in Recommendation ITU-T H.626.The main functions of VAU include: Service control function, media control function (such as PTZ control command transmit), media processing function (such as multimedia transcoding), configuration management function.

Figure 7-2: Function components of VAU

Service Control Function:

- Initiate, maintain and release the connection with SCU.

- Process service requests from M_CU, forward them to SCU and respond to M_CU.

- Acquire media stream from MDU and/or PU.

Media Control Function:

- Receive media stream from MDU and/or PU according to M_CU’s request.

- Transmit or forward media stream to M_CU.

- Process control and report indications from M_CU.

Media Processing Function:

- Media stream transcoding before transferring stream from MDU and/or PU to M_CU according to the ability of the M_CU.

- Adjust the key frames interval and bit rate of media stream according to the packet loss rate reported by M_CU before VAU transfers the stream from MDU and/or PU to M_CU (in addition, when packet loss rate increases, VAU decrease key frames interval and bitrate, and when the packet loss rate decreases, VAU increase the key frame interval and bitrates).

Configuration Management Function:

- Manage the overall configuration in VAU, including transmission, decoding, transcoding, and other control of stream.

7.3.3 M_CU

The M_CU is the functional entity within mobile VS system, such as mobile phones used by the visual surveillance end users. It is used to present multimedia information (such as audio, video, image, alarm signal, etc.) to the end user. It implements the following functions:

- Media stream receiving, decoding and playback. Media stream packet loss rate reporting to VAU periodically (e.g. if RTP is used for media stream transmitting, M_CU reports the loss rate by RTCP protocol).

- PU list, real-time image/video presentation.

- Console interface for end user to operate the VS system (such as PTZ control).

## 7.4	Reference Points

….

### 7.4.3	Reference Point Mv: M_CU-VAU

The reference point Mv is between the M_CU and the VAU.

The Reference Point Mv is used to deliver the related media operations between the M_CU and the VAU.

- Sending media request and receiving response by M_CU to VAU.

- Delivering control instruction of PTZ and receiving response by M_CU to VAU.

- Reporting packet loss rate of media stream to VAU periodically.

End proposed content

___________________________