| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 12 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 889 (GEN/12) |
|  |  | English only Original: English |
| Question(s): | 13/12 | Geneva, 29 May - 7 June 2012 |
| TD |  |  |
| Source: | Chair WP3/12 |  |
| Title: | Specification of SLR and RLR for Telepresence Systems |  |

This document provides information on how to specify sending and receiving loudness ratings (SLR, RLR) for Telepresence Systems. It is based on material extracted from Rec. P.300 (11/2001) “Transmission performance of group audio terminals (GATs)”.

#### Sending Loudness Rating

The acoustic pick-up port of the microphone shall be placed at the centre of the Reference Sphere in an anechoic environment and the Artificial Mouth shall be placed with the lip plane tangential to the Sphere. The reference axis of the Mouth shall coincide with the x-axis of the Sphere, while the microphone shall be oriented as specified by the manufacturer. In case of microphones intended to pick up the sounds coming from multiple directions (either omni-directional or steered devices), the extreme incidence angles on the horizontal plane as stated by the manufacturer shall be tested, as well as an intermediate incidence direction. The testing angles actually used for measurements shall be stated in the test report.

The nominal value of Sending Loudness Rating (SLR) shall be:

SLR = (+ 13–Fs) dB

where Fs is determined based on the following considerations:

In order to take into account the difference between the reference test positioning and the actual microphone-speaker operating distance (ds), for which the terminal is adjusted, the following correction factor Fs is defined:

(ds in meters)

#### Receiving Loudness Rating

The nominal value of Receiving Loudness Rating (RLR) shall be:

RLR = (+ 5–Fr) dB

where Fr is determined based on the following considerations:

Each loudspeaker shall be placed in turn at the centre of the Reference Sphere, with its main axis coincident with the x-axis and its outer surface containing the z-axis. The measurement microphone shall be placed on the x-axis, its acoustic centre being at the intersection with the Reference Sphere.

In order to take into account the difference between the reference test positioning and the actual loudspeaker-listener operating distance (dr) for which the terminal is adjusted, the following correction factor Fr is defined:

(dr in metres)

___________________