|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 12 – C 335 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | May 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3, 5, 6/12 |  |  |  |
| STUDY GROUP 12 – CONTRIBUTION 335 |  |  |  |  |  |
| Source: |  | France Telecom Orange |  |  |  |
| Title: |  | Proposed measurements of audio parameters for Telepresence systems |  |  |  |

Summary

ITU-T Q5/16 is currently specifying the requirements for Telepresence systems, defined as “interactive audio-visual communications experience between remote locations, ...”. To guarantee audio performance and interoperability, these systems need some calibration: audio levels, frequency response, ...

This contribution lists the audio parameters that should be measured, the possible values, and proposes some procedures, standardized when existing, to achieve the measurements.

## Background

The level alignments of the terminal audio chains can strongly influence the effective performance of a videoconference terminal. The document ITU-T H.TPS-AV [1] discussed in Q5/16 defines some audio parameters and recommends associated values to ensure audio performance: network levels (RLR/SLR), frequency responses. At last Q5/16 meeting, a new parameter characterizing the room gain (i.e. the coupling loss between loudspeakers and microphones), was proposed and added ([1] [2]). As for now, no procedure has already been defined to measure these parameters, although only reliable and repeatable procedures can guaranty the optimal performance.

This document proposes some procedures to measure reliably the audio parameters, especially the audio alignment. In addition, this document lists good practises for the installation of Telepresence system in order to maximize signal to noise ratio, signal to reverberation ratio, naturalness of the audio-visual scene (participants voices’ are perceived at the right place), comfort of the participant. Some advice about the microphones and loudspeakers quality will be proposed. Finally, procedures for the maintenance of the system will be described.

## Discussion

## Telepresence alignment

The following criteria and audio alignment procedure have been inspired from the normalized ones defined by the European Telecommunication Standard for Integrated Service Digital Network (ISDN); Conference Service; Conference terminal video and audio characteristics (August 1996, Draft prETS 300 807, Source : ETSI TC-TE, ICS: 33.080, Reference : DE/TE-04043).

## Network Audio level

The following is currently proposed in the document ITU-T H.TPS-AV [1]:

Telepresence systems shall send audio to other Telepresence systems with an average audio level of AudioLevel -31 dBov, as measured by Rec. P.56.

When a telepresence system connects to non-telepresence devices, it should use the AudioLevel expected by that device.  If this is unknown, it should use the Telepresence AudioLevel.

## Room Audio levels

The purposes of the audio alignment for the sound recording are the following:

- Set the microphone preamplifiers gain value in order to use the whole dynamic of the AD converter without saturating it.

- Set the microphone preamplifiers gain value of all the microphones in all the rooms in communication in such way that the same sound pressure levels at any conferee position create the same electrical value before the AD conversion.

The following alignment process is proposed:

In order to be reproducible and the most representative, the calibration should be realized using a mannequin complying with ITU-T Recommendation (HATS for instance as described in P.581). The sound source (the artificial mouth of the mannequin) should be placed at 10 cm from the border of the conference table, 30 cm above the table (average normal position of a conferee's mouth). The sound source should point in direction of the screen (average normal direction of a conferee's mouth). The microphones of the system on calibration should be positioned in the conference room as in normal use.

The signal used to calibrate the system should be speech-shaped. Moreover, in order to be representative of audio bandwidth, we propose to use full-band signals, that is to say a sampling frequency superior or equal to 44.1 kHz. A good candidate could be the new ITU P.501 signal, sampled at 48 kHz. Even if there is some incertitude about the ability of HATS to reproduce average human properties above 10 kHz, ETSI STQ agreed that HATS could be used with such signal to calibrate systems (see document STQ(11)0161r1 [3]).

Using the artificial mouth, the level should be, in free field condition, -4.7 dB(Pa) at the MRP (Mount Reference Point).

With such calibration, the preamplifier gain value of each microphone should be adjusted in order to achieve an output level of -31 dBov on the transmission channel corresponding to the conferee position being tested.

The purposes of the audio alignment for the sound restitution are the following:

- Set the loudspeakers gain value in order to obtain a comfortable sound level at the conferee's positions (not too low and not too loud).

- Set the loudspeakers gain value so that the same electrical value received from the distant rooms after DA conversion generate the same sound pressure level at conferee's positions.

The following alignment process is proposed:

When the speech-shaped noise P.501 is fed to the input of the amplifier at a level of -31 dBov, the amplifier gain value should be adjusted in order to achieve an average sound pressure level greater than or equal to 54 dB(A) at every conferee's position. Notice that the measurement points correspond to the previous positions of the sound source during the alignment of the microphones (10 cm from the border of the conference table, 30 cm above the table).

## Frequency responses

The draft standard Draft ETSI TS 102 925 [4] gives the following recommendations:

The microphones should have a flat frequency response from 50 Hz up to 18 kHz with ± 4 dB Mean tolerance.

The loudspeakers should have a flat frequency response from 50 Hz up to 18 kHz with ± 5 dB Mean tolerance.

Some equalization filters may be implemented in order to guarantee the frequency responses.

Note: above and below the spectrum bounds (50 and 18kHz), the templates follow a decreasing 6dB/octave slope.

## Microphones quality and placement

The microphone acoustic equivalent noise level should be around 30 dB(A).

The microphones should be placed in such way to avoid noise transmission by solid vibration.

The microphones should be placed, if possible, in such way that they cannot be hit, covered or moved by a conferee.

## Coupling

The weightedMicrophoneLoudspeakerCouplingLoss parameter measures the minimum coupling loss over each pair of loudspeaker/microphone. Even if this value is still under discussion, we propose the following measuring procedure:

- calibrate the system following §‎2.3 and §‎2.4 (including equalization filters)

- for each loudspeaker/microphone pair:

  - diffuse the P.501 signal through the loudspeaker, calibrated at -31dB numeric level

  - simultaneously record the signal picked-up by the microphone

  - compute and store the coupling loss in the same way as done in the B.4/G.122 [5] in the [100Hz-16kHz] frequency range

- The weightedMicrophoneLoudspeakerCouplingLoss is derived as the minimum coupling loss over all the LS/microphone pairs.

Note: during this procedure, every signal processing module such as noise suppressor, echo canceller, automatic gain control, must be switched off (except for the equalization filters if any).

## Room maintenance

Room maintenance shall be performed periodically to check the system is still operational and conforms to the requirements. Basically, it should be checked that any degradation, drift, deficiency of a microphone/loudspeaker/amplifier has occurred. The position and the direction of loudspeaker and microphones should be also verified, since it impacts the spatialisation (at least for rendering, and also for scene capture when using directional microphones), and the coupling (and so the interactivity of the system).

One simple way achieve such maintenance is to periodically measure the coupling loss parameter of each pair of microphone/loudspeaker. Indeed, any of the listed deficiencies at least impacts the coupling value of one pair. The Source proposes to follow a 2-step procedure:

The 1st step of the procedure, achieved during room calibration (see §‎2.6), consists in measuring and storing the coupling values between each loudspeaker/microphone pair or decoder output/encoder input pair. Then, a periodic comparison between the current values and the initial ones would help in diagnosing any deficiency.

This verification would help in guaranteeing a given QoS for the end-user, and a constant user-experience.

## Conclusion

This contribution proposes some audio calibration procedures (RLR/SLR, frequency response, coupling) for Telepresence systems. Currently, the procedures do not apply to super-wideband/full-band signals (32/44.1/48 kHz frequency sampling) as required by ITU Q5/16. It is then needed to progress on these topics in order to supply Q5/16 with reliable and standardized methods.

# References

[1] TD 768R1/WP2-16, “H.TPS-AV “Audio/Video Parameters for Telepresence systems” (New): Output Draft (Ed.0.7)”, source: Editor H.TPS-AV, Geneva, 30 April – 11 May 2012

[2] COM 16 – LS 300 – E, “Reply LS to SG 12 on proposed audio-visual performance metrics for Telepresence Systems”, source: ITU-T SG 16, Geneva, 30 April - 11 May 2012

[3] ETSI STQ(11)0161r1 STQ#38 (Vienna) Meeting report

[4] ETSI STQ(12)39_038r1 Draft ETSI TS 102 925 “Transmission requirements for Superwideband/Fullband handsfree and conferencing terminals from a QoS perspective as perceived by the user”

[5] Rec. ITU-T G.122 "Influence of national systems on stability and talker echo in international connections",  March 1993

_________________