|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 12 – C 340 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | May 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 18/12 |  |  |  |
| STUDY GROUP 12 – CONTRIBUTION 340 |  |  |  |  |  |
| Source: |  | Deutsche Telekom AG |  |  |  |
| Title: |  | Methodology for the assessment of audiovisual quality for simulated video calls |  |  |  |

Summary

This contribution proposes an experimental methodology for assessing the quality of simulated video calls. This approach is based on a methodology that was previously developed for speech telephony and was standardized by ETSI. Results proving the accuracy of the method for video calls are presented in the document. This contribution provides a basis for discussion concerning the potential definition of a new ITU standard for assessment and prediction of the overall quality of entire video calls.

1. Introduction

Voice and video quality perception is an important topic for packet-based networks. For the evaluation of quality, subjective test methodologies, such as P.800 or P.910, but also objective models, such as P.863, G.107, J.341, G.1070, are recommended by ITU-T. However, both of the approaches were designed with the focus on assessment of short signal samples (typical duration of 8 to 12 s). This test duration is insufficient when the quality of entire voice and video calls has to be addressed. Effects such as how users remember the quality (i.e. the "recency effect") have to be taken into consideration. So far, only one methodology that is focused on listening-quality of voice calls has been standardized in ETSI TR 102 506 (STQ). In this contribution, we present how that methodology was applied for the estimation of video-telephony calls. The methodology is summarized, and some examples of test scenarios are presented. Finally, the judgements of video call quality are compared with quality estimates that were obtained according to the modelling approach proposed by ETSI. Pearson correlation above 0.96 and root mean square error (RMSE) below 0.27 for two data sets were achieved. The presented methodology can therefore be efficiently used for assessing the quality of video calls based the quality of short samples.

2. Description of the methodology

The call quality experiment presented in this contribution was based on a procedure defined in ETSI TR 102 506 v.1.2.1 [3], but here applied to audiovisual transmission scenarios. The test consisted of emulated phone calls, whereas call emulation consisted of five samples presented in a meaningful order to test participants.  After each of the first four samples, subjects were asked to orally answer a content-related question.  After the last sample, the audiovisual quality of the emulated video call was judged on an 11-point MOS scale. The test samples and pauses for answering the questions were approx. 9 s long, and an emulated call took app. 90 seconds.

2.1 Experimental methodology

Two subjective audiovisual quality experiments were carried out in order to assess the dialog-end judgments based on individual short samples.

In the first experiment, the quality of short audiovisual samples was evaluated. The content of these samples was classical "head and shoulder" video-telephony scene with a fixed background. In each sample, a speaker uttered one or two sentences related to a certain task, such as to making an appointment with a doctor, ordering kitchen equipment, or reporting from a party. Each sample was about 9 s of length in order to be in line with ITU-T Rec. P.911 [2] for the subjective evaluation of audiovisual stimuli. After each video clip, the test participants are asked to rate the audiovisual, the video and audio qualities in that specific order, as instructed in ITU-T Rec. P.911. For that purpose, an 11-point ACR scale was defined in ITU-T Rec. P.910 [1] was used.

In the second experiment, the quality of simulated videophone conversations was evaluated. A simulated videophone conversation was constructed as a set of five consecutive couples of short video clips used in the first experiment. The samples were linked by the content. Participants are asked to watch each clip and then to verbally answer a question about the audiovisual content of the clip. The question could be related to the auditory content only, to the video content only, or to the audiovisual one, that is to say, a question that could be answered by using clues from both visual and auditory channels. The questions asked to the participants after each sample of the simulated dialogs and the corresponding answers were presented on the computer screen after each sample. They were directly related to the audiovisual content of the short samples and could easily be answered. The number of multiple choice answer options varied between 3 and 5. As explained in [4], the questions were introduced to let people pay attention to the content before rating the quality.

For each sample, we used a set of different questions in order to prevent people from memorizing the answers. The position of the right answer was also randomly changed in order to avoid participants answering mechanically. The video clips and the time period allowed to answer each question were of the same length (9 s), leading to a total duration of 90 s. At the end of each simulated conversation, test participants rated the audiovisual quality of the entire dialog using the same scale as the first experiment.

Participants had to watch 60 stimuli composed of 15 quality profiles applied on 4 speakers. As each profile had a total duration of 90 s, the test was about 90 min long, thus conducted in two sessions of 45 min, with a break in between (to limit the fatigue).

The test participants were thus asked to show up twice, i.e. once for each experiment. All of them received monetary compensation and were not experienced assessors.

2.2	Quality profiles

In order to obtain dialogs with different temporal quality profiles, we concatenated short samples with different targeted levels of quality to form a so-called “quality profile”. We used the types of temporal quality profiles defined by ETSI STQ [3], see Figure 1. The profiles contained constant good, middle and poor qualities (profiles 1 to 3), constantly rising and constantly decreasing quality (profiles 4 to 5), constantly good quality with a single strong degradation at the beginning, in the middle or in the end (profiles 6 to 8), constantly good with a moderate degradation in the middle (profile 9), and constantly good but falling in the end or rising at the beginning (profiles 10 and 11). Those profiles are said to be symmetric in the sense that both audio and video are degraded with a comparable relative perceptual intensity. We also combined the ETSI profiles in an asymmetric way. We had a constantly rising video quality combined with constantly poor audio (profile 12) and the other way round (profile 15). On the opposite, we combined constantly rising video quality with constantly good audio quality (profile 13) and the other way round (profile 14). Details of the asymmetric profiles are presented in Figure 2.

Figure 1: Schematic representation of symmetric quality profiles

Figure 2: Schematic representation of asymmetric quality profiles

2.3 Specifications of the audiovisual stimuli

Audiovisual clips were composed of at least four speakers balanced in gender, comprising different scene backgrounds and conversation topics. Those clips involved the video channel by the means of visual cues (i.e. showing objects to the camera, pointing dates on a wall calendar, body gestures), so that the test participants had to pay attention to the video channel. We used four different topics (kitchen purchase, car rental, birthday party storytelling and making a doctor appointment). Each topic was assigned to a single speaker and each conversation was composed of 10 sentences cut in two sets of five consecutive samples, leading to a final set of 8 source stimuli sequences.

Each sentence part of a simulated conversation (short sample) was recorded in raw format (uncompressed planar YUV 4:2:0) with a VGA (640x480) resolution and a frame rate of 25 frames per second. The audio recordings were made using a sampling frequency of 16 kHz and 8 bit quantization.

Once recorded, the samples were degraded with different levels of impairments. Videos were encoded using the H.264 codec with different encoding bit rates. Additional information loss was introduced by dropping data packets of streamed video with a fixed packet loss rate using a network emulator with a random loss distribution. For the audio channel, encoding degradations were introduced by using different speech codecs: A narrowband (GSM-EFR) and a wideband codec (AMR-WB). Network frame loss was simulated using the same packet loss emulation tool. Each sample was artificially degraded with five equidistant targeted levels of quality - covering the entire quality range - for both the audio and video channels leading to 25 possible predefined combinations of audio and video quality levels for each sample.

3. Experimental results

In this section, the performances of two models for predicting the dialog-final judgments are presented. The first approach referred as “average” consists of predicting the call-final quality of the audiovisual call by using the average of the audiovisual scores obtained for each short sample which is constituent of that call. The second approach was standardized by the ETSI STQ [3] and consists of taking a weighted average of the individual scores as an estimate of the end-quality. The weights take into account the temporal location of the impairments in relationship to the call-final instance (samples being close to the judgment point-in-time receive a higher weighting than long-ago samples), as well as the magnitude of those impairments.

Those two models originally developed for speech-only calls nevertheless produce high correlation when applied to audiovisual samples of a simulated video call, as shown below in Table 1.

Table 1: Performances of temporal integration models for predicting dialog-final judgments for two different datasets. Two different modelling approaches are compared, i.e. simple averaging and the ETSI model.

|  | Dataset I |  | Dataset II |  |
| --- | --- | --- | --- | --- |
|  | Correlation | RMSE | Correlation | RMSE |
| average | 0.93 | 0.29 | 0.94 | 0.24 |
| ETSI | 0.96 | 0.27 | 0.97 | 0.15 |

4. Conclusion

This contribution addressed the assessment and prediction of the overall quality of video telephony calls in packet-based networks. It was presented how the overall video call quality can be tested based on a modified version of the methodology described in ETSI TR 102 506 (STQ), that has been standardized for the assessment of listening-only quality of telephone calls. Based on the observed experimental results this contribution is intended to trigger a discussion on a definition of a new ITU recommendation for the assessment and prediction of the overall quality of entire video calls.

Bibliography

[1]	ITU-T Recommendation P.910, “Subjective video quality assessment methods for multimedia applications,” International Telecommunication Union, Geneva, Sep. 1998.

[2]	ITU-T Recommendation P.911, “Subjective audiovisual quality assessment methods for multimedia applications,” International Telecommunication Union, Geneva, Dec. 1998.

[3]	ETSI TR 102 506: Speech Processing, Transmission and Quality Aspects (STQ); “Estimating Speech Quality per Call”.

[4]	B. Weiss, S. Möller, A. Raake, J. Berger, and R. Ullmann, “Modeling Call Quality for Time-Varying Transmission Characteristics Using Simulated Conversational Structures,” Acta Acustica united with Acustica, vol. 95, no. 6, pp. 1140–1151, Dec. 2009.

_______________