| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4334 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | H.TPS-AV: AudioLevel value for Telepresence Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Polycom proposes changing the default AudioLevel parameter value to match the nominal level used by the ITU-T to characterize codec performance.

# Introduction

Current telepresence and videoconferencing systems transmit audio at a variety of different levels – resulting in level mismatches which in some cases exceed 10 db.

Although there is agreement that standardising a common audio level would improve interoperability, Q5/16 has not received any clear guidance from either SG12 or SG16 WP3 audio experts on what that level should be.

Polycom believes that this mismatch needs to be addressed, and that the best way forward is to use the nominal level that is used to characterize ITU-T full band codec performance.

# Discussion

The TD 768R1 (WP 2/16) draft of H.TPS-AV specifies a default value for the AudioLevel parameter of -31 dBov.  It also includes the following editor’s note:

{Editor's note :  -31 dBov needs more discussion, especially in the light of the codec overload point assumed.}

However, in recent years ITU-T codec performance has been characterized assuming a nominal input level of -26 dBov.

In order to progress the work, Polycom proposes that the value for AudioLevel be changed to -26 dBov and the Editor’s note be removed.

# Conclusion

Polycom proposes the following changes to section 7.1.3.4 of H.TPS-AV

#### 7.1.3.4	Audio Level

| Identity: | AudioLevel |
| --- | --- |
| Description: | This parameter indicates the audio level sent in the Telepresence audio stream. |
| Format: | dBov |
| Possible Values: | -26 dBov |
| Default: | -26 dBov |
| Reference: | Not Application |
| Interface: | None. |

Telepresence systems shall send audio to other telepresence systems with an average audio level of AudioLevel dBov as measured by [ITU-T P.56].

When a telepresence system connects to non-telepresence devices, it should use theAudioLevel expected by that device.  If this is unknown, it should use the Telepresence AudioLevel.

__________________