| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4342 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 22/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom,ZTE |  |  |  |
| Title: | H.IVSReqs: Proposed to add a new scenario - Event Detection in clause 5.2 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a new scenario Event Detection” for draft ITU-T Recommendation H.IVSReqs  “Intelligent visual surveillance requirements.”

# Introduction

Intelligent visual surveillance could automatically identify specific objects, behaviours or attitudes in video. It transforms the video into data to be transmitted or archived so that the video surveillance system can act accordingly.

Event Detection detects the events by analyzing the video obtained from cameras, such as Perimeter Prevention, Remnants detection, Theft Detection,etc.

# Discussion

This contribution proposes a new scenario “Event Detection” for draft ITU-T Recommendation H.IVSReqs  “Intelligent visual surveillance requirements.”

# Conclusion

It’s proposed to accept the scenario for H.IVSReqs in clause 5.2.

Start proposed content

# Scenarios for IVS

## Target Recognition

### Perimeter Prevention

Perimeter prevention / boundary detection of some key areas which develops the virtual cordon or virtual warning region can determine the direction of the target, identify the target object, such as person, transportation and so on.

Typical applications of perimeter prevention include the following 4 categories:

- Region detection: Alarm output must be based on the polygon region which is defined by users. When an object enters or leaves, alarm output could be triggered.

Fig.5-1 Region detection

- Region stranded: Alarm output must be based on the polygon region which is defined by users. When the retention time of an object exceeds a prescribed threshold, alarm output could be triggered.

Fig.5-2 Region standed

- Tripwire: A line that is drawn within the surveillance area. When an object crosses that line an alarm is set off.

Fig.5-3 Tripwire

- Directed tripwire: A line that is drawn within the surveillance area. When an object along the specified direction crosses that line,an alarm is set off.

A virtual cordon that is drawn within the video image. When an object which is depending on different targets beyond that line, an alarm is set off. As shown in Figure 5-4, the direction of the virtual cordon can be unidirectional or bidirectional.

Illegal intruder                                             Retrograde detection

Fig.5-4  Directed tripwire and region

End proposed content

___________________