| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | AVD-4344 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Modified architecture of telepresence in section 6 of draft F/H.TPS-Arch |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

In this contribution, we propose a modified high level functional architecture for telepresence system for section 6 of draft F/H.TPS-Arch.

- Discussion

In draft F/H.TPS-Arch, section 6 describes the high level architecture. There is an editor’s note that says the current figure is needed to be updated to illustrate the functional architecture from the logical view point.

In the proposed architecture, the identified components of the telepresence system which are telepresence endpoints, legacy video conferencing endpoints, legacy audio endpoints, MCU, GK, GW, and the management system are included.

We also considered both legacy H.323 and legacy SIP video conferencing endpoints as the legacy video conferencing endpoints. We assumed that these endpoints can interwork through a gateway as described in subsection 6.1.4.

- Proposal

This contribution proposes to a modified functional architecture of telepresence system with the modified text to be applied to section 6.

========= begin ===========

Figure x illustrates a high level diagram of the telepresence architecture.

Figure x: High level functional architecture

6.1.x 	Legacy conferencing endpoints

A legacy conferencing endpoint can be legacy H.323 or non H.323 conferencing endpoint. In case of legacy H.323 endpoint, the main capabilities and functions are identical to the definition in [ITU-T H.323]. In case of non H.323 endpoint, it can be a legacy endpoint which uses non H.323 open standards (i.e. SIP). This kind of endpoints can be a SIP hard phone and SIP soft phone (i.e. PC or Laptop). The legacy non H.323 endpoints can connect to other components (e.g. Gatekeeper) through a gateway which performs protocol conversion and stream adaptation.