| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-28 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | GNU Gatekeeper Project |  |  |  |
| Title: | Proposed Amendment to H.225.0 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This amendment formalizes the widely implemented call transfer mechanism in H.323 that relies upon the FACILITY message with a reason of “callForwarded”.

Recommendation ITU-T H.225.0

Call signalling protocols and media stream packetization for packet-based multimedia communication systems

Amendment 1

# Changes to Clause 7.2.2.16

7.2.2.16 Facility

In order to signal call redirection specific to H.323 procedures (call forwarding, call transfer, redirecting a call to the MC, or forcing a call to be routed to the gatekeeper) or in case of supplementary service signalling according to ITU-T Rec. H.450, the User-user information element of the Facility message is used. This particular case shall be indicated by coding a Facility IE of length zero; i.e., the Facility information element shall consist of exactly 2 octets as follows:

•	Octet No. 1 (information element identifier) shall be set to "00011100" ("1C'H) to indicate the Facility IE.

•	Octet No. 2 (information element length) shall be set to "0" to indicate that no further octets belonging to this information element follow.

In order to indicate call forwarding or call transfer, the Facility IE shall be empty and the Facility-UUIE shall indicate in the alternativeAddress or the alternativeAliasAddress the terminal to which the call is to be redirected. In this case, the facilityReason shall be set to callForwarded.

...

__________________