| Rapporteur Meeting of Questions 2, 3, 5, 12, 21 and 22/16 |  |  |  | TD-35 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Brisbane, 24 - 28 September 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor F/H.TPS-Arch |  |  |  |
| Title: | F/H.TPS-Arch “Telepresence System Architecture” (New): output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor’s output draft for F/H.TPS-Arch “Telepresence System Architecture” (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| F/H.TPS-Arch Ed. 0.1 | Initial baseline text. Output from Boston meeting 2011-07. Based on AVD-4084, 4094, 4095, 4091, 4107, 4108 and 4109. |
| F/H.TPS-Arch Ed. 0.2 | Input text to the November/December 2011 ITU-T SG16 meeting. It is a copy of the output of the Boston meeting with changes marks accepted and additional marked up minor editorial additions. |
| F/H.TPS-Arch Ed. 0.3 | Output text from the November/December 2011 ITU-T SG16 meeting. It incorporates changes from: C.712, C.713, C.745, C.746 and C.758. |
| F/H.TPS-Arch Ed. 0.4 | Input text to the November/December 2011 ITU-T SG16 meeting. It is a copy of the output of the Geneva 2012-12 meeting with changes marks accepted. |
| F/H.TPS-Arch Ed. 0.5 | Output text from Geneva 20-24 February 2012 meeting. It incorporates changes from: AVD-4204 (items 1 and 2) and AVD-4214. |
| F/H.TPS-Arch Ed. 0.6 | Input text to the Geneva 30 April - 11 May 2012. It is a copy of TD-19 from the Geneva 20-24 February 2012 meeting with change marks accepted. [TD 757/WP2 (2012-04)] |
| F/H.TPS-Arch Ed. 0.7 | Output text from the Geneva 30 April – 11 May 2012. It incorporates changes from: C.874, C.937 and C.938. |
| F/H.TPS-Arch Ed. 0.8 | Input text to the Brisbane 24 – 28 September 2012 meeting. |
| F/H.TPS-Arch Ed. 0.9 | Output text from the Brisbane 24 – 28 September 2012. It incorporates changes from: AVD-4288, AVD-4324, AVD-4344. |

Living List Items:

- The current Telepresence Architecture builds on the H.323 architecture. Using the term H.323 telepresence endpoint may cause confusion at a future point in time if one of the Telepresence Recommendations attains anH.32x designation. This is item kept as a reminder to consider this when discussing Recommendation number allocation with the TSB.

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 5 4 Abbreviations and acronyms 5 5 Conventions 6 5.1 Requirement terminology 6 5.2 Icons used 6 6 High level architecture 8 6.1 Description of sub-system components 9 6.1.1 Telepresence Endpoint 9 6.1.2 Media Control Unit 10 6.1.3 Gate Keeper 10 6.1.4 Gateway 10 6.1.5 Management System 10 7 Sub-system architectures 10 7.1 Endpoint architecture 10 7.1.1 Endpoint Architecture Description 12 7.2 Multipoint Control Unit 16 7.3 Gate Keeper 17 7.4 Gateway 17 8 H.323 architecture 17 8.1 Mapping telepresence on a H.323 architecture 17 8.1.1 Peer-to-peer communication 19 8.1.2 Multi-point communication 20 8.2 Peer-to-peer call process 21 9 Telepresence room design 23 9.1 Room layouts 23 9.1.1 Single row 3-screen telepresence system 23 9.1.2 Multi row 3-screen telepresence system 24 9.1.3 Multi row 4-screen telepresence system 25 9.2 Room characteristics 26 9.2.1 Room dimensions and shape 26 9.2.2 Wall floor and ceiling surfaces 26 9.2.3 Lighting 26 9.2.4 Acoustics 26 9.2.5 HVAC 27 9.2.6 Power requirements 27 9.2.7 Network connectivity 27 Annex A <Annex Title> 27 Appendix I <Appendix Title> 27 Bibliography 27

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1: Icons used in this Recommendation 8 Figure 2: High level Telepresence architecture network model 9 Figure x: High level functional architecture 10 Figure 3: Endpoint Architecture 12 Figure 4: Video capture unit architecture 13 Figure 5: Camera models: converging camera model (left)，parallel camera model (middle)，con-optical centre camera model (right) 13 Figure 6: Video Render Unit Architecture 14 Figure 7: Audio capture unit architecture 15 Figure 8: Audio render unit architecture 16 Figure 9: H.323 Telepresence network model 18 Figure 10: H.323 Telepresence system architecture 19 Figure 11: H.323 Telepresence peer-to-peer communication 20 Figure 12: H.323 Telepresence multi-point communication 21 Figure 13: H.323 telepresence peer-to-peer call processes 22 Figure 14: H.323 telepresence multi-point call processes 23 Figure 15: H.323 telepresence peer-to-peer detailed call processes 24 Figure 16 : Three-screen telepresence room layout (single row) 25 Figure 17 : Three-screen telepresence room layout (multi-row) 26 Figure 18: Four-screen telepresence room layout (Multi row) 27

Draft new Recommendation F/H.TPS-Arch

Telepresence System Architecture

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

This Recommendation first describes a Telepresence system in a high level technology agnostic manner. This is then followed by a more in depth description of the components. The later parts of this Recommendation then describe how [ITU-T H.323] relates to a Telepresence system. It then details aspects regarding Telepresence room design.

Unless explicitly stated in the text, the interfaces depicted in the figures and functions in this Recommendation are logical and thus do not necessarily represent a particular physical implementation.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[H.TPS-AV]	Recommendation ITU-T X.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T Q.931]	Recommendation ITU-T Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

# 3	Definitions

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

{Editor’s note: The IETF Clue working group currently have an internet draft (draft-wenger-clue-definitions-xx)[http://datatracker.ietf.org/doc/draft-wenger-clue-definitions] defining Telepresence related terms. Rather than re-defining terms here it may be advantageous to reference the IETF terms.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Telepresence Media Codec:  This term (and function) is used to describe any media (i.e. audio or video) codec that would be applicable for use in a Telepresence system.

3.2.2	Telepresence System:  A set of functions, devices and network elements which are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a Telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers,  microphones, codecs, MCUs,  PCs) and environmental characteristics are used to establish Telepresence.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| GK | Gate Keeper |
| --- | --- |
| GW | Gateway |
| HVAC | Heating, Ventilation and Air Conditioning |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| LAN | Local Area Network |
| LCD | Liquid Crystal Display |
| MCU | Multipoint Control Unit |
| PC | Personal Computer |
| PDP | Plasma Display Panel |
| PSTN | Public Switched Telephone Network |
| RAS | Registration, Admission and Status |
| RTP | Real-time Protocol |
| RTCP | RTP Control Protocol |
| SIP | Session Initiation Protocol |
| TCU | Telepresence Control Unit |
| TMC | Telepresence Media Codec |
| TMIO | Telepresence Media Input / Output Control Unit |
| TMP | Telepresence Media Processor |
| WAN | Wide Area Network |

# 5	Conventions

## 5.1	Requirement terminology

In this Recommendation the following conventions are used:

–	"Shall" indicates a mandatory requirement.

–	"Should" indicates a suggested but optional course of action.

–	"May" indicates an optional course of action rather than a recommendation that something take place.

## 5.2	Icons used

Figure 1 shows the meaning of icons used in this Recommendation.

Single Screen

Telepresence Endpoint

2 - Screen

Telepresence Endpoint

3 - Screen

Telepresence Endpoint

Document Camera

Telepresence Control

Unit

Gate Keeper

Legacy H.323 Endpoint

Main Camera

Management System

Mobile Phone

Multipoint Control Unit

Microphone

Personal Computer

Phone

Presentation Display

Router

Switch

Telepresence Media

Codec

Telepresence Media

Processor

Telepresence Media

Input / Output Control Unit

Loudspeaker

Touch Control Panel

Main Display

Voice Gateway

Figure 1: Icons used in this Recommendation

# 6	High level architecture

Figure 2: High level Telepresence architecture network model

A high level telepresence architecture includes: telepresence endpoints, legacy video conferencing endpoints, legacy audio endpoints, Multipoint Control Units (MCU), Gate Keepers (GK), Gateways (GW), the management system, etc. There may be multiple forms of telepresence endpoints, such as three-screen endpoint, single-screen endpoint and etc. The architecture supports inter-operation of heterogeneous telepresence endpoints and inter-operation between telepresence endpoints and legacy video conferencing and audio endpoints.

There may be multiple MCUs and GKs in the architecture.

The architecture may have multiple GWs that connect to other types of networks, such as fixed and mobile telephone networks.

{Editor’s note: There was an agreement that defining a functional architecture (in addition to the above section) may add value to the architecture. Further work is needed in this area. Whilst incomplete, the figure below is included to stimulate further Contribution. Further Contributions are needed to analyse the applicable interfaces and describe their use. Control and media plane aspects should be considered.

Figure x: High level functional architecture

}

## 6.1	Description of sub-system components

### 6.1.1	Telepresence Endpoint

The Telepresence Endpoint is the logical point of final termination, and may consist of more than one physical device. The main functions of the telepresence endpoint include:

- Setting up call sessions through capability negotiation,

- Capturing, encoding, packetizing and sending local media streams,

- Receiving, de-packetizing, decoding and rendering remote media streams.

In contrast to a telepresence endpoint, an MCU may also send and receive media streams, but it cannot initiate or terminate media streams through capturing and/or rendering. There may be multiple forms of telepresence endpoints, such as: 3 camera/screen endpoint, single camera/screen endpoint, audio-only endpoint and etc.

Endpoint characteristics include:

- The number of main cameras, spatial relationships of cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- Viewpoint and field of view for main cameras, including overlap regions, missing regions or whether images of cameras can be composited to a seamless image in over the entire depth range. The number of presentation cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- The video stream capabilities for data sharing such as: resolution, frame rate and aspect ratio.

- The number, type, size of main screens, spatial relationships of screens and screen capabilities, such as: resolution, frame rate, aspect ratio, and screen border width.

- The number and type of presentation screens, spatial relationship of presentation screens and screen capabilities.

- Spatial relationships of audio signals of microphones and capture capabilities of microphones.

- Spatial relationships of loudspeakers and parameters of loudspeakers.

- Display strategies for multipoint conference, such as the static or dynamic mapping of camera-to-display.

In general endpoint characteristics are transmitted through signalling streams in session initialization and used for capability negotiation in inter-operation of different telepresence endpoints.

Note: [ITU-T H.245] defines a terminal as “any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server”. As such the term H.323 Telepresence Terminal is equivalent to the term “Telepresence Endpoint”.

### 6.1.2	Media Control Unit

The MCU is a device that connects two or more telepresence endpoints together to one single telepresence conference. An MCU can setup and control multiple videoconferencing call sessions. After receiving media streams from multiple endpoints or cascading MCUs and GWs, an MCU processes the media streams and sends the processed media stream to an endpoint, MCU or GW.

An MCU must support video and audio mixing and media adaptation for different types of endpoints and should also support control functions, for example, auxiliary stream token control, voice switch and control based on endpoint screens, etc.

### 6.1.3	Gate Keeper

A GK is the resource management device for video conferencing and can provide resource management for endpoints, MCUs and GWs. The functions of a GK include: address resolution, access control and bandwidth management.

### 6.1.4	Gateway

A GW can connect various types of networks and provide interworking of signalling and media streams, such as interworking H.323 and SIP. It may also provide adaptation for media streams that have different characteristics e.g. codec format.

### 6.1.5	Management System

A management system mainly contains control and management functions that provide specific telepresence services, such as device management, conference scheduling and user authentication, etc.

### 6.1.6	Legacy conferencing endpoints

A legacy conferencing endpoint can be legacy H.323 or non H.323 conferencing endpoint. In case of legacy H.323 endpoint, the main capabilities and functions are identical to the definition in [ITU-T H.323]. In case of non H.323 endpoint, it can be a legacy endpoint which uses non H.323 open standards (i.e. SIP). This kind of endpoints can be a SIP hard phone and SIP soft phone (i.e. PC or Laptop). The legacy non H.323 endpoints can connect to other components (e.g. Gatekeeper) through a gateway which performs protocol conversion and stream adaptation.

# 7	Sub-system architectures

## 7.1	Endpoint architecture

Figure 3: Endpoint Architecture

A telepresence endpoint consists of the following components:

- A Video Capture Unit (VCU) that consists of multiple main cameras and presentation cameras (optional) in certain spatial relationships.

- A Video Render Unit (VRU) that consists of multiple screens for main video streams and screens (optional) for presentation in certain spatial relationships.

- An Audio Capture Unit (ACU) that consists of multiple microphones in certain spatial relationships.

- An Audio Render Unit (ARU) that consist of multiple loudspeakers in certain spatial relationships.

- A Telepresence Control Unit (TCU) that connects and controls other units. It includes: a Telepresence Media Codec (TMC), Telepresence Media Processor (TMP) and Telepresence Media Input/Output (TMIO) Control units.

- A Network Unit (NU), including: wired and wireless network interfaces and network switch, etc.

- An Interactive Unit (IU), including: touch control panels, PCs, etc.

### 7.1.1	Endpoint Architecture Description

#### 7.1.1.1	Video capture unit

A telepresence endpoint may have multiple video sources including main cameras, presentation cameras and auxiliary video stream from a PC or other device. Video sources connect through a telepresence media input/output control unit that can select and control multiple video sources.

Figure 4: Video capture unit architecture

Multiple main cameras may constitute a camera set for capturing images of participants at the telepresence endpoint. Each main camera capture images of participants in one region of the room corresponding to a particular spatial arrangement and the multiple images of cameras are combined to a wide view angle, high resolution view. Where spatial information is known the video capture unit shall support the carriage of this information. A camera set may use one of the following three models: the converging camera model, the parallel camera model and the con-optical centre camera model.

Figure 5: Camera models: converging camera model (left)，parallel camera model (middle)，con-optical centre camera model (right)

In the converging camera model, each of multiple cameras looks out directly in a different direction. The parallel camera model, in contrast to the converging camera model, all optical camera axis have the same direction. Multiple cameras capture multiple virtual images reflected by a mirror and the captured images almost have the same optical centre in the con-optical centre camera model. For the first two camera models, there may be overlap and missing regions between two images and individual images may not be able to be integrated seamlessly unless objects are extremely distant. While in the con-optical centre camera model, individual images can be stitched to a seamless image over the entire depth range. The composited image can be scaled to an image having a single camera’s resolution for inter-operation with a legacy videoconferencing endpoint that does not support this capability.

Presentation cameras are used to capture images of documents or objects in a conference room. There may be various spatial placements for presentation cameras, for example, hanging the cameras from the ceiling to capture images of objects on a desk.

Auxiliary/s video stream/s from a PC or other devices are used for sharing data.

#### 7.1.1.2	Video render unit

A video render unit may comprise of one or more main screens and presentation screens. Main screens are primarily used to display the video streams captured by local and remote main cameras and can be LCD, PDP monitors or projectors. In a fully immersive system a main screen should have sufficient size to display actual-sized images. Multiple monitors are usually integrated to present a panoramic view, or a single projection screen and multiple projectors can also be used.

Figure 6: Video Render Unit Architecture

The telepresence media input/output control unit can dynamically arrange display patterns and contents for main screens and presentation screens in an automatic or manual way:

- Main screens may display the local and remote main video streams and presentation video streams with reproduction in a spatially correct manner based on the spatial information associated with the streams.

- Typically where a presentation screen does not have a camera capable of capturing the participant/s it should only display the local and remote presentation video streams, because eye contact cannot be achieved when a main video stream is presented on a presentation screen.

#### 7.1.1.3	Audio Capture Unit

An audio capture unit may comprise of multiple microphones in some spatial placement for capturing audio signals at the telepresence endpoint. This spatial placement would largely be dependent on the Telepresence endpoint configuration. For example: microphone placement may differ between single-row and multiple-row configuration. There may be no direct mapping between microphones and audio streams. Where spatial information is required the audio capture unit should support the carriage of this information.

Figure 7: Audio capture unit architecture

Microphones may connect through a media input/output control unit that can select and control multiple audio sources. Auxiliary audio streams from a PC or other devices also connect to endpoint through media input/output control unit. Audio signals from multiple microphones may be combined into one single audio stream for transmission.

{Editor’s note: Further contribution is required to discuss in more detail how the audio signals are combined and transmitted.}

#### 7.1.1.4	Audio Render Unit

An audio render unit may comprise of multiple loudspeakers in some spatial placement for rendering audio signals of the remote telepresence endpoint. There are no direct mappings between loudspeakers, microphones and screens, but the spatial information of sound from the remote endpoint can be reproduced through the audio render unit. Both the sound and image positions of remote participants should coincide.

Loudspeakers should connect through the media input/output control unit that can select and control audio output dynamically.

Figure 8: Audio render unit architecture

#### 7.1.1.5	Telepresence Control Unit

A telepresence control unit connects other units in a telepresence endpoint. The main functions of the telepresence control unit include:

- To provide interfaces to other functional unit in an endpoint, invoking other units to complete processing.

- To handle signalling processes, including: registration, authentication, session initialization, session control and session termination, capabilities negotiation for inter-operation, etc.

- To send and receive multiple media streams.

- To provide contacts and other interfaces.

- To provide device management and debug interfaces, etc.

- etc.

#### 7.1.1.6	Telepresence media codec

The telepresence media codec is a logical unit that be used for coding and decoding of multiple video and audio streams. Synchronization between video and audio streams is supported. The telepresence media codec can be implemented using two configurations, a centralized solution or distributed solution. A centralized solution uses a single device to code and decode multiple media streams. Synchronizing media streams for this configuration is simpler but the device requires higher processing capabilities. The distributed solution needs more than one device for coding and decoding of multiple media streams which results in more complicated synchronization strategies. However this configuration may use devices that have lower processing capabilities than that of the centralized solution.

#### 7.1.1.7	Telepresence media input/output control unit

The telepresence media input/output control unit is a logical unit that can select and control multiple input/output media streams that include main and auxiliary video and audio streams. Video and audio capture units, video and audio render units, PCs and other audio endpoints are connected to the telepresence media input/output control unit.

#### 7.1.1.8	Telepresence media processor

The telepresence media processor is used for processing video and audio steams to provide the required audio and visual experience. The main functions of telepresence media processor include:

- Geometric correction for multiple camera images to get better image mosaic effect.

- Colour correction for making luminance and chrominance of multiple camera images uniform.

- Geometric correction for multiple video stream rendering, that make the video outputs adapt to various types of screen and get better display performance, for example: geometric correction for arc projection screen.

- Colour correction for multiple video stream rendering for a better rendering performance.

- Pre-processing for audio signals captured by audio capture unit, such as audio de-noising and enhancement for multiple microphones, etc.

- Pre-processing for audio signals rendered by audio render unit, for example, improving audio quality of loudspeakers.

## 7.2	Multipoint Control Unit

The main functions of an MCU are basically identical to the definitions in [ITU-T H.323], as per the following descriptions.

Access functions:

- To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints.

- To support multi-image composition videoconferencing.

- To support single/multiple auxiliary media streams.

- To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode.

Adaptation functions:

- Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc.

- Adaptation for media streams according to characteristics of endpoint, such as screen aspect ratio, etc.

Control functions:

- To support voice switch and manual switch mode.

- To support switch based on endpoint or screens when an endpoint has multiple screens.

## 7.3	Gate Keeper

The main functions of GK are identical to the definitions in [ITU-T H.323].

## 7.4	Gateway

The main functions of GW are identical to the definitions in [ITU-T H.323].

# 8	H.323 architecture

## 8.1	Mapping telepresence on a H.323 architecture

The Telepresence system architecture is based on the existing [ITU-T H.323] system. A Telepresence endpoint can be viewed as another kind of H.323 endpoint, as depicted in the Figure 9.

Figure 9: H.323 Telepresence network model

[ITU-T H.323] specifies the interfaces between the different components of a H.323 based multimedia system. This is illustrated in Figure 10.

Figure 10: H.323 Telepresence system architecture

The main interfaces (reference points) in H.323 telepresence system architecture:

- A: Interface between legacy video conferencing endpoint and MCU, for transmitting conference call and control information,  completing  creation and release of media channels and media interaction.

- B: Interface between legacy video conferencing endpoint and GK, for transmitting registration, authentication and call process information using RAS messages.

- C: Interface between MCU and service system, for transmitting conference scheduling information, creating and ending conferences.

- D: Interface between MCU and GK, for registration, authentication, call process of MCU and transmitting resolving information of user addresses.

- E: Interface between GKs, transmitting resolving information of user addresses using RAS messages, etc.

- F: Interface between GK and service system, for transmitting GK’s management and configuration information, user information, billing data, etc.

- G: Interface between legacy videoconferencing endpoints, for transmitting call signalling, media streams and related control information when a terminal has a peer-to-peer communication.

- H: Interface between MCUs, for transmitting media streams between MCUs and information of MCU cascade connection and inter-operation.

- I: Interface between the MCU and PSTN/ISDN network.

- J: Interface between MCU and [ITU-T H.320] network, for inter-operation between H.323 and H.320 service network.

- K: Interface with other systems.

In order to support telepresence endpoints several new interfaces based on existing ones are introduced:

- A1: Interface between telepresence endpoint and MCU, for transmitting call and control information of telepresence conference, completing creation and release of media channels and exchanging  video and audio streams between telepresence system and MCU.

- B1: Interface between telepresence endpoint and GK, for transmitting registration, authentication and call process information using RAS messages.

- G1: Interface between telepresence endpoints, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with another.

- G2: Interface between legacy video conference endpoint and telepresence endpoint, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with a legacy video conference endpoint.

### 8.1.1	Peer-to-peer communication

Figure 11 illustrates a peer-to-peer communication scenario for telepresence.

Figure 11: H.323 Telepresence peer-to-peer communication

The communication between telepresence endpoints should have a single call control signalling stream, multiple video streams and a single audio stream.

The control signalling stream is used for controlling the whole calling process, for example: enabling/disabling the sending of auxiliary streams.

### 8.1.2	Multi-point communication

Figure 12 illustrates a multi-point communication scenario for telepresence.

Figure 12: H.323 Telepresence multi-point communication

Different types of telepresence endpoints may connect to an MCU.

Telepresence endpoint sends media streams according to results of capability negotiation.

The MCU completes the adaptations for media streams, for example: adaptation for single channel and multi-channel audio streams, retransmission for multiple video streams and adaptation for conference control.

## 8.2 Call process

### 8.2.1 Peer-to-peer call process

The peer-to-peer call processes based on the existing H.323 system is depicted in Figure 13.

Figure 13: H.323 telepresence peer-to-peer call processes

The whole call process consists of the following steps:

- Registration process

- Call session setup process

- Media stream transport and control process

- Call session release process

- Deregistration process

The key steps that affect inter-operability are steps 2 and 3.

The detailed processes of step 2 and 3 are depicted by the following figure. A peer-to-peer call (multi-point call is similar) example through GK is given:

Figure 14: H.323 telepresence peer-to-peer detailed call processes

The detailed signalling processes include:

- Call setup process, include RAS and [ITU-T Q.931] call signaling.

- Media setup process, mainly include mast-slave determination, capability negotiation, logical channel opening through H.245 control signaling.

- Media stream transport and control process, include media streams transmission and controls using RTP/RTCP, [ITU-T H.245] control signaling.

### 8.2.2 Multi-point call process

The multi-point call processes based on the existing H.323 system are similar to peer-to-peer processes, as illustrated in Figure 15.

Figure 14: H.323 telepresence multi-point call processes

The whole call process consists of the following steps:

- Registration process

- Call session setup process

- Media stream transport and control process

- Call session release process

- Deregistration process

The key steps that affect inter-operability are steps 2 and 3.

# 9	Telepresence room design

## 9.1	Room layouts

This clause details several example room layouts. Other configurations are possible.

In these examples where the multiple capture regions are utilized the boundaries between the regions may be convergent or appear to be parallel. These boundaries may have an impact on the apparent gap between renderings of the captures.

### 9.1.1 Single row seating 3-screen telepresence system

The following figure depicts the room layout of a single row 3-screen telepresence system, with the following characteristics:

- Three main cameras and three main screens.

- Three regions for participants in the room, and each region contain two seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and covered by borders of two main screens.

Figure 16 : Three-screen telepresence room layout (single row)

### 9.1.2 Multi row seating 3-screen telepresence system

The following figure depicts the room layout of a multi-row 3-screen telepresence system, with the following characteristics:

- Three main cameras and three main screens.

- Three regions for participants in the room, and each region contain five seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and covered by borders of two main screens.

Figure 17 : Three-screen telepresence room layout (multi-row)

### 9.1.3 Multi row seating 4-screen telepresence system

The following figure depicts the room layout of a multi-row 4-screen telepresence system, with the following characteristics:

- Four main cameras and four projectors.

- Four regions for participants in the room, and each region contains four or five seats.

- Each main camera captures images of one region.

- Three overhead microphones capturing multi-channel audio from the room

Each projected image shows the far end participants at their true size.  The display height is chosen so that standing individuals of normal height are displayed without cropping.  In order to increase the height, 4:3 aspect ratios are used.  Since projectors are used, there is no gap between the displayed images.

Main cameras are placed behind the projector screens, and are placed at eye level. The cameras are aligned to minimise gaps between captures. The seating is organised so that a person does not span a gap between captures.

The three microphones are overhead, and arranged to capture multichannel room audio.

The second row of participants is elevated about 25 cm above the first row.  This allows all participants to be easily seen.

Figure 18: Four-screen telepresence room layout (Multi row)

### 9.1.4 Single row seating 6-screen telepresence system

The following figures depict the room layout of a 6-screen telepresence system, with the following characteristics:

–	Three main cameras and six screens with narrow frame borders.

- Three screens in one row, two rows of screens.

- Main cameras placed between thescreens of the bottom and top row.

–	Three regions for participants in the room, and each region containing two or three seats.

–	Each main camera captures images of one region.

–	One wall projector displays a selected presentation/stream.

As can be seen in Figure 19, a set of three screens are arranged in one row. Each screen shows the far end participants at their true size. Three main cameras are placed at eye level and sandwiched between two rows of screens; each camera is placed in the centre of the bottom frame of the higher screen and in the centre of the top frame of the lower screen.

Camera 1

Camera 2

Camera 3

Screen 1

Screen 2

Screen 3

Screen 4

Screen 5

Screen 6

Figure 19: Screen and camera layout for Six-screen Telepresence

The use of two rows of screens allows for other display scenarios not possible with a single row of screens. This is particularly useful in scenarios with more than two end points in a call. For example the following scenarios are possible:

- The simultaneously display of two sites, with one site shown in the first row and the other in the second row;

- Four adjacent screens, e.g. the left two and the middle two, displaying a scaled image of the current speaker, and the remaining two screens displaying in life size;

- All the screens displaying switched speakers, behaving like a switched video wall.

Note: this is possible in a single row. An additional row allows for more participants

- The upper row displaying active speakers while the lower row displays presentation and auxiliary information;

- The upper row displaying the local site while the lower row displays the remote site.

As shown in Figure 20, each main camera captures a region of two (or three) seats along a table. Presentation screens and microphones are mounted on the long table. An additional wall projector is provided to display desired streams/presentations upon user demand.

Since screens with narrow frame borders are used, the displayed images need more manipulations to obtain a continuous view. So image processing methods, such as cropping and scaling, are required to provide better visual experience when interoperating with screens with broader frame borders.

Note: The dashed lines in Figure 20 represents objects that may be used when operating in a non-telepresence mode.

Camera

Projector

Seat

Screen

Microphone

Presentation screen

Region A

Region B

Region C

Figure 20: Six-screen Telepresence room layout

## 9.2	Room characteristics

### 9.2.1	Room dimensions and shape

TBD.

### 9.2.2	Wall floor and ceiling surfaces

TBD.

### 9.2.3	Lighting

TBD.

### 9.2.4	Acoustics

TBD.

### 9.2.5	HVAC

TBD.

### 9.2.6	Power requirements

TBD.

### 9.2.7	Network connectivity

TBD.

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

_________________