| Question(s): | 2 |  | Meeting, date: |  | 14 - 25 January 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-TD): |  | TD |
| Source: | Editor |  |  |  |  |  |
| Title: | H.323 Amendment 1 “Use of FACLITY message to enable Call Transfer” |  |  |  |  |  |
| Contact: | Paul E. Jones Cisco Systems, Inc. USA |  |  |  | Tel: +1 919 476 2048 Fax: Email: paulej@packetizer.com |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

Introduction

This text represents the output from the previous Q2/16 Rapporteur meeting held in Brisbane, Australia 24 - 28 September 2012 proposing an amendment to H.323 to allow the FACILITY message to be used for call transfer.  In practice, the FACILITY message is already widely used for call transfer, but the procedures were never formalized.

AAP Summary

This Amendment to H.323 formalizes the widely implemented call transfer mechanism in H.323 that relies upon the FACILITY message with a reason of “callForwarded”.

Summary

This Amendment to H.323 formalizes the widely implemented call transfer mechanism in H.323 that relies upon the FACILITY message with a reason of “callForwarded”.

Recommendation ITU-T H.323

Packet-based multimedia communications systems

Amendment 1

# Changes to Clause 8.1.10

8.1.10 Call forwarding and call transfer

An endpoint wishing to forward an unestablished call or transfer an established call to another endpoint may issue a Facility message with FacilityReason callForwarded indicating the address of the new endpoint. The endpoint receiving this Facility indication should send a Release Complete and then restart the Phase A procedures with the new endpoint.

__________________