|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 2 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 2 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP¦H.248.TLS: Why separation of TLS and TCP work items? |  |  |  |

Background:

The initially combined work item on TLS/TCP was finally divided in separate work items (and Recommendations:

- TLS: H.248.TLS & H.248.TLSPROF

- TCP: H.248.TCP

We’d like to recall again the arguments of that work splitting because neither explicitly described in above H.248 work items.

# Rationale:

There are primarily following reasons for the separation in different Recommendations:

- TLS is designed as transport independent protocol (and TCP is just one transport mode option besides others).

- TCP bearer connections are independent of any transport security approach, and also already used without TLS in H.248 profiles.

- TLS-over-TCP: the establishment and release processes at the different levels of TCP connections and TLS sessions is basically decoupled.

- Note 1: A successfully established L4 transport connection is of course a precondition for the start of TLS session establishment, but both are temporarily decoupled. The application may decide for enabling/disabling transport security during the lifetime of the underlying active transport connection.

- TLS-over-MPTCP (multipath TCP): normally, the variant of singlepath and multipath TCP should be hidden to TLS, however, this is still debated and not yet confirmed (by IETF WGs TLS & MPTCP).

- Note 2: The TLSoMPTCP use case is transparent for the MG in case of TLS-to-TLS transparent forwarding. MPTCP would be relevant for the MG when located between a “MPTCP domain” and a “legacy single path TCP domain”.

- Technological evolution: there is a stable and mature protocol baseline, but both protocols are still evolving (see IETF WG TLS for TLS; see IETF WG MPTCP for TCP)

Conclusions:

Following observations should be noted in the meeting report:

- H.248.TCP and H.248.TLS define basically disjoint H.248 control tools, which should be applicable for isolated bearer types ‘TCP’ and ‘TLS’ and combined bearer types ‘TLS/TCP’ or ‘TLS/SCTP’;

- H.248.TCP and H.248.TLS describe both SDP-based and H.248 Property based control options for bearer types ‘TCP’ and ‘TLS’ respectively;

- SDP-based control of combined ‘TLS/TCP’ bearer type needs to be carefully analyzed; if there are some minimum use cases, then any feature interaction with pure ‘TLS’ and ‘TCP’ control needs to be investigated first.

_________________

E.g., an H.248 IP-IP MG providing a „MPTCP proxy” function (see e.g. http://www.ietf.org/proceedings/85/slides/slides-85-mptcp-0.pdf ).