|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 3 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 3 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP¦H.248.TLS: Evaluation of state models for state-oriented H.248 bearer types |  |  |  |

# Summary:

This contribution evaluates some state models concerning state-oriented H.248 bearer types. This subject is primarily relevant for work items H.248.TCP and H.248.TLS.

Overview:

Summary:	1

1	Background:	2

2	Discussion	2

2.1	Starting point: bearer connection state model for AAL2	2

2.2	Baseline: Generic bearer connection state model	3

2.3	Bearer connection state model for ‘TCP’	3

2.4	Bearer connection state model for ‘TLS’	4

2.5	Bearer connection state model for ‘SCTP’	5

2.6	Extension for “bearer connection modification”?	6

2.7	Test: Q.1950 vs generic bearer connection state model?	8

3	Conclusions:	9

Appendix I: Technology specific state models	10

I.1	AAL2	10

I.2	TCP	11

I.3	TLS	12

I.4	SCTP	12

# 1	Background:

The subject of state machine for bearer connection endpoint modelling is touched by parallel contributions C-43 and C-4. The topic is twofold:

- Such kind of state models were so far not really documented in H.248. But there are definitely assumptions of state models behind a few existing H.248 packages concerning bearer connection control (especially Q.1950).

- Value of a state model with scope on bearer types TCP, TLS, but also the tight combination of TLS/TCP? And if, then a generic or bearer specific models?

Purpose of this contribution is to start the discussion on this topic.

# 2	Discussion

## 2.1	Starting point: bearer connection state model for AAL2

Looking back in the history of supported H.248 controlled bearer types, then the best specified and documented state model is the one related to AAL2/ATM bearer connections, see Q.2630.

Appendix I.1 provides a copy of the two separated state machines for active (Fig. I.1) and passive (Fig. I.2) bearer connection establishment, as required for client/server oriented bearer protocols.

The AAL2 control state machines differentiates the communication in four basic phases (Idle → Establishment → Established → Release (→ Idle)), using a model with seven states.

## 2.2	Baseline: Generic bearer connection state model

The two AAL2 state machines are merged to a single, generic model in Figure 1:

Figure 1 – Generic bearer connection state model

We need to check whether such a generic model could be used as common baseline for bearer independent bearer connection controls. Some example bearer types are evaluated in following subsections.

## 2.3	Bearer connection state model for ‘TCP’

The TCP connection state machine (see Appendix I.2) defines twelve states. Figure 2 provides a mapping attempt on the generic model from clause 2.2:

Figure 2 – H.248 bearer connection state model for ‘TCP’

Observations:

- Establishment: passive and active is OK; active-active (= simultaneous) is also OK (because there’s no need for MGC or MG to intervene);

- Release: multiple states to be transitioned from Established to Idle, primarily due to data acknowledgement process and two-way handshake; further: asymmetry due to oneway closure; again there’s no need for intervening in the various interim steps of the transitioning by H.248 entities;

- H.248 state view: perhaps a two-state model (Idle, Established) might be sufficient? (TBC)

- Note: the oneway release could lead to traffic direction dependent state machines.

## 2.4	Bearer connection state model for ‘TLS’

There isn’t an explicit defined state model from the TLS protocol (see Appendix I.3). Figure 3 provides a mapping attempt of TLS session handshake procedures on the generic model:

Figure 3 – H.248 bearer connection state model for ‘TLS’

Observations:

- A mapping is feasible, but not that strict than for TCP due to the lack of explicitly defined state model by TLS itself;

- H.248 state view: perhaps again a two-state model (Idle, Established) might be sufficient? (TBC)

## 2.5	Bearer connection state model for ‘SCTP’

The SCTP connection state machine (see Appendix I.4) defines eight states. Figure 4 provides a mapping attempt on the generic model from clause 2.2:

Figure 4 – H.248 bearer connection state model for ‘SCTP’

Observations:

- similar than TCP, but simpler (due to bidirectional concept of an SCTP association);

- H.248 state view: perhaps again a two-state model (Idle, Established) might be sufficient? (TBC)

## 2.6	Extension for “bearer connection modification”?

We’ve focused on the processes of bearer establishment and bearer release, but Q.1950 is also additionally differentiating the third process of possible bearer modification (during active communication phase). We would get an extended state model, see Figure 5:

Figure 5 – Extended generic bearer connection state model with support of modification

Observations:

- an H.248 use case would be given e.g. when the MGC wants to be informed about successful bearer connection modification procedures (note: this information is not related to the H.248 MODIFY.reply command, rather the event-based H.248 NOTIFY.request command);

- however, any extension of the basic state model is probably not needed;

- Concerning TCP and TLS: modification process isn’t really subject of these protocols

- Note - possible bearer connection properties for modification: typically transport capacity or QoS classes, which are both out of scope of bearer control protocols for TCP, TLS, SCTP, …

## 2.7	Test: Q.1950 vs generic bearer connection state model?

We’d like to test possible reuse of generic Q.1950 control capabilities for generic bearer connection control, talking into account above generic model. We need to consider the MG local bearer connection endpoint only, see Figure 6:

Figure 6 – Different interfaces concerning TCP bearer control

Any state transitioning may be described by a rule format, with a set of conditions X related to the triggering of a state transition, and a set of (optional) actions Y associated with the transition:

- Elements of state transition rule conditions X: incoming BCP signaling, incoming H.248 signaling; and

- Elements of state transition rule actions Y: outgoing BCP signaling, outgoing H.248 signaling.

Figure 7 illustrates an initial (still incomplete) test. The H.248 tools for generic bearer connection control are the H.248.1 generic (g) package and the Q.1950 generic bearer connection (gb) package.

Figure 7 – Test: Q.1950 vs generic bearer connection state model

# 3	Conclusions:

The contribution does request any concrete action for existing H.248 work items, rather provide some general observations:

- the generic bearer connection state model (Fig. 1) would be a good starting point, if an explicit state model would be required in context of e.g. TCP, TLS, TLS/TCP, SCTP, DTLS;

- this model might be further simplified to a two-state model, if there would not be any requirements to be involved from H.248 perspective (“which seems to be the case, at least for TCP and TLS”)

- if any new H.248 properties (primarily type Property, Event and Signal) would be defined for bearer specific bearer connection control, then it would be beneficial to illustrate the underlying state model in informative text (such as an Appendix)

Backup material:

# Appendix I: Technology specific state models

## I.1	AAL2

Ref. Q.2630.1:

Figure I.1 (= Fig. 8-1/Q.2630.1) – State transition diagram for the outgoing protocol procedure

Figure I.2 (= Fig. 8-3/Q.2630.1) – State transition diagram for the incoming protocol procedure

## I.2	TCP

Ref.: Fig. 6/RFC 793:

Figure I.3 – RFC 793: TCP Connection State Diagram

TCP: twelve states.

## I.3	TLS

Connection state model defined in clause 6.1/RFC 5246.

## I.4	SCTP

Ref.: Fig. 6/RFC 4960:

Figure I.5 – RFC 4960/Figure 3: State Transition Diagram of SCTP

SCTP: eight states.

_________________