|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 6 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 6 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Clause 1 – Text input for scope section |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

# Discussion

Clause 1 is still lacking an initial text proposal. First input text is proposed by this contribution, primarily based on the work item description. An editor’s note shall reflect the still premature state of this Draft.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

[Begin Proposal]

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

{Editor’s note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

In scope of this Recommendation are TCP-based bearer interfaces of H.248 IP media gateways. Such an H.248 MG could provide various functions for processing of TCP/IP packets, TCP payload data and involvement in functions related to TCP protocol control information (based on TCP header flags). This Recommendation focuses primarily on the aspect of TCP connection control, which comprises

- establishment,

- modification (tbd) and

- release

of TCP bearer connections.

The Recommendation needs to consider

- connection models with a single or multiple TCP bearer connection endpoints within an H.248 context;

- establishment, cut-through and release behaviour in case of end-to-end TCP bearer connections across two H.248 TCP terminations (“a so-called H.248 TCP media stream”);

- different H.248 control models

further, this Recommendation provides information

- about existing H.248 tools with respect to other  TCP support functions;

## 1.1	Applicability statements

{Contributor’s note: this subclause is identical to H.248.TLS, however applicable as well for this work item.}

Table 1 summarizes all possible TCP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TCP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TCP-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TCP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

## 1.2	Package-less TCP connection control

Figure A1 summarizes the methods for H.248-based TCP connection control (at the MG bearer interface). The general approach would be based on package-defined H.248 properties by this Recommendation, - besides SDP elements. However, in simple and limited network environments might be already the package-less option sufficient.

Figure A1 – Overview: Methods for H.248-based TCP connection control

{Edittor’s note: a mixed model of SDP-based and Property-based control might be also an option. To be cross checked again when the package designs get stable ...}

[End Proposed Correction]

_________________