|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 9 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 9 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Clause 6 – Update proposals |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

# Discussion

Additional bearer connection use cases and models are suggested to be added to clause 6.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

[Begin Proposal]

# 6	Motivation, use cases and models

## 6.1	Bearer connection network use cases

There are three fundamental use cases from perspective of H.248 MGs and the TCP bearer connection control (see Figure X):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection; there are two interconnected H.248 stream endpoints (SEP), - a TCP stream endpoint pair (TCP SEPP) -, within the H.248 context; two sub-cases may be distinguished from an overall network perspective:

  - Use case #1.1.1: there is a single, continuous end-to-end TCP connection, any interim H.248 IP-IP MG is not fully terminating the TCP protocol; this use case relates to a TCP relay or merge mode behaviour of the MG;

  - Use case #1.1.2: the end-to-end TCP connection is divided in two separate TCP connection segments, the H.248 IP-IP MG provides complete TCP protocol terminations towards the remote TCP bearer connection endpoints; this use case relates to the TCP proxy mode behaviour of the MG;

- Use case #1.2: a TCP to non-TCP interworking scenario; the H.248 MG is acting as a single TCP endpoint.

Further, but less focus:

- Use case #2: multipath TCP (MPTCP) and H.248 MG involvement.

- Use case #3: multiple TCP bearer connection endpoint models such as applied by media servers (e.g., an [ITU-T H.248.69] based MSRP switch).

Figure X – Bearer connection network use cases with TCP stream endpoints

The TCP connection control aspect is a primary scope of this Recommendation, some general conclusions could be derived, see Table 1:

Table 1 – Use cases and TCP connection control aspects

| Use case: |  | TCP connection control aspects: |
| --- | --- | --- |
| 1.1.1 |  | TCP connection establishment (and release) is true end-to-end, meaning that the TCP connection control procedures at the two H.248 SEPs are tightly coupled. E.g., an incoming TCP connection establishment request (i.e. a TCP SYN packet) from remote endpoint X would lead to an immediate forwarding of that “bearer connection control request” towards remote endpoint Y. This relates to a TCP relay mode scenario. |
| 1.1.2 | TCP connection establishment (and release) is decoupled from end-to-end perspective. This looks like a back-to-back TCP endpoint as given by TCP proxy mode. The TCP bearer connection control procedures are independent for each SEP. |
| 1.2 | The H.248 MG provides an IP host function for a single TCP connection endpoint. |

## 6.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a connection model with up to two ITU-T H.248 terminations. In addition, no assumptions are made regarding either the lower layer protocols beneath the TCP layer or the upper layer protocols being carried by the TCP payload. This allows the use of the Recommendation's procedures in various connection models and use cases such as illustrated in the sub-clauses.

### 6.2.1	Model for single TCP termination scope only

Figure Y1 details the generic connection-model where a TCP-enabled termination is connected to a single other termination.

Figure Y1 – Two-termination context with scope on a single TCP termination only

This model relates to use case #1.2.

### 6.2.2	Model for context-level TCP termination scope

Figure Y2 represents the general model for use case category #1.1. The type of interworking function (IWF) determines the particular MG behaviour concerning TCP handling (see next sub-clause 6.2.3).

Figure Y2 – Two-termination context with context-level TCP scope

### 6.2.3	Model with focus on context-internal interworking function

Figure Y3 depicts a two termination model with scope on example interworking functions.

Figure Y3 – Two-termination context with context-level TCP scope

The IWF is principally a context-level function, associated to a particular SEPP (here: SEPs T1(S1) and T2(S1)). Table 2 lists some example IWFs, which are relevant in scope of this Recommendation and TCP processing, classified from a layered protocol perspective:

Table 2 – Example IWFs

| IWF: |  | Example functions: |  | Comments from TCP perspective: |  |
| --- | --- | --- | --- | --- | --- |
| L3 IWF | - Back-to-back IP host (B2BIH) | Conditional useful for all kind of TCP interworking (NOTE 1). |
|  | B2BIH with: - Network address translation (NAT) - Network prefix translation (NPT in case of IPv6) |  |
|  | - IP protocol version interworking (IPv4-to-IPv6 or vice versa) |  |
|  | - NAT-T according [ITU-T H.248.37] | No impact (NOTE 2). |
|  | - IP router (IPR) [ITU-T H.248.64] - IPR with NAT | Unconditional for all kind of TCP interworking. TO BE CONFIRMED |
| L4 IWF | - this Recommendation | There are two crucial aspects to be noted: - Cut-through characteristic (“when the two SEPs are interconnected for enabled traffic flow”); - BCP (for TCP) information of one SEP as possible stimuli for procedures at partner SEP, or not? (NOTE 3) |
|  | - NAT-T according [ITU-T H.248.84] | Impact, see clause X.Y. |
| L4+ IWF | - Bearer-level ALG [ITU-T H.248.78] | Interactions to be considered. |
|  | - Transport level security [ITU-T H.248.TLS] | There are impacts, dependent on TLS-to-TLS or TLS-to-non-TLS type of interworking. |
| NOTE 1 – Exception: attention to ICMP (see clause 6.7/[ITU-T H.248.79], such as TCP import ICMP information with end-to-end significance. NOTE 2 – This is rather a SEP related function than SEPP scope. NOTE 3 – Example: an incoming TCP connection release triggers an outgoing TCP connection release, or not. |  |  |

[End Proposed Correction]

_________________