|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 10 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 0010 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Clause 3 & 7 – Update proposal for clause 7 and References |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

# Discussion

Purpose of clause 7 is to clarify the relation with other ITU-T Recommendations dealing with H.248 tools which could be used for TCP. Clause 7 was more or less a placeholder so far. We are coming to the conclusion that at least relations to Q.1950, H.248.67 and H.248.84 could be identified. The result of our evaluation is contained in below change proposals.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

Proposal #1:

[Begin Proposal]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP).

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control Protocol: NAT-traversal for peer-to-peer services.

[ITU-T Q.1950]	Recommendation ITU-T Q.1950 (12/2002), Bearer independent call bearer control protocol.

[End Proposed Correction]

Proposal #2:

[Begin Proposal]

# 7	Relation to similar H.248 packages

## 7.1	Overview of TCP related packages

There are some packages from the past which are either defined for TCP or could be theoretically reused for TCP, see Figure 1.

Figure 1 – Overview of TCP related packages

The specific relation to this Recommendation is described below.

## 7.2	ITU-T Q.1950

This Recommendation introduces 11 H.248 packages for usage in so-called Bearer-Independent Call Control type of network architectures. Four out of the eleven could be principally applied (with extensions) for TCP networks:

### 7.2.1	Package ‘gb’ (generic bearer connection):

TCP is a connection-oriented protocol which implies a connection state model (see clause  in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table 1):

Table 1 – Q.1950-defined H.248 packages – gb package

| Package name: | Purpose: |
| --- | --- |
| Generic bearer connection package | The gb package provides control elements for such statefull bearer technologies, which implies basic stimuli for the bearer path coupled bearer control signalling protocol as well as a basic connection state concept. The bearer connection endpoint models supports also incoming and outgoing bearer control protocol procedures, which is e.g. required for the modelling of active/passive open (or close) procedures or client/server behaviour. {Editor’s note (2013-01): unclear is the usage of “cut-through” values?} |
| Possible relation to this Recommendation: |  |
| The TCP connection state model could be basically mapped on the underlying gb package state model. The gb package could be used for the majority of TCP connection control procedures. Open/differences: - TCP explicitly supports simultaneous bearer connection establishment and release, which would be so called collision scenarios in case of the gb package. - TCP bearer connection release: unidirectional concept (called half close), whereas the gb package considers bidirectionality only. - others? |  |

### 7.2.2	Package ‘bcp’ (bearer characteristics):

The notion of bearer characteristics relates to a particular (or multiple) protocol layer(s) in the bearer plane (see Table 2):

Table 2 – Q.1950-defined H.248 packages – bcp package

| Package name: | Purpose: |
| --- | --- |
| Bearer characteristics package | The existing bcp package supports an initial set of bearer technologies such as RTP/IP for IP-based networks. However, bearer type “TCP” is not supported. {Editor’s note (2013-01): assumption that the unassigned codepoints are reserved for additional bearer types?} |
| Possible relation to this Recommendation: |  |
| Usage of the gb package would imply the need for an explicit bearer type indication such as supported by the bcp package. Open/differences: - A new version or an extension package with additional codepoints would be required (at least bearer type “TCP”, but possibly also “TLS”, “TLS/TCP”, “DTLS”, “SCTP”, “DCCP”). - Instead of the Property-based bearer type indication (via bcp package) at LCD level exists the alternative of the SDP-based bearer type indication (via “m=” line element proto) at LD/RD level. - Use case TLS-over-TCP: the establishment/release of the TCP connection and the TLS session could be basically controlled via the the gb package. The bearer connection control procedures at TCP and TLS level could be basically temporarily decoupled, which would imply the modification of the bcp/BNCChar property value, a scenario originally out of scope of the bcp package. |  |

### 7.2.3	Package ‘bnct’ (bearer network connection cut through):

See Table 3:

Table 3 – Q.1950-defined H.248 packages – bnct package

| Package name: | Purpose: |
| --- | --- |
| Bearer network connection cut through package | The MG can indicate whether the cut through will occur "early" or "late", which are effectively related to bearer control protocol signalling events. Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. {Editor’s note (2013-01): the semantics are not fully clear from the package descriptions?} |
| Possible relation to this Recommendation: |  |
| The aspect of “cut-through” control is basically relevant for TCP as well, perhaps even more important due to a) the assured transport characteristic of TCP (e.g., handling of acknowledgments, loss, retransmission) and H.248 MG TCP modes (such as relay, merge or proxy). Open/differences: - The explicit cut-through semantic might be also an implicit capability of TCP modes (TBC). - Unclear is the interaction with StreamMode property settings? - others? |  |

### 7.2.4	Package ‘ri’ (reuse idle):

TCP is a connection-oriented protocol which implies a connection state model (see Figure 6 in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table 1):

Table 4 – Q.1950-defined H.248 packages – ri package

| Package name: | Purpose: |
| --- | --- |
| Reuse idle package | The ri/RII property is used by the MG to indicate to the MGC that an idle bearer is to be reused. {Editor’s note (2013-01): the semantics are not really clear: - What’s the supposed model of a “bearer”, - end-to-end or MG local scope of a bearer connection endpoint only? I.e., a question related to network level or MG local resource management. - Notion of “idle” implies a bearer connection state model, which? What’s the semantic of state “idle”?} |
| Possible relation to this Recommendation: |  |
| Reuse of existing TCP connections is a supported concept, such as in SIP networks by [IETF RFC 4145.] See also clause 13.6/[ITU-T H.248.84]. Open/differences: - Possible models of MG-level TCP resources, e.g.: - resource component “TCP local source/destination transport address”; - resource component “TCP local statefull bearer connection endpoint” which would imply the TCP connection state machine; - resource component “end-to-end TCP bearer connection”, based on the n-tuple of local and remote transport source/destination addresses and connection state; - others? - The semantic of reuse idle bearer would be then dependent on the agreed TCP resource model. It might be even transparent for the MG! |  |

7.3	ITU-T H.248.67

### 7.3.1	Package ‘trm’ (GCP transport mode indication):

See Table 5:

Table 5 – H.248.67-defined H.248 package – bnct package

| Package name: | Purpose: |
| --- | --- |
| GCP transport mode indication package | Related to the H.248 Control Association, which may use TCP-based transport modes. |
| Possible relation to this Recommendation: |  |
| None: [ITU-T H.248.67] and this Recommendation are both diametrically opposed. TCP usage for the H.248 signalling interface is out of scope of this Recommendation. |  |

7.4	ITU-T H.248.84

The TCP enabled SEP will be activated through the indication of the TCP-proxy mode as defined in [ITU-T H.248.84]. This Recommendation only defines properties that are needed on top of [ITU-T H.248.84] for the control of a TCP enabled SEP.

This Recommendation does not define TCP-related statistics and events which are already covered by [ITU-T H.248.84]. Whenever needed those statistics and events of the related packet as defined by [ITU-T H.248.84] must be used.

### 7.4.1	Package-independent procedures for NAT-T with TCP bearers:

See Table 6:

Table 6 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers

| Procedures: | Purpose: |
| --- | --- |
| Clause 13/[ITU-T H.248.84] | a) Indication of IP transport protocol 'TCP' b) Indication of 'TCP mode' for H.248 MG |
| Possible relation to this Recommendation: |  |
| To a): - Normative for SDP-based bearer type indication “TCP”. To b): - The TCP merge mode could be used as NAT traversal service. The TCP modes as such could interact with TCP connection control. E.g., the tcpcc package would be not required (and should be not used) for TCP connection establishment when NAT-T is used (because it provides already end-to-end TCP connection establishment (Note: TCP connection release is different). |  |

### 7.4.2	Package ‘tcphp’ (TCP hole punching):

See Table 7:

Table 7 – H.248.84-defined H.248 package – tcphp package

| Package name: | Purpose: |
| --- | --- |
| TCP hole punching package | NAT traversal support service for TCP. |
| Possible relation to this Recommendation: |  |
| Orthogonal to this Recommendation; the tcphp package could be basically used in conjunction with this Recommendation. |  |

### 7.4.3	Package ‘tcptv’ (TCP traffic volume metrics):

See Table 8:

Table 8 – H.248.84-defined H.248 package – tcptv package

| Package name: | Purpose: |
| --- | --- |
| TCP traffic volume metrics package | Support of octet and packet count statistics for TCP bearer protocol. |
| Possible relation to this Recommendation: |  |
| Performance monitoring is an orthogonal application; hence, the H.248 statistics could be basically used in conjunction with this Recommendation. |  |

### 7.4.4	Package ‘tcpccm’ (TCP connection control metrics):

See Table 9:

Table 9 – H.248.84-defined H.248 package – tcpccm package

| Package name: | Purpose: |
| --- | --- |
| TCP connection control metrics package | Statistics related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation: |  |
| Basically all three statistics could be used. |  |

### 7.4.5	Package ‘tcpcqm’ (TCP connection quality metrics):

See Table 10:

Table 10 – H.248.84-defined H.248 package – tcpcqm package

| Package name: | Purpose: |
| --- | --- |
| TCP connection quality metrics package | Additional statistic related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation: |  |
| The round-trip time based statistic may be used as well. |  |

### 7.4.6	Inventory of TCP functions and their possible impact on H.248 TCP gateways:

See Table 10:

Table 10 – H.248.84-defined H.248 package – tcpcqm package

| Inventory in: | Purpose: |
| --- | --- |
| Appendix II/[ITU-T H.248.84] | TCP Functions versus H.248 TCP Modes of Operation, in ten areas: - Basic Capabilities - Topology Hiding - Protocol Encryption - Security - Application Data Inactivity Detection - Interactions with other policy rules - Performance measurements & statistics" - Connection keep alive support - Add-on's to TCP - IP layer operations |
| Possible relation to this Recommendation: |  |
| This informative analysis is principally valid as well. |  |

7.5	ITU-T H.248.43 and ITU-T H.248.79

Related to filtering of TCP traffic. See overview in clause 9/[ITU-T H.248.79].

[End Proposed Correction]

_________________