|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 11 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 11 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Clause 9 – TCP connection control package update proposals |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TCP. The proposed changes are marked up against the proposed input draft to this meeting TD 19 (WP 2/16).

Clause 9 contains the TCP connection control package, which is proposed to be redesigned as outlined in parallel contribution C-13.

Further information

There are originating and terminating points with respect to bearer connection control procedures. They correspond to client and server roles in case of TCP. See model in Figure A at next page. There are consequently two options from H.248 perspective, e.g. considering the originating side:

- Indirect approach: MGC assigns “client role” to Stream Endpoint (SEP), which triggers subsequently a bearer connection establishment procedures; or

- Direct approach: MGC requests a bearer connection establishment procedures at a SEP.

Both options are functionally equal. The existing TCP connection control package defines an indirect approach. The proposed package revision would use the direct concept (mainly due to alignment with Q.1950).

Figure A – H.248 Termination/SEP perspective: connection model (clause 5/H.248.1) for TCP

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 19 (WP 2/16).

The proposed/discussed “package less” semantics should be later on added to clause 9.6.

[Begin Proposal]

# 9	TCP basic connection control package

|  | Package Name: | TCP basic connection control package |
| --- | --- | --- |
|  | Package ID: | tcpbcc (0x####) |
|  | Description: | This package provides tools for the MGC for controlling TCP endpoints instantiated on a SEP. For a MGC strictly controlled approach the MG can notify the MGC about state changes of the TCP connection. State changes of the TCP connection will not be propagated between the SEPs. By the use of signals the MGC can control each SEP independently. The following capabilities are provided: - The MGC can trigger the setup of a TCP connection, instructing the TCP endpoint acting either as a TCP-client or as a TCP-server; - The MG can notify the MGC in case the setup of the TCP connection has completed; - The MG can notify the MGC in case the closure of the TCP connection has completed ; - The MGC can instruct the MG to release the TCP connection; In case no explicit TCP connection control is required by the MGC, the TCP connection can be setup and released by the MG in an autonomous manner by propagating TCP connection state changes between the SEPs. {Contributor’s note: package description text is not yet updated. Needs to be done if package partitioning proposal is agreed.} |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

## 9.1.1	TCP connection setup role

|  | Property Name: | TCP connection setup role |
| --- | --- | --- |
|  | Property ID: | csr (0x0001) |
|  | Description: | This property defines the MG behaviour for the TCP connection setup. |
|  | Type: | Enumeration |
|  | Possible values: | “notset” (0x0001) This value indicates that the MGC has not yet provided any information whether the MG shall act as a TCP server or TCP client. “client” (0x0002) This value indicates that the MG will initiate the TCP connection setup as a TCP-client. “server” (0x0003) This value indicates that the MG will act as a TCP-server. |
|  | Default: | “notset” |
|  | Defined in: | Local |
|  | Characteristics: | Read/Write |

{Contributor’s note: Property could be realized also as Signal, e.g., in order to get consistency with TCP connection release … See new clause 9.3.x}

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, - either in LD or RD or both -, represents 
a) an implicit TCP client role assignment and
b) a request for immediate start of TCP connection establishment procedure.
Any possible race condition of an incoming TCP connection establishment request (from remote TCP entity) at the MG bearer interface will be resolved as TCP simultaneous open procedure.

Option 2:

SDP-based indication of “TCP” (according clause 13.4/H.248.84) to a stream or termination, but consideration of LD and RD scope with following logic:
Condition 1: IF LD only THEN “TCP server” role of stream endpoint/termination; Wait on incoming TCP connection establishment request (from remote TCP entity);
Condition 2: IF RD only THEN “TCP client” role of stream endpoint/termination; proceed as in option 1;
Condition 3: IF LD AND RD THEN EITHER “Hold connection establishment” OR “error case” OR “…”.

Conclusion: a package-less semantic could be defined, but there are multiple options. Any option shall be aligned with H.248.84 behaviour.
}

## 9.1.x	TCP connection setup cut-through behaviour

## placeholder

{Contributor’s note: this property is a placeholder. We are not really familiar with the Q.1950 semantics on cut-through behaviour. See also contribution C-4, clause 2.4.  Property might be deleted again …}

## 9.2	Events

### 9.2.1	TCP connection established

|  | Event Name: | TCP-connection established |
| --- | --- | --- |
|  | Event ID: | estab (0x0001) |
|  | Description: | This event notifies the MGC that the TCP connection setup is finished, i.e. the TCP connection setup handshake has completed and application data may flow bi-directionally. |

#### 9.2.1.1	EventsDescriptor parameters

None

#### 9.2.1.2	ObservedEventsDescriptor parameters

None

### 9.2.2	TCP connection closed

|  | Event Name: | TCP connection closed |
| --- | --- | --- |
|  | Event ID: | closed (0x0002) |
|  | Description: | This event allows the MGC to be notified when the TCP connection returns to the “CLOSED” state as defined in [IETF RFC 793]. |

#### 9.2.2.1	EventsDescriptor parameters

None

#### 9.2.2.2	ObservedEventsDescriptor parameters

None

### 9.2.x	TCP connection state change (“BNC change”)

|  | Event Name: | TCP connection state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TCP) bearer network connection occurs. For example a bearer has been established or a bearer has been released. This event notifies the MGC that the TCP connection setup is finished, i.e. the TCP connection setup handshake has completetd and application data may flow bi-directionally. |

#### 9.2.x.1	EventsDescriptor parameters

##### 9.2.x.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Enumeration |
|  | Optional: | Yes?? |
|  | Possible values: | Est, [0x01] Bearer Established Mod, [0x02] Bearer Modified Cut, [0x03] Bearer Cut through Mfail, [0x04] Bearer Modification Failure NOTE 1 – For release indication see: General Package E.1.2/H.248.1 Cause Event. |
|  | Default: | ??? |

#### 9.2.x.2	ObservedEventsDescriptor parameters

##### 9.2.x.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type, (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Enumeration |
|  | Optional: | Yes?? |
|  | Possible values: | Est, [0x01] Bearer Established Mod, [0x02] Bearer Modified Cut, [0x03] Bearer Cut through Mfail, [0x04] Bearer Modification Failure NOTE 1 – For release indication see: General Package E.1.2/H.248.1 Cause Event. |
|  | Default: | ??? |

{Discussion – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).
}

## 9.3	Signals

### 9.3.x	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to close the TCP connection). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |
|  | Additional Parameters: | Not applicable |

### 9.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TCP connection). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |
|  | Additional Parameters: | ?? (see gb package) |

{Discussion – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination. Any support of TCP connection reuse would be excluded.

Other options?

None!?

Conclusion: a package-less semantic could be defined.
}

## 9.4	Statistics

### {Contributor’s note: both statistics are independent of above H.248 properties/events/signals concerning TCP bearer connection control. They should be separated in a self-contained package due to the option of “package-less” TCP bearer connection control. 
Such a “statistics” package could be called “TCP assured transport metrics package” or “TCP retrnsmission metrics package”
=> see proposal in clause 11}

### None.

## 9.5	Error Codes

None

## 9.6	Procedures

### {Contributor’s note: description of procedures needs to be updated as soon as package partitioning proposal is agreed.}

### 9.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection control enabled SEP in the MG. This will be indicated on the H.248 interface through the use of the “m=” line as well as the “a=setup” attribute for the affected termination. The rules as specified in [ITU-T H.248.84], 13.4 and 13.5 apply.

{Editor’s note (2012-09 meeting): H.248.84 uses the term « TCP-proxy mode », it might be more appropriate to use the term « TCP connection control mode » instead and to define the « TCP-proxy mode » as an application of the TCP connection control mode.}

Whether this SEP will act as a TCP client or TCP server is indicated by the property Connection Setup Role (csr). In case the role is not yet determined at the time of the SEP’s creation, the value “notset” will be used, and any received TCP segments for this SEP are dropped by the MG.

### 9.6.2	TCP connection establishment

In case the role is not yet determined at the time of the SEP’s creation, the value “notset” will be used for the property Connection Setup Role (csr). Any received TCP segments for this SEP are dropped by the MG in this case.

Once the role of the TCP endpoint is determined by the MGC the property Connection Setup Role (csr) will be set either to “client” or to “server”. The MG will initiate the TCP connection setup according to the indicated role and the TCP connection setup handshake is performed by the MG without any further involvement of the MGC.

The event TCP connection established (estab) will indicate the completion of the three-way-handshake. It corresponds to a state change of the TCP state machine to “ESTABLISHED” as defined in [IETF RFC 793].

## 9.6.3	TCP application data transfer

Once the application data may flow between the termination and the remote TCP-endpoint. The event TCP connection established (estab) does not provide any statement regarding the application data flow between the two SEPs within the same H.248 context.

Thus application data may be received at a point in time where the other stream endpoint of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent.

{Editor’s note (2012-09 meeting): Just two options:

- Acknowledge and buffer the received application data

- Drop the application data and rely on the TCP-retransmission until the other stream endpoint is able to accept and process the application data}

The stream mode property of the Local Control Descriptor affects the application data flow rather than the TCP control information used to setup or close the TCP connection.

{Editor’s note (2012-09 meeting): Consensus required on this behaviour.}

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

## 9.6.4	TCP connection release

The TCP connection release might require different behaviours in the MG, depending on any combinations of

- the application
E.g. TLS to non-TLS interworking; a TCP-release on termination Ta might result in a TLS-release procedure on termination Tb.

- the used transport protocol on the other SEP
E.g. TCP to UDP interworking; no connection release for UDP

- the involvement of the MGC
E.g. the MGC might be involved actively in the release sequence (MGC controlled behaviour)

- …
{Editor’s note (2012-09 meeting): other use cases to be added}

The required behaviour is controlled through the properties MG Autonomous Bearer Release Indicator (abri) and Duplex Closure Indicator (dci).

If the property MG Autonomous Bearer Release Indicator (abri) is set to True, a TCP closure condition that has been indicated by the remote TCP-endpoint or that has been detected locally in the SEP will be passed to the other SEP of the context to release the bearer connection there as well.

If the property MG Autonomous Bearer Release Indicator (abri) is set to False, a detected release condition will not affect the bearer connection on the other SEP. In this case the bearer connection of the other SEP might be released explicitly by the MGC, by the corresponding remote bearer connection endpoint or by other means.

The property Duplex Closure Indicator (dci) controls whether a received half-closure indication from the remote TCP endpoint shall lead to the closure of the complete TCP connection or not. If set to True, a TCP-FIN received for an established TCP connection will lead to the generation of a TCP-FIN by the MG without any involvement of the other SEP.

In case the closure of the TCP connection results in the generation of a TCP-FIN, the MG is required to send the TCP-FIN only after all previous TCP segments for that termination have been sent and have been acknowledge by the remote TCP endpoint.

The closure of a TCP connection is completed if according to the procedures as defined in [IETF RFC 793] the state “CLOSED” is reached. The event TCP connection closed (closed) may inform the MGC about this state change.

## 9.6.4.1	TCP-FIN received from remote TCP endpoint

A received TCP-FIN is acknowledged autonomously by the MG. The MGC can be notified by this event by the use of the TCP hole punching package as defined in [ITU-T H.248.84], see event tcphp/rrel.

In case Duplex Closure Indicator (dci) is set to True, the MG will add a TCP-FIN for transmission to its send buffer. After the TCP-FIN is sent and acknowledged by the remote TCP-endpoint, the TCP connection is closed and the MGC is notified by the event closed if requested.

In case Duplex Closure Indicator (dci) is set to False, the connection will stay in the half-closed state, i.e. application data received from the other SEP will still be transmitted. In this case the connection must be closed by other means, e.g. by an explicit close signal from the MGC or by a received bearer connection release indicator from the other SEP.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the received TCP-FIN indication will be passed to the other SEP to trigger the release of the bearer connection there as well. If MG Autonomous Bearer Release Indicator (abri) is set to False, the other SEP will not be affected by the TCP-FIN. In this case the other SEP must be released by other means, e.g. by a signal from the MGC or by a release indication received externally.

## 9.6.4.2	Close signal received from MGC

The MGC may send the TCP connection closed (closed) signal to trigger the closure of the TCP connection. If this signal is received while the connection is open in the send direction (i.e. either the connection is “established” or it was “established” and the remote TCP endpoint indicated a half-closure by having sent a TCP-FIN), then the MG will send a TCP-FIN. The signal close will not affect the other SEP of the same context. If desired it is up to the MGC to release this SEP accordingly.

## 9.6.4.3	Close indication received from SEP

In case a bearer release indication is received from the other SEP (e.g. it is a TCP endpoint, configured with abri=True and a TCP-FIN has been received) while the TCP connection is open in the send direction, then the MG will send a TCP-FIN.

### 9.6.2	Path MTU Discovery

{Editor’s note (2012-09 meeting): Further study required.}

### 9.6.4	Interaction of TCP-retransmissions with traffic control functions

{Editor’s note (2012-09 meeting): Further study is required.}

# 10	TCP extended connection control package

|  | Package Name: | TCP extended connection control package |
| --- | --- | --- |
|  | Package ID: | tcpecc (0x####) |
|  | Description: | {Contributor’s note: package description needs to be inserted as soon as package partitioning proposal is agreed.} |
|  | Version: | 1 |
|  | Extends: | tcpbcc |

## 10.1	Properties

## 10.1.1	Stream Endpoint Pair Interlinkage

|  | Property Name: | Stream Endpoint Pair Interlinkage |
| --- | --- | --- |
|  | Property ID: | sepplink (0x0001) |
|  | Description: | The semantical scope of this property relates to a Stream Endpoint Pair (SEPP). This property defines if a detected condition leading to the establishment or closure of the TCP connection will trigger an MG-autonomous establishment or release of the bearer connection of the related SEP as well. Only TCP protocol control information (e.g., typically TCP header flag type of information or sequence/acknowledgement numbers) is part of such conditions. Normal TCP data traffic is not affected by this property. |
|  | Type: | Boolean |
|  | Possible values: | True On closure of the TCP connection the MG will autonomously release the bearer connection on the other related SEP as well. False The bearer connection of the other SEP is not affected by the closure of the TCP connection. |
|  | Default: | True |
|  | Defined in: | LocalControl {Editor’s note (2012-09): Its unclear how this property relates to the other Terminations/Streams in the Context. i.e. if a release is triggered as a result of this property are other terminations affected if they don't have this property set? Are events triggerred on these other terminations. Might be also a Context-level property? For further study.} |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“False” semantic”, because the two SEPs within a SEPP are normally decoupled in H.248. Usually an explicit H.248 signalling request (such as a SUB.req for a SEP or termination) would trigger outgoing bearer control procedures at the MG bearer interface.

Option 2:

“True” semantic”, but justification?

Conclusion: a package-less semantic could be defined, any interaction with H.248.84 needs not to be considered because “H.248.84 NAT-T” is only for TCP connection establishment.
}

## 10.1.2	Oneway Release Indicator

|  | Property Name: | Oneway Release Indicator |
| --- | --- | --- |
|  | Property ID: | ori (0x0002) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | True Indicate the duplex closure of the connection by sending a TCP-FIN autonomously. False Keep the TCP connection in the half-closed state, i.e. the MG will not generate the TCP-FIN autonomously. |
|  | Default: | False |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

{Discussion – Package-less semantic (“default MG behaviour”):

Option 1:

“True” semantic”, with the justification that the H.248 media stream concept is fundamentally bidirectional. Such a semantic would lead to a single solution for …

Option 2:

“False” semantic” in order to support this protocol specific characteristic of TCP.

Conclusion: a package-less semantic could be defined, but …. The semantic may be delegated to a profile specification (e.g., when all communication services across TCP would be of type bidirectional).
}

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.3	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

FIXTHIS.

# 11	TCP retransmission metrics package

|  | Package Name: | TCP retransmission metrics package |
| --- | --- | --- |
|  | Package ID: | tcprm (0x####) |
|  | Description: | {Contributor’s note: package description needs to be inserted as soon as package partitioning proposal is agreed.} |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

None.

## 11.3	Signals

None.

## 11.4	Statistics

### 11.4.1	Number of Retransmitted TCP-Segments

|  | Statistic Name: | Number of Retransmitted TCP-Segments |
| --- | --- | --- |
|  | Statistic ID: | retxseg (0x0001) |
|  | Description: | This statistics logs the number of TCP-fragments which are retransmitted due to the expiration of the acknowledgement timer. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 11.4.2	Number of Retransmitted TCP-Octets

|  | Statistic Name: | Number of Retransmitted TCP-Octets |
| --- | --- | --- |
|  | Statistic ID: | retxoct (0x0002) |
|  | Description: | This statistics logs the number of TCP-octets which are retransmitted due to the expiration of the acknowledgement timer. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 11.5	Error codes

None.

## 11.6	Procedures

FIXTHIS.

[End Proposed Correction]

_________________