|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 13 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 13 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TCP: Backup info – Overview Package Redesign Proposal |  |  |  |

Summary

This contribution provides complementary information to work item H.248.TCP, with scope on clause 9/H.248.TCP concerning package redesign discussions.

The ideas and motivation for redesign is subject of parallel contributions C-2, C-3, C-4 and C-43.

The actual change request is contained in C-11.

Page 2: H.248.TCP – Package redesign proposal – Overview

Page 3: H.248.TCP – Package redesign proposal – Details

_________________

Figure 1 – H.248.TCP – Package redesign proposal – Overview

Motivation:

- Package-based TCP control must be always applied when a pure SDP-based control is not possible, which is the case in the very majority of scenarios, particularly in case of H.248 profiles with support of multiple, TCP related services.

- The single tcpcc package is proposed to be partitioned in three packages. The names of some elements are revised.

- Performance monitoring and TCP connection control could be considered as disjoint applications, thus separated, independent packages (tcprm vs tcpbcc and tcpecc).

- TCP connection control could be defined in two services:

  - Basic service (tcpbcc): based on the assumptions that

    - condition I: H.248 TCP bearer types will be used as bidirectional communication paths in the very majority of scenarios (Note: this is a limitation of RFC 793);

    - condition II: H.248 control mode may be “MGC strictly controlled” as default

  - Extended service (tcpecc): the extension package provides additional elements which would allow to “overrule” both limiting conditions (i.e., support of TCP half closure procedures according RFC 793 and tightly coupled e2e connection establishment and release procedures).

- The base TCP connection control (tcpbcc) should (and could) be aligned with H.248.TLS TLS session control package.

Open:

- Not yet decided: generic or bearer type specific package design (see C-4).

Figure 2 – H.248.TCP – Package redesign proposal – Details