|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 14 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 14 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Backup info – Overview Package Redesign Proposal |  |  |  |

Summary

This contribution provides complementary information to work item H.248.TLS, with scope on clause 8/H.248.TLS concerning package redesign discussions.

The ideas and motivation for redesign is subject of parallel contributions C-2, C-3, C-4 and C-43.

The actual change request is contained in C-20.

Page 2: H.248.TLS – Package redesign proposal – Ideas behind functional partitioning

Page 3: H.248.TLS – Package redesign proposal – Overview

Page 4: H.248.TLS – Package redesign proposal – Details

_________________

Figure 1 illustrates the basic idea, - decoupling the two functions of basic session control and capability negotiation:

Figure 1 – H.248.TLS – Package redesign proposal – Basic idea

Motivation: both functions could be considered as disjoint capabilities, from gateway control perspective!

Figure 2 provides the complete picture, - the entire TLS capabilities are divided in four functional areas:

Figure 2 – H.248.TLS – Package redesign proposal – Complete picture:
MG local TLS support: functional TLS architecture proposal 
by partitioning TLS in four functions

All four “H.248 TLS functional areas” should be basically disjoint. Figure 3 illustrates the concrete package redesign proposal. Four base packages, i.e., not any extension principle.

Figure 3 – H.248.TLS – Package redesign proposal – Overview

Motivation - general:

- Package-based TLS control must be always applied when a pure SDP-based control is not possible.

- All four TLS packages are disjoint, i.e., H.248 profiles could them use in any combination.

- When an H.248 profile is not using a particular TLS package, then a MG default behaviour applies.

- Such a (package-less) default behaviour should be explicitly specified in H.248.TLS.

- The TLS session control (tlsc) should (and could) be aligned with H.248.TCP TCP basic connection control package.

Motivation – package specific:

- Package “TLS session control”:  if not used, then SDP-based control would be required.

- Package “TLS capability negotiation”:  if not used, then MG attempts a TLS capability negotiation based on a) supported TLS capabilities and/or b) local policy.

- Package “TLS session maintenance control”:  if not used, then handling of “error procedures” according H.248.TLS ...

- Package “TLS traffic volume metrics”: performance monitoring is orthogonal in any case.

Open:

- Not yet decided: generic or bearer type specific package design concerning TLS session control package (see C-4).

Figure 4 – H.248.TLS – Package redesign proposal – Details