|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 15 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | September 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 15 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Editorial update proposals |  |  |  |

Summary

This contribution provides editorial update proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

There are concrete editorial changes proposed as well as some electronic comments inserted. Some of the comments are already addressed by parallel contributions.

Proposals

It is proposed to accept the marked up changes below.

[Begin Proposal]

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.TLS Ed. 0.1 | Initial draft (TD-23) from WP 2/16 Geneva meeting, Feb. 2012 Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1202_Gen/TD-23.zip |
| H.248.TLS Ed. 0.2 | Input (TD 749 (WP2/16)) from ITU-T SG16 Geneva meeting, Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749 |
| H.248.TLS Ed. 0.3 | Output (TD 749R1/WP2) from ITU-T SG16 Geneva meet., Apr/May 2012 Location: http://itu.int/md/T09-SG16-120430-TD-WP2-0749R1 |
| H.248.TLS Ed. 0.4 | Input (AVD-4232) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4232.zip |
| H.248.TLS Ed. 0.5 | Output (TD-23) from WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-23.zip |
| H.248.TLS Ed. 0.6 | Input (TD 20 (WP2/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: |
| H.248.TLS Ed. 0.7 | Output (TD 20 R1 (WP2/16)) from ITU-T SG16 Geneva meet., Jan 2013 Location: |
| H.248.TLS Ed. 0.8 |  |
| H.248.TLS Ed. 0.9 |  |
| H.248.TLS Ed. 0.10 |  |

CONTENTS

Page

1 Scope 6 1.1 Applicability statements 6 2 References 6 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 7 4 Abbreviations and acronyms 8 5 Conventions 9 6 Use case descriptions 9 6.1 Bearer connection network use cases with H.248 IP-IP gateways 9 6.2 Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways 10 6.3 Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway) 10 7 Models 11 7.1 Network model from H.248 entity point of view 11 7.2 Bearer connection model 11 8 TLS service negotiation control package 12 8.1 Properties 13 8.1.1 TLS Versions 13 8.1.2 Cipher Suites 14 8.1.3 Compression Methods 14 8.1.4 Support for Session Resumption 14 8.1.5 Support for Renegotiation of the Security Context 15 8.1.6 Renegotiation Period 15 8.1.7 TLS-Endpoint Role 16 8.1.8 TLS- Domain-Profile-Identifier 16 8.1.9 Client Authentication Required 16 8.1.10 Autonomous bearer release indicator 17 8.2 Events 17 8.2.1 TLS session established 17 8.2.2 TLS-Session Termination Alert 18 8.2.3 TLS-Session Warning Alert 19 8.2.4 Lower Layer Abort 20 8.3 Signals 20 8.4 Statistics 21 8.4.1 Number of Received TLS-Fragments 21 8.4.2 Number of Sent TLS-Fragments 21 8.4.3 Number of Received Application Data Octets 21 8.4.4 Number of Sent Application Data Octets 22 8.5 Error Codes 22 8.6 Procedures 22 8.6.1 Determining TLS-Related Capabilities 22 8.6.2 Mode of Operation 22 8.6.3 TLS-Endpoint Role 23 8.6.4 TLS-Profile Handling 23 8.6.5 Completion of the initial TLS handshake 26 8.6.6 Security Context Renegotiation 26 8.6.7 TLS-Session Resumption 26 8.6.8 Statistics 27 8.6.9 TLS-Session Termination 27 8.7 Example for the TLS profile concept 28 9 Security considerations 31 Appendix I Sample use-cases of TLS bearer encryption 31 I.1 Use-case #1: Terminal-to-MG TLS session, provisioned TLS service negotiation 31 I.2 Use case #2: Terminal-to-MG TLS session, MGC-controlled TLS service negotiation 32 II.3 Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support 32 Appendix II Example call flows 33 II.1 Initial session setup using SDP security descriptions 33 II.2 TLS to non-TLS IWF, subsequent bearer connection setup 36 Bibliography 40

List of Tables

Page

Table 1 – Principal TLS-based interfaces of H.248 entities and their relevance for this Recommendation 6 Table 2 – Example of a TLS domain profile 29 Table 3 – Example of a MG TLS profile 30 Table 4 – Example of an intersection between a TLS domain profile and a MG TLS profile 30

List of Figures

Page

Figure 1 – Bearer connection network use cases with H.248 IP-IP gateways 10 Figure 2 – Network model from H.248 entity point of view (“half call/bearer connection model”) 11 Figure 3 – TCP/IP protocol stack with TLS protocol 12 Figure 4 – Two-termination context with a TLS termination 12 Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG 32 Figure II.1 – Configuration for the example call flow 33 Figure II.2 – Example call flow, setup of a TLS to non-TLS IWF 34 Figure II.3 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-1 35 Figure II.4 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-2 36 Figure II.5 – Example call flow for a strictly MGC controlled bearer connection setup 38 Figure II.6 – Example call flow, strictly MGC controlled release of a TLS to non-TLS IWF, release initiated by UE-2 39

Draft new ITU-T Recommendation H.248.TLS

Gateway control protocol: 
H.248 packages for control of transport security

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

FIXTHIS …

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP)

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP)

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 2 (12/2009).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), TCP support packages

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	TLS-client [IETF RFC 5246]: The application entity that initiates a TLS connection to a server.  This may or may not imply that the client initiated the underlying transport connection.  The primary operational difference between the server and client is that the server is generally authenticated, while the client is only optionally authenticated.

3.1.2	TLS-server [IETF RFC 5246]: The server is the application entity that responds to requests for connections from clients.  See also "client".

3.1.3	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): stateful forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a stateful TCP connection state machine.

NOTE 3 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

3.1.x	TLS protocol profile: … see e.g. ANNEX E in [3GPP 33.310] ….

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	MG TLS profile: The TLS profile supported by the MG.

3.2.2	TLS Domain: A set of entities in a network domain which use the same set of TLS related parameters, procedures and resources.

3.2.3	TLS domain profile: The TLS profile defined for a TLS domain.

3.2.4	TLS endpoint: Either a TLS-client or a TLS-server.

3.2.5	TLS endpoint profile: The TLS profile used by a TLS endpoint for the setup of a TLS session.

3.2.6	TLS-profile: A selection of options from a set of TLS related parameters and procedures.

3.2.7	TCP proxy variants: the basic TCP proxy mode is defined in clause 3.1.3. The following TCP proxy variants are useful for the various TCP services as supported by H.248 MGs:

There are two orthogonal attributes:

- Discrimination with respect to TCP payload data awareness:

- Application-aware TCP proxy (the MG knows the carried protocol (stack) by TCP packets)

- Application-agnostic TCP proxy (the MG is unaware of the TCP payload content)

- Discrimination with respect to statefull TCP connection control:

- Statefull TCP proxy (by support of the complete TCP connection state machine, see Figure 6 in [IETF RFC 793])

- Lightweight TCP proxy (by support of simplified TCP connection state models)

Example: “application-agnostic, statefull TCP proxy” mode.

{Editor’s note (2012-09): the definition proposals in clause 3.2 are still under discussion .}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2B | Back to Back |
| --- | --- |
| B2BIH | Back to Back IP Host (see also [b-ITU-T H.248.64]) |
| B2BTE | Back-to-Back TCP Endpoint |
| B2BUA | Back to Back User Agent (SIP) |
| BGF | Border Gateway Function |
| FTP | File Transfer Protocol |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MAC | Message Authentication Code |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| SSL | Secure Sockets Layer |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.1) or IP-to-non-IP gateway (see clause 6.2) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.3).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Bearer connection network use cases with H.248 IP-IP gateways

The notion “IP-to-IP” (briefly “IP-IP”) indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 1):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as “non-TLS to non-TLS”);

- Use case #1.2: an end-to-end TCP connection traverses two network domains with different transport security policies (“TLS”, “no TLS”). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as “TLS to non-TLS”);

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as “TLS to TLS”);

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as “TLS to TLS*”);

Figure 1 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor’s note 1 (2012-02): we need to define the term “TLS profile”.}

{Editor’s note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

## 6.2	Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways

Theoretical scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer.

For further studies.

## 6.3	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

{Editor’s note (2012-02): see also discussion of clause 13.3 in H.248.NATTP2P.}

For further studies.

# 7	Models

## 7.1	Network model from H.248 entity point of view

Following network model (Figure 2) may illustrate the relevant areas covered by this Recommendation. The H.248 MG is basically peering a TLS-capable IP host as remote endpoint. Both TLS endpoints spanning a network transport security domain (here TLS domain).

Figure 2 – Network model from H.248 entity point of view 
(“half call/bearer connection model”)

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

{Editor’s note (2012-02): there might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution. This may imply a modified Fig. 2, as well as an extend list above.}

## 7.2	Bearer connection model

{Editor’s note (2012-02): below text is adapted from H.248.77.}

All protocol elements and procedures described in this Recommendation are limited to the extent of a single ITU-T H.248 termination. In addition, no assumptions are made regarding either the lower layer protocols beneath the TLS level or the upper layer protocols being carried by the TLS/TCP transport. This allows the use of the Recommendation's procedures in various connection models and use-cases.

Figure 3 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 3 – TCP/IP protocol stack with TLS protocol

Figure 4 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 4 – Two-termination context with a TLS termination

# 8	TLS service negotiation control package

{Editor’s note (2012-04): the initially proposed two packages by C-826 were “merged” into a single package in this clause (“because the five properties of the “TLS capability package” are also part of the tlsn package” ). Still to do: 
- check/describe provisioning aspects,
- check descriptor level (TerminationState & LocalControl)
This package may be fundamentally used in scenarios

- MGC does not control bearer-path level TLS service negotiations, but MGC may audit supported TLS capabilities;

- MGC want to influence TLS service negotiations;

- others.

Contributions are solicited.}

|  | Package Name: | TLS service negotiation control package |
| --- | --- | --- |
|  | Package ID: | tlsn (0x####) |
|  | Description: | When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. This package provides control tools for the MGC for influencing bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. ### |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0001) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS-RFC (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the “0x”, and if needed, a leading “0” is added for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl, Root {Editor’s note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited.} |
|  | Characteristics: | Read/Write |

### 8.1.2	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0002) |
|  | Description: | This property indicates the cipher suites and their precedence for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sublist of Strings |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA. The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA will be coded into the first two characters, and the second value will be coded into the last two characters of the string. Thus each value in its hexadecimal representation will be converted into the double-hexdigit string. The “0x” will be omitted and if needed, a “0” is used for padding. Over-decadic digits are to be represented by lower case characters (“a”..”f”). {Refer to http://www.iana.org/assignments/tls-parameters/tls-parameters.xml, other contributions are welcome. } |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 8.1.3	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0003) |
|  | Description: | This property declares the list of compression methods and their preference for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 8.1.4	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0004) |
|  | Description: | This property declares the support for the TLS session resumption in the MG |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported by the MG True Session Resumption shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write, Root |

### 8.1.5	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0005) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported by the MG |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported by the MG True Renegotiation shall be supported by the MG |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.6	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0006) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. {Editor’s note: Do we need other criteria for a renegotiation trigger, like nbr fragments sent/rec., nbr octets sent/rec., etc?} |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.7	TLS Endpoint Role

|  | Property Name: | TLS Endpoint Role |
| --- | --- | --- |
|  | Property ID: | er (0x0007) |
|  | Description: | This property declares whether the MG shall act as a TLS-client or TLS-server. |
|  | Type: | Enumeration |
|  | Possible values: | “tcp-alike” (0x0000) The MG’s role shall depend on the TCP-role: If the MG acts as a TCP-client, then the MG shall act as a TLS-client as well. If the the MG acts as a TCP-server, then the MG shall act as a TLS-server aswell. “tls-client” (0x0001) The MG shall act as a TLS-client “tls-server” (0x0002) The MG shall act as a TLS-server |
|  | Default: | “tcp-alike”, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.8	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0008) |
|  | Description: | This property identifies the TLS domain profile that shall be used by the MG to setup the TLS session. |
|  | Type: | String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.9	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0009) |
|  | Description: | This property declares if the MG – if acting as a TLS-server – shall authenticate the client |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS-server, the client is not requested to be authenticated. True In case the MG acts as a TLS-server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.10	Autonomous bearer release indicator

|  | Property Name: | MG Autonomous Bearer Release Indicator |
| --- | --- | --- |
|  | Property ID: | abri (0x0010) |
|  | Description: | This property defines if a detected condition leading to the closure of the TLS session will trigger an MG-autonomous release of the bearer transport connection of the related SEP as well. |
|  | Type: | Boolean |
|  | Possible values: | True On closure of the TLS session the MG will autonomously release the underlying bearer transport connection on the other related SEP as well. False The bearer transport connection of the other SEP is not affected by the closure of the (TLS session)/TCP connection. {Editor’s note (2012-02): wording session/connection should be cross-checked again. Contributions are solicited.} |
|  | Default: | True |
|  | Defined in: | LocalControl or TerminationState (or Root)? {Editor’s note (2012-02): the scope needs to be clarified. Contributions are solicited.} |
|  | Characteristics: | Read/Write |

{Editor’s note (2012-02): the TLS protocol (RFC 5246) should be cross-checked again with regards to relation between TLS session release and (optional) subsequent TCP connection release. Contributions are solicited.}

## 8.2	Events

### 8.2.1	TLS session established

|  | Event Name: | TLS session established |
| --- | --- | --- |
|  | Event ID: | estab (0x0001) |
|  | Description: | This event indicates that the initial TLS-handshake has completed successfully and that application data may flow in both directions. This event is triggered when the Finish message has been sent and the Finish message has been received from the remote TLS-endpoint has been verified successfully. The event shall be set at stream level. |

#### 8.2.1.1	EventsDescriptor parameters

None

#### 8.2.1.2	ObservedEventsDescriptor parameters

#### None

### 8.2.2	TLS-Session Termination Alert

|  | Event Name: | TLS-Session Termination Alert |
| --- | --- | --- |
|  | Event ID: | tlst (0x0001) |
|  | Description: | This event indicates the termination of the TLS session. It does not provide any statement regarding the state of the underlying TCP-connection. The event shall be set at stream level. |

#### 8.2.2.1	EventsDescriptor parameters

None.

#### 8.2.2.2	ObservedEventsDescriptor parameters

##### 8.2.2.2.1	Session Termination Initiator

|  | Parameter Name: | Session Termination Initiator |
| --- | --- | --- |
|  | Parameter ID: | sti (0x0001) |
|  | Description: | Indicates which side initiated the termination of the TLS session. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Session termination was initiated by the MG “remote” (0x0002) Session termination was initiated by the remote TLS-endpoint |
|  | Default: | NA |

##### 8.2.2.2.2	Session Termination Alert Description

|  | Parameter Name: | Session Termination Alert Description |
| --- | --- | --- |
|  | Parameter ID: | stad (0x0002) |
|  | Description: | Indicates the alert description for the termination of the TLS session. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA? {Editor’s note (2012-02): seems to be contradicting with above “IANA value assignment” indication. Hm?. Contributions are solicited.} |
|  | Default: | NA |

### 8.2.3	TLS Session Warning Alert

|  | Event Name: | TLS-Session Warning Alert |
| --- | --- | --- |
|  | Event ID: | swa (0x0003) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the termination of the session. The abnormal condition may be detected on the local or on the remote TLS endpoint. The event shall be set at stream level. |

#### 8.2.3.1	EventsDescriptor parameters

None

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### 8.2.3.2.1	Session Warning Initiator

|  | Parameter Name: | Session Warning Initiator |
| --- | --- | --- |
|  | Parameter ID: | swi (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “local” (0x0001) Abnormal condition detected by MG “remote” (0x0002) Abnormal condition detected by the remote TLS endpoint |
|  | Default: | NA |

##### 8.2.3.2.2	Session Warning Alert Description

|  | Parameter Name: | Session Warning Alert Description |
| --- | --- | --- |
|  | Parameter ID: | swad (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition. The values are as assigned by IANA for the TLS Alert Registry. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | NA |
|  | Default: | NA {Editor’s note (2012-02): seems to be contradicting with above “IANA value assignment” indication. Hm?. Contributions are solicited.} |

### 8.2.4	Lower Layer Abort

|  | Event Name: | Lower layer abort |
| --- | --- | --- |
|  | Event ID: | lla (0x0004) |
|  | Description: | This event indicates that the layer below TLS has become unavailable, thus indicates the termination of the TLS session. The event shall be set at stream level. |

#### 8.2.3.1	EventsDescriptor parameters

None

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### None

## 8.3	Signals

8.3.1	Alert

|  | Signal Name: | Alert |
| --- | --- | --- |
|  | Signal ID: | al (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates “fatal”, or if the ad parameter is set to “close_notify(0)” the session will be terminated. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

8.3.1.1	Additional parameters

8.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertLevel”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

8.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for “AlertDescription”, refer to [IETF RFC 5246]. |
|  | Default: | NA |

## 8.4	Statistics

### 8.4.1	Number of Received TLS-Fragments

|  | Statistic Name: | Number of Received TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recfrag (0x0001) |
|  | Description: | This statistic logs the number of TLS-fragments received by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.2	Number of Sent TLS-Fragments

|  | Statistic Name: | Number of Sent TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentfrag (0x0002) |
|  | Description: | This statistic logs the number of TLS-fragments sent by the MG. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.3	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0003) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the layer above TLS. Received application data messages from the remote TLS endpoint are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 8.4.4	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0004) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 8.5	Error Codes

FIXTHIS …

## 8.6	Procedures

### 8.6.1	Determining TLS Related Capabilities

The MGC can determine the properties of the MG TLS profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr). Normally the audit will be performed on the Root termination.

The Supported TLS Versions property provides the TLS-versions supported by the MG without any specific sequence.

The Supported Cipher Suite property may include the cipher suite TLS_NULL_WITH_NULL_NULL (“0000”), which is used as the initial cipher suite during the first handshake for the TLS-session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

{Editor’s note: TLS-resumption use cases to be identified.}

### 8.6.2	Mode of Operation

The instantiation of a TLS endpoint is indicated to the MG by the “m=” line in the Local and Remote Descriptor. The protocol being used is defined in [IETF RFC 4572] for the media agnostic case or in the media related specification (e.g. [IETF RFC 4975] for MSRP).

With the creation of a TLS endpoint in the MG the underlying TCP endpoint is created as well implicitly. The SDP attribute “a=setup” must be set to “actpass” in the Local and Remote Descriptor to indicate the TCP-proxy mode, refer to [ITU-T H.248.84] and to [ITU-T H.248.TCP].

### 8.6.3	TLS-Endpoint Role

The TLS-Endpoint Role (er) property controls whether the MG shall act as a TLS client or a TLS server. Usually per TLS endpoint this role is coupled to role of the TCP layer, however use cases might exist where the role in the TLS layer and the TCP layer may differ.

At creation time of a TLS endpoint in the MG the TCP connection setup role and the TLS-Endpoint Role (er) might not be defined yet, as normally the SEP is created in the MG before the client/server role negotiation is completed on the call control signalling plane (SDP offer/answer). In such a case the TCP connection setup role “notset” will be used (refer to [ITU-T H.248.TCP]) and the TLS-Endpoint Role (er) will be set to “tcp-alike”.

Once the TCP connection setup role is set to a value different from “notset” (via TCP connection control package property tcpcc/csr) in the MG, the TCP connection setup will be triggered in the MG, and at completion the TLS session will be established autonomously in the MG according to the value of the property TLS-Endpoint Role (er).

### 8.6.4	TLS-Profile Handling

A MG may be requested to support more than one TLS domain. For each such TLS domain different behaviour of the TLS implementation may be required. For each TLS domain those requirements are defined by a “TLS domain profile”. Generally the TLS domain profile must be provisioned on the MG. The property TLS Domain Profile Identifier (dpid) is used to refer to a TLS domain profile. This property must be aligned between the MG and the MGC, as the MGC will use it to indicate to the MG which TLS domain profile will be applicable for the creation of a TLS endpoint in the MG.

The TLS related parameters and procedures supported by the TLS implementation in the MG are referred to as “MG TLS profile”.

On creation of a TLS endpoint in the MG the intersection of the applicable TLS domain profile and the MG TLS profile will be build. This intersection is called the TLS endpoint profile. It defines the TLS related parameters and procedures the MG will use to establish the TLS session on the endpoint.

Figure 1 illustrates the relation between the profiles.

Figure 1: Intersection of the TLS domain profile 1 and the MG TLS profile

{Editor’s note (2012-09 meeting): two clarifications are requested:

- notion of “overall TLS profle”( what exactly? required?)

- relation of “MG TLS profile” to “TLS domain profile(s)”: subset, equal, superset, - all options possible?

Contributions are solicited.}

8.6.4.1	TLS-profile handling: TLS version

The list of TLS versions used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. This list is supposed to be provisioned per dpid on the MG.

In case the MGC provides the property TLS Versions (tlsv), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG is acting as a TLS-client, the highest TLS versions as defined by the TLS-endpoint profile are included into the initial TLS-handshake message (ClientHello). In case the MG acts as a TLS-server, the MG shall select the highest TLS-version that is present in the list of TLS-versions offered from the TLS-client and in the TLS Versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS-handshake is regarded as failed.

{Editor’s note (2012-09 meeting): Error cases need to be worked out, e.g. in case not all TLS-versions received from the MGC are supported by the MG}

8.6.4.2	TLS-profile handling: Cipher suites

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of cipher suites are provisioned per dpid on the MG.

In case the MGC provides the property Chipher Suites (cs), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint. In case the MG is acting as a TLS-client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS-server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS-version.

8.6.4.3	TLS-profile handling: Compression methods

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the MG TLS profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account. It is supposed that the client and the server list of compression methods are provisioned per dpid on the MG.

In case the MGC provides the property Compression Methods (cm), the MG will use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle than the negotiation for the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS-version.

8.6.4.4	TLS-profile handling: Client authentication

The MG’s TLS server implementation must support the client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this setting is provisioned per dpid on the MG.

In case the MGC provides the property Client Authentication Required (car), the MG will use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

8.6.4.5	TLS-profile handling: Self-signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute a=fingerprint. If present in the Remote Descriptor, the client certificate will be self-signed and will be verified against the fingerprint.

8.6.4.6	TLS-profile handling: signed client certificates

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). This list of root certificates is supposed to be configured per dpid on the MG.

8.6.4.7	TLS-profile handling: Server authentication

In case the TLS endpoint in the MG is configured as a TLS server, the MG must provide the server certificate to the client as defined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). Different TLS domains may require different server certificates, because the list of accepted root certificates may differ between TLS domains.

8.6.4.8	TLS-profile handling: Session resumption

FIX THIS

8.6.4.9	TLS-profile handling: Renegotiation support

The TLS implementation in the MG must provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid). It is supposed that this value is provisioned per dpid on the MG.

In case the MGC provides the property Support of Renegotiation of the Security Context (srsc), the MG will use this value instead.

### 8.6.5	Completion of the initial TLS handshake

The TLS handshake will be performed autonomously by the MG with the properties as defined by the TLS endpoint profile.

The initial TLS handshake is completed when the following both conditions are true:

- The TLS-Finish message has been sent, using the negotiated set of security parameters

- The TLS-Finish message has been received and has been successfully verified

At this point in time TLS application data may be sent and received using the negotiated security parameters. The event TLS session established (estab) can be used to notify the MGC accordingly.

The event estab does not provide any statement about the state of the other SEP of the Context.

### 8.6.6	Security Context Renegotiation

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, and the Renegotiation Period (rp) property. In case the TLS-endpoint profile disables renegotiation, the MG will never initiate a renegotiation. If receiving a renegotiation request from the remote TLS-endpoint, the MG will respond with a no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS-version.

If renegotiation shall be supported, the MG will accept and process a renegotiation request from the remote TLS-endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to 0, no renegotiation will be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS-handshake completed successfully. On expiration of the timer the MG will trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS-version. In case the MG receives a trigger for the renegotiation from the remote TLS-endpoint while this timer is running, the MG will cancel and restart this timer.

{Editor’s note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.}

### 8.6.7	TLS-Session Resumption

FIXTHIS …

### 8.6.8	Statistics

FIXTHIS …

### 8.6.9	TLS-Session Termination

The trigger for a TLS session termination might come from different sources:

- The TLS session is terminated by the remote TLS endpoint by sending a close_notify alert or a fatal alert;

- The underlying TCP connection is terminated

- A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicated the closure of the TLS session by the signal Alert;

- The release of the bearer connection of the other SEP triggers the release autonomously in the MG;

- In case the MG acts in an application aware mode, the application part in the MG may indicated the release of the TLS session;

8.6.9.1		TLS alert

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a close_notify was received the TLS endpoint will generate a close_notify on its own and the TLS session is closed. The MG will autonomously initiate the release of the TCP connection as well. If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). In this case the parameter Session Termination Initiator (sti) will be set to “remote” and the parameter Session Termination Alert Description (stad) will contain the value from the AlertDescription as received in the TLS alert message.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.2		TCP connection closed

In case the underlying TCP connection is closed, e.g. due to a received TCP-FIN or TCP-RST the MG will autonomously complete the closure of the TCP session if needed.

If requested the MGC will be notified by the event Lower Layer Abort (lla).

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.3		Non recoverable error condition detected

In case an error condition is detected locally in the TLS layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert will be sent to the remote TLS endpoint with the AlertLevel set to “fatal” and the AlertDescription is provided accordingly.

If requested the MGC will be notified by the event TLS-Session Termination Alert (sta). The parameters Session Termination Alert Description (stad) will contain the same values as sent to the remote TLS endpoint, and the parameter Session Termination Initiator (sti) will be set to “local”.

In case the property MG Autonomous Bearer Release Indicator (abri) is set to True, the MG initiates the bearer release of the related SEP as well.

8.6.9.4		Signal alert received from MGC

In case the signal alert is received and the parameter Alert Level (al) is set to “fatal” or if the parameter Alert Description (ad) is set to “close_notify”, the TLS session closure is initiated by the MG by sending a TLS alert with the same parameter as received from the MGC.

The alert signal does not generate the notification TLS-Session Termination Alert (sta) in return.

Independent of the value of the property MG Autonomous Bearer Release Indicator (abri) the other SEP will not be affected by the alert signal. If desired the MGC needs to release the bearer connection on that SEP separately as well.

8.6.9.5		MG autonomous release initiated on related SEP

The TLS enabled SEP might receive the indication that the bearer connection of the related SEP is released. If such an indication is detected the termination of the TLS session is initiated by sending a TLS alert (close_notify) to the remote TLS endpoint. The closure of the underlying TCP connection is initiated by the MG autonomously.

The event TLS-Session Termination Alert (sta) is not reported to the MGC in this case, as the MGC has the option to be notified by the related SEP about the release of the bearer connection.

8.6.9.6		Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session will be closed autonomously by sending a TLS alert (close notify) and the underlying TCP connection is closed autonomously by the MG as well. The property MG Autonomous Bearer Release Indicator (abri) will have no effect, as it will depend on the application if the bearer connection of the related SEP needs to be released as well.

{Editor’s Note: In this case the behaviour of the application may be controlled by a similar property than tlsn/abri. This would need to be clarified in the application related specification.}

{Editor’s Note:

Observation: There are a lot of commonalities to H.248.TCP:

- Bearer connection state “established” and “closed” are reported by event notifications

- MGC may trigger the release of the bearer connection by a signal (“close”)

- The property MG Autonomous Bearer Release Indicator (abri) allows a “MGC-strictly controlled” behaviour as well as a”MG-autonomous” behaviour. The property  is present in both packages, indeed this indicator is bearer type independent

- Release procedure differs only in minor protocol dependent details (TCP: half closure; TLS: “duplex” closure)

Thus defining a bearer protocol independent package to control the bearer connection setup and release might be an option. Such a packet could be the base for other bearer connection protocols as well like SCTP, DTLS, etc. Such a package could simplify interworking functions between different bearer connection protocols.}

## 8.7	 Example for the TLS profile concept

This section provides examples for the TLS profile concept.

Table 2 depicts an example for a TLS domain profile. The profile is partitioned into three parts: part one describes the TLS client related properties, part two describes the TLS server related properties, and the last part covers the properties which are common to both TLS server and TLS client. Additionally for some profile properties like the cipher suite the a distinction is made between items that must be supported and items that are optionally allowed to be supported.

Table 2 – Example of a TLS domain profile

| Scope | Property | Example |
| --- | --- | --- |
| Client related profile entities | Mandatory list of TLS versions | TLS1.1 |
|  | Optional list of TLS versions | TLS1.2 |
|  | Mandatory list of cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | Optional list of cipher suites | TLS_RSA_WITH_3DES_EDE_CBC_SHA |
|  | Mandatory list of compression methods | NULL |
|  | Optional list of compression methods | DEFLATE, LZS |
|  | Extensions |  |
| Server related profile entities | Mandatory list of TLS versions | TLS1.1, TLS1.2 |
|  | Optional list of TLS versions |  |
|  | Mandatory list of cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | Optional list of cipher suites |  |
|  | Mandatory list of compression methods | NULL |
|  | Optional list of compression methods | DEFLATE, LZS |
|  | Extensions |  |
| Common profile entities | List of root certificates |  |
|  | Client authentication required | yes |
|  | List of supported client certificate types | Signed, self-signed |
|  | Renegotiation support | yes |
|  | Resumption support | no |
|  | Extensions |  |

Table 3 depicts an example for a MG TLS profile. It also shows which properties can be audited on the Root termination by the MGC.

Table 3 – Example of a MG TLS profile

| Scope | Example | Corresponding H.248 signalling element for property audit by MGC |
| --- | --- | --- |
| TLS versions | TLS1.1 TLS1.2 | tlsn/tlsv |
| Cipher suites | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA | tlsn/cs |
| Compression methods | NULL, DEFLATE, LZS | tlsn/cm |
| Client authentication required | yes | tlsn/car |
| Resumption support | no | tlsn/ssr |
| Extensions |  |  |

The following Table 4 shows an example how the intersection of the TLS domain profile and the MG TLS profile is built to determine the values for the TLS endpoint profile. The properties of the TLS endpoint profile will be used by the MG for the TLS handshake. This table does not indicate  which properties can be provided by the MGC.

Table 4 – Example of an intersection between a TLS domain profile and a MG TLS profile

| Property | TLS Domain profile | MG TLS profile | TLS endpoint property value |
| --- | --- | --- | --- |
| TLS version | Mandatory: TLS1.1 Optional: TLS1.2 | TLS1.1, TLS1.2 | TLS1.1, TLS1.2 |
| Cipher suite | Mandatory: TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA Optional: TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_DH_RSA_WITH_AES_256_CBC_SHA256 | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_RC4_128_MD5, TLS_RSA_WITH_DES_CBC_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_NULL_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA |
| Compression method | NULL, LZS | NULL | NULL |
| Client authentication required | Yes | Yes | Yes |
| Root certificates | List: A, B, C | List: X, A, B | List: A, B Provisioned per TLS domain profile |
| MG certificate |  | List of certificates, one certificate per TLS domain profile | Provisioned per TLS domain profile |
| Support for renegotiation | No | Yes | No |
| Support for resumption | Yes | No | No |

{Editor’s note (2012-09 meeting): A clear view on the MG certificate is still missing. It is used during the TLS handshake, thus it is a part of the “set of TLS related parameter and procedures” (current definition of a TLS profile), but it is neither defined for the TLS domain profile nor (currently) for the MG TLS profile. The MG certificate is network element specific and TLS-domain specific.}

# 9	Security considerations

…

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use-case #1:   Terminal-to-MG TLS session, provisioned TLS service negotiation

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain
{Editor’s note: MGC is shown as being outside of the  the bearer TLS-domain, although the MGC might need to provide the MG with  information about the applicable TLS-domain.}

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by SIP/TLS/TCP.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS

- Bearer path: Application/TLS/TCP/IP

- UE: TCPclient and TLS client, MG: TCP server and TLS server (NOTE: this assumption could be satisfied with SDP “a=setup:” attribute)

- H.248 MG TCP-mode: TCP-proxy.
{Editor’s note: Any other TCP-modes and potential transitions between different TCP-modes are to be studied. See also work on H.248.TCP }

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case #2: Terminal-to-MG TLS session, MGC-controlled TLS service negotiation

This use case is an extension of the first use case (clause II.1) by allowing the MGC to influence the TLS service negotiation at bearer level. Use case #1 is based on preconditions which are partially subject of network provisioning. Use case #2 is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

{Editor’s note: this clause will be updated when the design and definition of H.248 packages for TLS will be more mature …}

## I.3	Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support

To be inserted.

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation)

## II.1	Initial session setup using SDP security descriptions

This use case is presented in Appendix I.1. Figure II.1 outlines the network configuration as used in the message flow.

Figure II.1 – Configuration for the example call flow

The scenario shows a simplified SIP session establishment. The TCP-client/server role is defined by the setup attribute (based on [IETF RFC 4145]), and the fingerprint of the UE’s certificate is included in the SDP (based on [IETF RFC 4572]).

The H.248 MG is triggered by the MGC to create two terminations, whereby the bearer for the first SEP/termination is secured by the use of TLS. In the RD of this termination the MG also receives the fingerprint of the UE’s certificate.

The call flow (Figure II.2) shows the setup of the TCP-connection as well as the TLS handshake on the first termination. The self-signed certificate received from the UE is verified by the use of the fingerprint. After the TLS handshake is finished, application data may flow between the UE and the remote call session endpoint. In the message flow presented here the MG will forward these data in the TCP-proxy mode.

Figure II.2 – Example call flow, setup of a TLS to non-TLS IWF

An example for bearer connection release is depicted by Figure II.3. UE-1 initiates the release by sending a TLS-alert(close_notify).

Figure II.3 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-1

Figure II.4 shows the traffic call flow in case the release of the bearer connection is initiated on the non-TLS connection. In the shown example UE-2 initiates the closure of the TCP connection by sending a TCP-FIN. The example assumes that the connection has been setup as shown in Figure II.2.

Figure II.4 – Example call flow, release of a TLS to non-TLS IWF, release initiated by UE-2

## II.2	TLS to non-TLS IWF, subsequent bearer connection setup

This scenario is based on the same network configuration as shown in Figure II.1. It shows a simplified session establishment using SDP offer/answer. The H.248 MG is triggered by the MGC to create two terminations, whereby termination T1 is secured by TLS. The setup of the TCP connection on termination T2 will start after the setup of the TLS session on T1 is completed.

The call flow (Figure II.5) shows the setup of the two terminations strictly controlled by the MGC.

Figure II.5 – Example call flow for a strictly MGC controlled bearer connection setup

The following call flow (Figure II.6) shows the release of the bearer connection strictly controlled by the MGC. The release is initiated by UE-2.

Figure II.6 – Example call flow, strictly MGC controlled release of a TLS to non-TLS IWF, release initiated by UE-2

Bibliography

[b-IETF rtcweb-data]	IETF draft-ietf-rtcweb-data-channel (2012), RTCWeb Datagram Connection.
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-data-channel/

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

____________________